IEEE P802.11
Wireless LANs

| LB97 HT legacy spec comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa Israel 31015 | +972-4-8651547 | Asssaf.kasher@intel.com |

Abstract

This documents suggests comment resolution to some HT Legacy spec related comments.  The CIDs described are: 2899, 1909,  2750, 2777

This document addresses HT Legacy specification comments.

| 2899 | 3.00 | "non-HT physical layer protocol data unit (non-HT PPDU): A Clause 20 PHY PPDU with the TXVECTOR FORMAT parameter set to NON_HT and the CH_BANDWIDTH parameter set to NON_HT_CBW20". "high throughput physical layer protocol data unit (HT PPDU): Any Clause 20 PPDU except when the TXVECTOR FORMAT parameter is set to NON_HT and the CH_BANDWIDTH parameter is set to NON_HT_CBW20." "non-HT duplicate frame: A frame transmitted in a Clause 20. PHY PPDU with the TXVECTOR FORMAT parameter set to NON_HT and the CH_BANDWIDTH parameter set to NON_HT_CBW40." So the non-HT duplicate frame that has only NON_HT attributes is defined as HT PPDU. Such a definition complicates understanding of the spec. | Change the definition to be consistent with the non-HT and HT terminology. |
| --- | --- | --- | --- |

Suggestion: Defered

| 1909 | 245.41 | why are rates 3, 4.5, and 27 called out? Either only call out 20 MHz rate, or include 5 & 10 MHz rates. And what happened to 18 Mbps? | as in comment |
| --- | --- | --- | --- |

Suggestion Counter

TGn Editor: Change the following text in page 234 lines 35-37 D2.04

When an HT device transmits a NON-HT format PPDU with modulations OFDM, UPPER-20-OFDM, LOWER-20-OFDM Clause 17 or Clause 19 packet (rates 3, 4.5, 6, 9, 12, 24 ,27, 36, 48, 54Mb/s)

using more than one transmit chain

| 2750 | 300.57 | 20.3.21.7 |  |  | "The receiver shall be able to decode a packet that was transmitted by a STA with a RIFS separation from the previous packet." Is this true for all packet types - i.e. just transmitted a CCK packet? | Indicate that this is only a requirement when following a HT_MF or HT_GF PPDU. |
| --- | --- | --- | --- | --- | --- | --- |

Suggestion: Rejects

Reason: section 9.2.3.0b already states (as of D2.04) that only HT PPDUs can be separated by RIFS.

| 2777 | 311.47 | 20.4.2 | "dot11ChannelAgilityEnabled Implementation dependent" I would be very interested to know how a 40MHz 2.4GHz HT BSS operates the DSSS channel agility option. | Provide a complete description of the coexistence management when the HT BSS is frequency hopping. Alternatively, (and possibly a teensie weensie bit easier) indicate that a HT BSS shall never set the DSSS channel agility enabled variable to true. |
| --- | --- | --- | --- | --- |

Suggestion: Reject

An HT AP can choose to operate in DSSS only mode and then operate channel agility.  This ability should not be denied from a HT BSS.