IEEE P802.11
Wireless LANs

| Section 3 (Definitions) LB 101 Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emmelmann, Marc | Technical University Berlin | Einsteinufer 25 10587 Berlin Germany | +49–30–31424580 | emmelmann@ieee.org |

Abstract

This documents presents proposed comment resolutions for the following comments referring to or affecting section 3 (definitions) of IEEE 802.11.2-D1.0. Rev. 1 of this document contains comment resolutions which were uncontroversial. All other comment resolution were moved from this document to DCN 11-07/2225r0

The following CIDs are addressed by this document:

1134, 1657, 1658, 1667, 1658, 1668, 92, 1659, 286, 93,

1207, 94, 419, 1132, 1136, 95, 97, 1662, 420, 1133,

1137, 96, 1139, 110, 111, 116, 1140, 1143, 1141, 1144,

1147, 104, 1146, 51

Total # of comments addressed: 34

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Revision History:

| Revision | Date | Comment |
| --- | --- | --- |
| 0 | 2007-07-17 | Addresses issues (1) – (10); numbers as noted in the title for each presented comment resolution |
| 1 | 2007-07-18 | Reflecting changes as incorporated during the discussion / presentation to TGT. Moved all proposed resolutions which the group did not agree on (Title numbered [5]) to document 11-07/2225r0. Moved all unfinished comment resolutioun proposals to document 11-07/2225r0. Added list of addressed CIDs to abstract. |

# (1) Missing Definitions of SUT and DUT

## Addressed  CIDs:

1134, 1657, 1658, 1667, 1658, 1668

### Comments:

| 1134 | Emmelmann, Marc | Neither the term DUT nor SUT is defined. Hence, the reader can only guess what the difference of a DUT vs. SUT is. | Include definitions for DUT and SUT. |
| --- | --- | --- | --- |
| 1657 | Kobayashi, Mark | Since DUT is referred to throughout the draft, the definition of a DUT (Device Under Test) should be defined | Add definition of DUT |
| 1658 | Kobayashi, Mark | Since SUT is referred to throughout the draft, the definition of a SUT (System Under Test) should be defined | add definition of SUT which is defined as the total system under test including the DUT (Device Under Test). |

| 1667 | Kobayashi, Mark | Y | 3.2.33 | 7,2 | T | DUT/SUT is not defined | define the terms DUT and SUT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1658 | Kobayashi, Mark | Y | 3.2 | 7,31 | T | Since SUT is referred to throughout the draft, the definition of a SUT (System Under Test) should be defined | add definition of SUT which is defined as the total system under test including the DUT (Device Under Test). |
| 1668 | Kobayashi, Mark | Y | 3.2.39 | 7,41 | T | DUT/SUT is not defined | define the terms DUT and SUT |

## Resolution:

ACCEPT CIDs: 1134, 1657, 1658, 1667, 1658, 1668

Include the following definitions in Section 3:

Device under test (DUT): a single device being tested, e.g. a single client, AP, mesh point, etc. Usually, the DUT is  tested in combination with a WLCP.

System under test (SUT): a system of devices, i.e. a specific combination of DUTs, being tested at the same time, e.g. client-AP system or a two-AP-one-client system.

# (2) Definition of Fresnel Zone

## Addressed CIDs:

92, 1659, 286 – Missing definition of Lambda

93 – Incomplete / Incorrect definition of Fresnel Zone

1207

## Comments:

| 92 | Foegelle, Michael | N | 3.2.10 | 5,49 | MT | Symbol lambda is undefined and may not be familiar to some readers. | Change to "… electromagnetic waves of wavelength <lambda> propagating within the…" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 93 | Foegelle, Michael | Y | 3.2.10 | 5,49 | MT | The latter half of this definition is imprecise and incomplete. I believe the formulation is defining the boundary of the fresnel zone and indicating that waves that reflect from the edge of the zone would have that path length to reach the receive antenna. As stated, it indicates that all waves (even waves not reaching the receive antenna or waves that encounter reflections before the edge of the zone) would have that path length. | Make this definition equivalent to other standard (and correct) references on the definition of the Fresnel zone. |
| 1659 | Kobayashi, Mark | Y | 3.2.10 | 5,51 | T | definition of lambda in the equation should be defined | Add definition of lambda |
| 286 | McCann, Stephen | N | 3.2.10 | 5,49 | MT | Define λ | λ = wavelength of the radiation. |

## Affected CIDs:

| 1207 | Emmelmann, Marc | Y | 5.5.1.2.1 | 26,44 | E | Def. of Fresnel zone given twice in 802.11.2 (once in the definition section as well as here). | Remove the note and replace def. 3.2.10, section 3.2, p. 5 with the defintion of the Fresnel zone as given in the note on page 26, sect. 5.5.1.2.1. |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Resolution:

Accept CIDs: 92, 93, 1959, 286  by adopting the following changes to the draft:

Counter CIDs: 1207 --- solved by new provided definition.

### Current (D1.0) Draft text:

3.2.10 Fresnel zone: One of a (theoretically infinite) number of concentric ellipsoidal regions surrounding

the LOS path between a transmitter and a receiver; electromagnetic waves propagating within the nth Fresnel

zone have a path length that is nλ/2 greater than the LOS path length.

### Suggested text change:

Fresnel zone: One of a (theoretically infinite) number of concentric ellipsoids of revolution which define volumes in the radiation pattern of a (usually) circular aperture surrounding the visual line-of-sight path between a transmitter and a receiver. The radius of  the nth Fresnel Zone (Fn, n being any integer from 1 to infinity) at  any point P in between sender and receiver is given by: Fn = sqrt[ (n lambda d1 d2) / (d1+d2) ], where d1 is the distance of P to the sender, d2 the distance of P to the receiver, and lambda the wavelength of the transmitted signal.

# (3) Indoor LOS Test environment (3.2.11)

## Addressed CIDs:

94, 419, 1132, 1136

## Comments:

| 94 | Foegelle, Michael | Y | 3.2.11 | 6,01 | MT | This definition indicates that an indoor LOS environment can only be used for testing the receive capabilities of the DUT, and not its transmit or bi-directional communication. | Correct "device emitting the RF test signal" to indicate the WLCP or something more appropriate to the intent of the definition. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 419 | Victor, Dalton | Y | 3.2.11 | 6,01 | T | Definition for Indoor LOS Environment uses the term 'LOS channel' which is ambiguous. What is the difference, in terms of channel characteristics between indoor LOS and indoor NLOS? These two terms need better descriptions from a modeling standpoint. | Definitions need to be more specific. Explain what the channel characteristics are. Simply stating that one can 'see the WLCP' does not describe RF channel characteristics and makes this environment very open to interpretation. |
| 1132 | Emmelmann, Marc | N | 3.2.11 | 6,01 | E | Incoherent usage of abbreviaation DUT | replace "device under test" with "DUT" |
| 1136 | Emmelmann, Marc | Y | 3.2.11 | 6,01 | T | The definition only refers to a DUT even thogugh a SUT is a valid originator / recepient too. | Replace "DUT" with "DUT/SUT" |

## Resolution:

Accept CIDs: 94

Counter CID 419: The definition section includes a definition of indoor LOS channel. Hence the comment will be considered and solved when addressing the indoor LOS channel definition.

Counter CIDs 1132, 1136: New wording of definition does no longer use the term DUT.

### Current (D1.0) Draft Text:

3.2.11 Indoor LOS test environment: An indoor test environment with a LOS channel between the device

under test and the device emitting the RF test signal.

### Suggested Change:

3.2.11 Indoor LOS test environment: An indoor test environment with a LOS channel between the devices emitting / receiving the RF test signal.

# (4) Indoor NLOS environment (3.2.12)

## Addressed CIDs:

95, 97, 1662, 420, 1133, 1137, 96

## Comments:

| 95 | Foegelle, Michael | Y | 3.2.12 | 6,04 | MT | This definition indicates that an indoor NLOS environment can only be used for testing the receive capabilities of the DUT, and not its transmit or bi-directional communication. | Correct "device emitting the RF test signal" to indicate the WLCP or something more appropriate to the intent of the definition. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 97 | Foegelle, Michael | Y | 3.2.12 | 6,04 | MT | This definition implies that the indoor NLOS environment is only suitable for two devices. | Change the latter clause to be more generic. |
| 1662 | Kobayashi, Mark | Y | 3.2.12 | 6,04 | T | definition of NLOS should be defined | Add definition of NLOS |
| 420 | Victor, Dalton | Y | 3.2.12 | 6,04 | T | Definition for Indoor NLOS Environment uses the term 'NLOS channel' which is ambiguous. What is the difference, in terms of channel characteristics between indoor LOS and indoor NLOS? These two terms need better descriptions from a modeling standpoint. | Definitions need to be more specific. Explain what the channel characteristics are. Simply stating that one cannot 'see the WLCP' does not describe RF channel characteristics and makes this environment very open to interpretation. |
| 1133 | Emmelmann, Marc | N | 3.2.12 | 6,05 | E | Incoherent usage of abbreviaation DUT | replace "device under test" with "DUT" |
| 1137 | Emmelmann, Marc | Y | 3.2.12 | 6,05 | T | The definition only refers to a DUT even thogugh a SUT is a valid originator / recepient too. | Replace "DUT" with "DUT/SUT" |
| 96 | Foegelle, Michael | N | 3.2.12 | 6,05 | E | "i.e., an unobstructed path does not exist between the two devices" This phrase has too many negatives. | Change to "i.e. There is no direct LOS path between the DUT and the WLCP." |

## Resolution:

Accept CIDs: 95, 97

Accept CIDs: 1662, 420 --- include new definition of NLOS channel later on and hence give the difference between LOS / NLOS channel

Counter CIDs: 1133, 1137 – New wording of definition does not include term DUT any more.

Counter CIDs: 96 – Comment does not apply to this definition any more but will be considered in the new, upcoming definition of NLOS channel.

### Current (D1.0) Draft Text:

3.2.12 Indoor NLOS test environment: An indoor test environment with a NLOS channel between the

device under test and the device emitting the RF test signal (i.e., an unobstructed path does not exist between

the two devices).

## Recommended Change:

3.2.12 Indoor NLOS test environment: An indoor test environment with a NLOS channel between the devices emitting / receiving the RF test signal.

# (6) Over-the-air test environment (3.2.25)

## Addressed CIDs:

1139

## Comments:

| 1139 | Emmelmann, Marc | N | 3.2.25 | 6,45 | T | The definition only refers to a single DUT even thogugh several DUTs may be tested in a conducted test env. | Replace "DUT" with "DUT(s)" |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Resolution:

Accept CIDs: 1139

### Original (D1.0) Draft Text:

3.2.25 Over-the-air (OTA) test environment: A test environment in which the RF test signals propagate

through the air to and from the antenna of the DUT.

### Proposed Change:

3.2.25 Over-the-air (OTA) test environment: A test environment in which the RF test signals propagate

through the air to and from the antenna of the DUT(s).

# (7) Radiating-Far-Field definition (3.2.30)

## Addressed CIDs:

110, 111

## Comments:

| 110 | Foegelle, Michael | Y | 3.2.30 | 7,07 | MT | This definition doesn't define the meaning of the far field, which is just as important as where it starts. | Expand the definition to indicate that in the far field, there are coherent wave fronts that appear to come from a single point source. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 111 | Foegelle, Michael | Y | 3.2.30 | 7,09 | T | What's the justification of this note? It has nothing to do with the definition and is technically incorrect. | Remove this note. |

## Resolution:

Accept CIDs: 111, 110

Note: There is also an error in the formula. “2D2” should be  “2D^2”, i.e. “2*sqr(D).” Seems to occurred when converting the text to framemaker.

### Original (D1.0) Draft Text:

3.2.30 Radiating far field: The region of space bounded on the inside by a distance r from an antenna or

radiating object with a largest dimension of D such that r > 2D2/λ; also known as the Fraunhofer region.

NOTE—The distance r above can be approximated as 10λ.

### Proposed Change:

3.2.30 Radiating far field: The region of space bounded on the inside by a distance r from an antenna or

radiating object with a largest dimension of D such that r > 2D^2/λ, where λ is the wavelength of the RF test  signal. Within the far field, radiated RF signals are observed as coherent wave fronts that appear to come from a single point source.

# (8) Shielded Enclosure definition

## Addressed CIDs:

116

## Comments:

| 116 | Foegelle, Michael | N | 3.2.35 | 7,25 | MT | "… outside RF…" is the wrong term given other definitions. | Use "… extraneous RF …" |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Resolution:

Accept CIDs: 116

### Original (D1.0) Draft Text:

3.2.35 Shielded enclosure: A shielded box, screen room, or chamber used to isolate a test environment from

outside RF signals.

### Proposed Change:

3.2.35 Shielded enclosure: A shielded box, screen room, or chamber used to isolate a test environment from

extraneous RF signals.

# (9) Improper use of term “endpoint”

## Addressed CIDs:

1140, 1143, 1141, 1144

## Comments:

| 1140 | Emmelmann, Marc | Y | 3.2.16 | 6,17 | T | The term endpoint is not defined. It is used as a synonum for "endstation." | Replace "endpoints" with "endstations" (alternatively, include definition of endpoint) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1143 | Emmelmann, Marc | Y | 3.2.17 | 6,2 | T | Even though the term "endstation" is explicitly defined in Cls. 3.2.8, it is not employed here. If there is a reason for using "station" instead of "endstation" it is not clear in this definition | Replace "station" with "endstation" |
| 1141 | Emmelmann, Marc | Y | 3.2.26 | 6,48 | T | The term endpoint is not defined. It is used as a synonum for "endstation." | Replace "endpoints" with "endstations" (alternatively, include definition of endpoint) |
| 1144 | Emmelmann, Marc | Y | 3.2.36 | 7,29 | T | Even though the term "endstation" is explicitly defined in Cls. 3.2.8, it is not employed here. If there is a reason for using "station" instead of "endstation" it is not clear in this definition | Replace "station" with "endstation" |

## Resolution:

Accept CIDs: 1140, 1143, 1141, 1144

### Information:

Def. of endstation as given in 802.11.2-D1.0:

3.2.8 Endstation: An originator or terminator of traffic, such as an IEEE Std 802.11 STA that is not con-

tained within an AP. (Frequently referred to as a “client” or a “non-AP STA.”)

### Cls. 3.2.16

### Original (D1.0) Draft Text affected by comment:

3.2.16 Latency: A secondary metric that measures the delay between packet transmission and packet recep-

tion over a data network between two endpoints, a transmitter and a receiver.

### Proposed Change:

3.2.16 Latency: A secondary metric that measures the delay between packet transmission and packet recep-

tion over a data network between two endstations, a transmitter and a receiver.

### Cls. 3.2.17

### Original (D1.0) Draft Text affected by comment:

3.2.17 Latency-sensitive traffic: Traffic that represents the exchange of latency-sensitive data between two

stations in a wireless local area network.

### Proposed Change:

3.2.17 Latency-sensitive traffic: Traffic that represents the exchange of latency-sensitive data between two

endstations in a wireless local area network.

### Cls. 3.2.26

### Original (D1.0) Draft Text affected by comment:

3.2.26 Packet loss: A secondary metric that measures packets lost in transmission over a data network

between two endpoints, a transmitter and a receiver.

### Proposed Change:

3.2.26 Packet loss: A secondary metric that measures packets lost in transmission over a data network

between two endstations, a transmitter and a receiver.

### Cls. 3.2.36

### Original (D1.0) Draft Text affected by comment:

3.2.36 Streaming media usage case: The usage case representing the flow of streaming traffic (which in

turn represents the delivery of streaming media such as audio and video) from one station to another station

in the wireless local area network.

### Proposed Change:

3.2.36 Streaming media usage case: The usage case representing the flow of streaming traffic (which in

turn represents the delivery of streaming media such as audio and video) from one endstation to another endstation

in the wireless local area network.

# (10) Problems with the “dependencies” of definitions

## Addressed CIDs:

1147, 104, 1146

## Comments:

| 1147 | Emmelmann, Marc | Y | 3.2.23 | 6,37 | T | Thie definition of Outdoor LOS test environment env. is derived from "outdoor OTA env." but the latter is not correctly defined. | Adopt resolutions as suggested on slides 19 through 22 of 11-07/527r0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1146 | Emmelmann, Marc | Y | 3.2.24 | 6,41 | T | Thie definition of Open OTA env. is derived from "outdoor OTA env." but the latter is not correctly defined. | Adopt resolutions as suggested on slides 19 through 22 of 11-07/527r0 |

| 104 | Foegelle, Michael | Y | 3.2.23 | 6,37 | MT | This definition indicates that an outdoor NLOS environment can only be used for testing the receive capabilities of the DUT, and not its transmit or bi-directional communication. | Correct "device emitting the RF test signal" to indicate the WLCP or something more appropriate to the intent of the definition. |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Resolution:

Counter CIDs: 1147, 1146 (proposed resolution text not a word by word copy of 07/527r0).

Accept CIDs: 104

### Notes:

11-07/527r0, slides 19 to 21, contains a presentation illustrating this problem and the proposed resolution. This document does only include the proposed changes to the Draft text.

### Original (D1.0) Draft Text:

3.2.23 Outdoor LOS test environment: An outdoor over-the-air test environment, representing outdoor

usage of the WLAN, with a LOS channel between the device under test and the device emitting the RF test

signal.

### Proposed Resolution:

Insert new definition:

Outdoor OTA test environment: An over-the-air test environment representing outdoor usage of the WLAN.

Replace Cls. 3.2.2.3 (Outdoor LOS test env.) by:

Outdoor LOS test environment: An outdoor LOS test environment with a LOS channel between the devices emitting and receiving the RF test signal.

# (11) Inheritance of Definitions from 802.11 or other Standards

## Addressed CIDs:

51

## Comments:

| 51 | Stephens, Adrian | N | 3.2.42 | 7,48 | MT | As far as I'm aware, 802.11.2 does not implicitly inherit definitions and abbreviations from 802.11. Therefor terms like STA and AP, which are used in this document need appropriate definition. | Add definitions . Review document for terms "inherited" from 802.11 and add them to definitions/abbreviations as appropriate. |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Resolution:

Counter CID 51.

Add a paragraph after “3.2 Definitions” as follows:

“This subclause does not repeat definitions from IEEE 802.11 or IEEE 100. The reader is refered to these standard documents for such definitions.”