IEEE P802.11
Wireless LANs

| Resolutions to S1 and S2 Security CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Meiyuan Zhao | Intel Corp. | RNB-6-61, 2200 Mission College Blvd, Santa Clara, CA 95052 | +1-408-653-5517 | meiyuan.zhao@intel.com |

Abstract

This contribution includes proposed resolutions to CIDs in the Security area labelled with S1 and S2 issue identifiers.

| CID | Topic Category | Issue Identifier | Comment | Proposed Change by Commenter | Resolution | Resolution Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 481 | Security | S1 | mesh management frame protection not described | document dependency on 11w; document which keys are used to protect unicast and multicast/broadcast mesh management frames (if not clear from 11w) | Reject | should not consider address issues involved in 11w until 11w goes to sponsor ballot. |
| 1542 | Security | S2 | Technically, there are no peer link open, peer link confirm, and peer link close messages because they are defined only as IE and are included in other messages. The current terms can be confusing to readers. | Please replace the peer link open, peer link confirm and peer link close messages with more appropriate terms. | Counter | replace them with Peer Link Open/Confirm/Close frame, these frames are defined in the updated draft D1.05. |
| 1671 | Security | S2 | Why are MKDDIE, EMSAIE and PeerlinkOpenIE included in Association Request frames, but excluded from Reassocation Request frames? | Match the contents of assoc request and reassoc request frames. | Reject | Reassociation Request frame is not used for any purpose in mesh, and Association Request frame is no longer used (has been replaced with Peer Link Open in D1.05). |
| 2192 | Security | S2 | While I can see that the use of peer link xxxx message is an IE in the assocation messages without reading pge 113 line 23-27 I would be completely confused. First off these are not messages they are IE's inside messages. To refer to them as messages is misleading. You also refer to them collectively as peer link establishement messages outside the overview clause. What that means is not explicitly described anywhere. | Remove all instance of any of the "peer link establsihement messages" Use the exact frame and explicitly state the IE you are referring to. Remove lines 23-27 on pg 113 | Counter | New set of Peer Link Management frames are defined with detailed specification on Peer Link Management information element. Replace all references to Peer Link Open/Confirm/Close message with Peer Link Open/Confirm/Close frame. |
| 2234 | Security | S2 | Why are MKDDIE, EMSAIE and PeerlinkOpenIE included in Association Request frames, but excluded from Reassocation Request frames? | Match the contents of assoc request and reassoc request frames. | Reject | same as 1671 |
| 4136 | Security | S2 | We do not need 2 names for the same frame. | Delete "referred to as a Peer Limk Open message", delete "Referred to as a Peer Link Confirm message" and "referred to as a Peer Link Close message" in lines 25,27,29. And change the text beginning in line 30 continuing throuhg to the top of Page 116 to refer to the Association Request and Response Frames (NOT messages). Change line 24 to "The MP uses three management frames to establish a mesh peer link: | Counter | same resolution as 2192 |