IEEE P802.11
Wireless LANs

| LB 101 Proposal for Comment Resolution, Section 4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mark Kobayashi | Broadcom Corp | 190 Mathilda Place Sunnyvale, CA 94086 | 408-543-3318 | mmk@broadcom.com |
| Dalton Victor | Broadcom Corp | 190 Mathilda Place Sunnyvale, CA 94086 | 408-922-5824 | dvictor@broadcom.com |

Abstract

This document contains the proposal for resolution of some comments to draft P802.11.2-D1.0. The comments referenced to are listed in the document IEEE 802.11-07-0659r9 and the comments addressed are: 127, 341, 511, 642, 758, 1854.

# Comments [2]

| CID | Commenter | Part of No Vote? | Editor-assigned Clause | Editor-assigned Page.Line | Editor-assigned Type (T,MT, E) | Comment | Suggested Remedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 224 | Sharma, Neeraj | N | 6.19.2.2.1 | 130.39 | E | "The use of an AETE as described in (e) is not required…" There is no (e) in list of resource requirements |  |
| 341 | Smith, Matt | Y | 4.3.2 | 12.04 | E | "The term 'relative', on the other hand, denotes metrics and environments which are only repeatable at the same location within a relatively short space of time." The preceding uses the term to be defined in the defiinition itself. Also, it is not clear what a "space of time" is. | Replace the quoted text with a non-circular definition. Replace "space of time" with "duration of time". |
| 511 | Ojard, Eric | Y | 4.3.2 | 12.04 | T | what is "a short period of time"? there's never any guarantee of how fast the channel or external impairments will change. | change definition of "relative" to denote metrics which are not repeatable. |
| 642 | Aldana, Carlos | Y | 4.3.2 | 12.04 | T | what is "a short period of time"? there's never any guarantee of how fast the channel or external impairments will change. | change definition of "relative" to denote metrics which are not repeatable. |
| 758 | Moorti, Rajendra | Y | 4.3.2 | 12.04 | T | what is "a short period of time"? there's never any guarantee of how fast the channel or external impairments will change. | change definition of "relative" to denote metrics which are not repeatable. |
| 1854 | Kim, Joonsuk | Y | 4.3.2 | 12.04 | T | "a short period of time" is not clear. | Clarify it |
| 127 | Foegelle, Michael | Y | 4.3.2 | 12.06 | T | These definitions are nonsense. The meaning of absolute and relative values should be well known, and this document is attempting to turn them into something else to justify poor test methods. Whether a metric provides and absolute or relative value is independent of its repeatability or reproducibility (the correct terms for the discussed quantities). What the authors appear to be trying to describe is the difference between quantitative (object A has performance X, object B has performance Y) and qualitative (object A appears to be better than object B) metrics. Unfortunately this document appears to have far too many of the latter metrics which offer no traceability. Given the problems with the concept as a whole, it's impossible to evaluate the merits of specific statements within this section. | Use the appropriate definitions of qualitative vs. quantitative and identify which metrics are which. Eliminate qualtitative metrics in favor of quantitative metrics with proven traceability. |

# Editted Draft Text [1]

4.3.2 Precision and accuracy of metrics and environments

The metrics and environments within this document can be divided into groups based on the precision and accuracy characteristics of each test system. Those test systems with a high precision and high accuracy result in measurements with a small variance and low systematic error. On the contrary, test systems with a low precision and low accuracy result in a high variance with high systematic error.

4.3.2.1 High precision and high accuracy metrics and environments

High precision and high accuracy metrics and environments provide a direct indication of the wireless performance of a DUT. They are desirable in that they realize measurement results with low measurement uncertainty. High precision and high accuracy metrics support calibration procedures and definitions which allow completely independent personnel or entities to reproduce result in low measurement uncertainty; thus the results from two or more sets of high precision and high accuracy measurements can be easily compared.

High precision and high accuracy metrics utilize calibration procedures such as determining the path loss for each relevant RF path, and are structured such that effects unrelated to the IEEE Std 802.11 MAC and PHY layers (such as codecs, buffering, memory, processing power, etc.) will not have a significant impact on the measured values.

The following test environments are categorized as high precision and high accuracy environments:

a) Calibrated over the air test (COAT)

b) Conducted

The following metrics are categorized as high precision and high accuracy metrics:

a) Throughput vs. attenuation

b) Throughput vs. received power

c) Transmit rate adaptation

d) Antenna diversity

e) Adjacent channel interference in a conducted environment

f) BSS transition time

g) Fast BSS transition time

h) Receiver sensitivity in a conducted (or a COAT) environment

i) Unicast intra-BSS throughput

j) Unicast ESS throughput

k) Multicast forwarding rate

l) Endstation association rate

m) Endstation database capacity

n) Power consumption

o) Packet loss

p) Latency

q) Jitter

r) Video performance (when MDI is used as the metric)

4.3.2.2 Low precision and low accuracy metrics and environments

Low precision and low accuracy metrics and environments result in high measurement uncertainty. However, they are useful in that they are adaptable to situations in which calibration (or characterization of the environment) is not convenient, or may not even be possible. The measurements obtained in these scenarios have a high variance and large systematic errors. Low precision and low accuracy metrics will generally yield larger measurement uncertainty than metrics with high precision and high accuracy.

The following test environments are categorized as low precision and low accuracy environments:

a) OTA outdoor LOS

b) OTA indoor NLOS

c) OTA shielded enclosure

The following metrics are categorized as low precision and low accuracy metrics

a) Throughput vs. range

b) Coexistence of overlapping BSSs in an OTA environment (OTA LOS, OTA indoor NLOS, or OTA shielded enclosure)

c) Video performance (when VDER or VQM are used as the metrics)

# Justification

Current Definitions:

Absolute: Repeatable in different locations at different times.

Relative: Repeatable at the same location within a short space of time.

Repeatable in location: The property of test that such that the test can be performed in different locations and the results from the tests performed at different locations agree, to within the specified precision of the test.

Repeatable in time: The property of a test such that the test can be performed at different times and the results from the tests performed at these different times agree, to within the specified precision of the test.

Comments about current draft text:

- Change all references to Absolute Metrics and Environments to Metrics and Environments with High Precision and High Accuracy

Absolute metrics and environments yield results with High Precision and High Accuracy

- Change all references from Relative Metrics and Environments to Metrics and Environments with Low Precision and Low Accuracy.

- The definition of relative includes the phrase ‘short space of time’ which is ambiguous.  Use the following:

Relative = Repeatable at the same location at the same time.

# Proposed Changes:

Counter comments 127, 341, 511, 642, 758 and 1854 by the following:

- Change 3.2.32 to:

3.2.32 Repeatable in time: The property of a test such that the test can be performed at different times at the same location and the results from the tests performed at these different times agree, to within the specified precision of the test.

- Add following text in Section 4.4.1 on line 32, page 13:

Another aspect of throughput measurements in conducted environment is that these measurements are repeatable in location and repeatable in time. Therefore measurements in conducted environment require calibration procedures and definitions such that completely independent personnel or entities can reproduce the results with high precision and accuracy; thus the results from two or more sets of measurements can always be compared.

On the contrary, throughput measurements in un-calibrated OTA environment are not repeatable in location so we cannot compare measurements performed at different locations.

# References

- P802.11.2-D1.0. Draft Recommended Practice for the Evaluation of 802.11 Wireless Performance.

- IEEE 802.11-07-0659r9. LB #101 Comment Resolution Spreadsheet.