IEEE P802.11
Wireless LANs

| LB 101 Resolution of Comments addressing LOS Channel defintion (Cls. 3.2.19) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emmelmann, Marc | Technical University Berlin | Einsteinufer 25 10587 Berlin Germany | +49–30–31424580 | emmelmann@ieee.org |

Abstract

Proposed resolution of  LB 101 CIDs targetted at Subcls 3.2.19, definition of LOS channel.

Addressed CIDs:

1145, 103, 1664

Total number of addressed comments: 3

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# LOS channel definition (3.2.19)

## Addressed CIDs:

1145, 103, 1664

## Comments:

| 1145 | Emmelmann, Marc | Y | 3.2.19 | 6,25 | T | The (wireless) LOS channel is considered of the (direct) path between receiver and sender as well as the 1st Fresnel zone are unobstructed. | Add requirement for unobstructed 1st Fresnel zone to definition. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 103 | Foegelle, Michael | N | 3.2.19 | 6,25 | MT | "For example…" isn't appropriate here. | Change to "This channel is…" |
| 1664 | Kobayashi, Mark | Y | 3.2.19 | 6,25 | T | an unobstructed path between two devices seems vague | an unobstructed RF transmission path should substitute for unobstructed path |

## Related CIDs not addressed here:

| 182 | Foegelle, Michael | Y | 5.5.1.2.1 | 26,37 | T | The first Fresnel zone is not a valid requirement for mimimizing reflections. The Fresnel zone equation is based on narrow beam directional antennas (i.e. circular radiation aperature) used for point-to-point communication and assumes random scattering objects (i.e. hills or buildings) over extremely long distances. For the omnidirectional antennas used in most 802.11 devices, a reflector at the edge of the first Fresnel zone will result in an extremely deep null (-123 dB!) at 300m. This requirement shows a lack of understanding of RF propagation behavior. | If a reflection free environment is required, the requirement should be based on the allowed magnitude of a reflected signal from any direction. For example, to cause <1 dB of error in the LOS signal, a reflection must be 20 dB lower than the direct path, thus the reflected path must be 10x the LOS distance. Since the effects can be additive, there's also a question of how many errors of a given magnitude should be allowed to provide acceptable results. |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Resolution:

Counter CIDs: 1664, 103, 1145

Note: CID 182 by Michael F. should be fixed when describing the related test set-up. The proposed (new) definition of LOS channel if technically correct even though it does not explain on how to assure a LOS channel characteristic between sender and receiver having an omnidirectional antenna. This should be done, if necessary, at the specific test environment.

### Current (D1.0) Draft Text:

3.2.19 Line of sight (LOS) channel: A channel between two devices in which there is an unobstructed path

between the two devices. For example this channel is usually modelled using Ricean statistics.

### Suggested Resolution:

3.2.19 Line of sight (LOS) channel: A channel between two devices in which there is an unobstructed RF transmission path between the entities receiving and transmitting the RF test signal. This channel is usually modelled using Ricean statistics. For  radiation patterns of circular aperture, having an unobstructed 1st Fresnel zone is usually sufficient to assume a LOS channel between the transmitter and receiver.