IEEE P802.11
Wireless LANs

| MDA Action Frame formats |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: july 19-2007 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dee Denteneer | Philips | HTC 37 5656 AE Eindhoven The Netherlands | + 33 40 27 46937 | Dee.denteneer@philips.com |
| Guido Hiertz | Philips | ComNets, RWTH Aachen University, Kopernikusstr. 16, 52074 Aachen, Germany | +49-241-802-5829 | hiertz@ieee.org |

Abstract

This contribution proposes to change the MDA container frame format to a number of individuals frame formats for the individual MDA elements.

- Mesh Resource Coordination action frame details

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field values.

| - Mesh Resource Coordination Action field values |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Request |
|  | Congestion Control Response |
|  | Neighborhood Congestion Announcement |
| 3 | MDA Setup Request |
| 4 | MDA Setup Reply |
| 5 | MDAOP Advertisements Request |
| 6 | MDAOP Advertisements |
| 7 | MDAOP Set Teardown |
| 8 | Beacon Timing Request |
| 9 | Beacon Timing Response |
| 10 | Mesh Channel Switch Announcement |
| 11 | Connectivity Report |
| 12-255 | Reserved |

- Mesh Deterministic Access MDA Setup Request frame format

The Mesh Deterministic Access MDA Setup Request frame is used to request the setup of a set of MDAOPs. It is transmitted by an MDA-active MP to a peer MDA-active MP. This frame is transmitted using individual addresses. The frame body of a Mesh Deterministic Access MDA Setup Request frame contains the information shown in Table 1.

| Table 1-- MDA Setup Request frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | MDA Setup Request- element |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field values for this action frame type.

The MDA Setup Request element is set as described in Error: Reference source not found.

      - Mesh Deterministic Access MDA Setup Reply frame format

The Mesh Deterministic Access MDA Setup Reply frame is used to reply to an MDAOP Setup Request. It is transmitted by an MDA-active MP to a peer MDA-active MP. This frame is transmitted using individual addresses. The frame body of a Mesh Deterministic Access MDA Setup Reply frame contains the information shown in Table 2.

| Table 2--MDA Setup Reply frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | MDA Setup Reply- element |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field values for this action frame type.

The MDA Setup Reply element is set as described in 7.3.2.67.

      - Mesh Deterministic Access MDAOP Advertisements Request frame format

The Mesh Deterministic Access MDAOP Advertisements Request frame is transmitted by an MDA-active MP to one or more peer MDA-active MPs. This frame may be transmitted using group addresses or individual addresses. The frame body of a Mesh Deterministic Access MDAOP Advertisements Request frame contains the information shown in Table 3.

| Table 3--MDAOP Advertisements Request frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field values for this action frame type.

      - Mesh Deterministic Access MDAOP Advertisements frame format

The Mesh Deterministic Access MDAOP Advertisements frame is transmitted by an MDA-active MP to one or more peer MDA-active MPs. This frame may be transmitted using group addresses or individual addresses. The frame body of a Mesh Deterministic Access MDAOP Advertisements frame contains the information shown in Table 4.

| Table 4--MDAOP Advertisements frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | MDAOP Advertisements element |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field values for this action frame type.

The MDAOP Advertisements element is set as described in 7.3.2.69.

      - Mesh Deterministic Access MDAOP Set Teardown frame format

The Mesh Deterministic Access MDAOP Set Teardown frame is transmitted by an MDA-active MP to one or more peer MDA-active MPs. This frame may be transmitted using group addresses or individual addresses. The frame body of a Mesh Deterministic Access MDAOP Set Teardown frame contains the information shown in Table 5.

| Table 5--MDAOP Set Teardown frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | MDAOP Set Teardown element |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field values for this action frame type.

The MDAOP Set Teardown element is set as described in Error: Reference source not found.