IEEE P802.11
Wireless LANs

| IGTK Sequence Number/PN Fixes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

This submission resolves TGw LB 107 CIDs 245, 660, 452, and 736, related to the use of sequence numbers and PNs.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Summary of Changes

CID 245: "It would be helpful to be more specific about the source of the replay counter."

Resolution in this document: change the phrase "value provided by the Authenticator" on page 19 line 20 to refer to the replay counter value supplied by Authenticator in the the IGTK KDE.

CID 660: "Comment Sequence number is PN in the refernced clause.  Clarify what the seq num is."

Resolution in this document: The packet number used for frames protected with the IGTK is the IGTK packet number or IPN.

CID 452: "'PN' is referenced here. Isn't this the sequence number for replay, which is different from the PN."

Resolution in this document: The IGTK PN is different from the CCMP PN.  To avoid confusion, references to the the IGTK PN are changed the IGTK PN (IPN)

CID 736: "PN" should be "IGTK PN"

Resolution in this document: The new "IPN" nomenclature is adopted in response to CID 452 is used.

# Text Changes

Changes to the text are shown in alternate color markup from TGw draft 2.1.

8.3.4.4 BIP replay protection

Change the paragraph beginning on line 17 to read as follows:

The transmitter shall insert a monotonically increasing value into the MMIE Replay field. The receiver shall maintain a 48-bit replay counter for each IGTK. The replay counter, provided in either the 4-way or Group Key handshakes, shall be set to the value of the IPN in the IGTK KDE provided by the Authenticator. The receiver shall interpret the MMIE Replay field as a 48 bit integer. It shall then compare this integer value against the replay counter for the IGTK identified by the MMIE Key ID field. If the integer value from the received MMIE Replay Field is less than or equal to the replay counter value for the IGTK, the receiver shall silently discard the frame and increment the dot11RSNAStatsBIPReplays counter by 1.

8.5.2 EAPOL-Key Frames

Change the paragraph beginning on page 27, line 24 to read:

The format of the IGTK KDE is shown in Table 64B. The IGTK Packet Number (IPN) corresponds to the last PN used by the broadcast/multicast transmitter, to be used by the receiver as the initial value for the BIP replay counter.

Change Table 64B on page 27, to read:

| KeyID | IPN | IGTK |
| --- | --- | --- |
| 2 octets | 6 octets | 16 octets |

8.5.2.1 EAPOL-Key frame notation

Change the IGTK notation beginning on line 50 to read:

IPN	IPN is the current IGTK replay counter value provided by the IGTK KDE

8.5.4.1 Group Key Handshake Message 1

Change page 29, line 35 to read:

- IGTK, IGTK’s key identifier, and IPN (see 8.5.2)

8.5.4.1 Group Key Handshake implementation considerations

Change the lettered list on page 29 to read as follows:

The following steps occur:

a) The Authenticator generates a new GTK, and IGTK. It encapsulates the GTK, and IGTK and sends an EAPOL-Key frame containing the GTK, and IGTK (Message 1), along with the last sequence number used with the GTK (RSC) and the last sequence number used with the IGTK (IPN).

b) On receiving the EAPOL-Key frame, the Supplicant validates the MIC, decapsulates the GTK, and IGTK and uses the MLME-SETKEYS.request primitive to configure the GTK, IGTK, RSC, and IPN in its STA.

References:

P802.11w/D2.1, July 2007