IEEE P802.11
Wireless LANs

| Proposed IBSS Comment Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jesse Walker | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR | +1-503-712-1849 | jesse.walker@intel.com |

Abstract

This submission proposes resolutions to CIDs 444, 18, and 238 from LB102.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

This submission proposes resolutions to CIDs 444, 18, and 238 from LB102.

# Proposed change of 8.4.4.2

Change the second paragraph of 8.4.4.2 of P802.11w D2.1 to the following:

In an IBSS two 4-Way Handshakes exchange RSN information elements to establish the security of the link between two STAs. The Robust Management frame protection capabilities are determined by the RSN information elements exchanged in the 4-Way Handshake initiated by the Authenticator of the STA with the larger MAC address. Table 59b details all of the possibilities, including the case when the STA with the larger MAC address does not support Robust Management frame protection.

To resolve CID 444, change “Table XX Robust Management Frame Selection in an IBSS”  of 8.4.4.2 of P802.11w D2.1, change the column heading:

Local STA’s State

to

STA’s State

and change the column heading

Peer STA’s State

to

Peer STA’s message contents

Also change the text in “Table XX Robust Management Frame Selection in an IBSS”to use the same terminology.

Replace the text in clause 8.4.4.2 of P802.11w D1.01 with the following, renumbering the table as appropriate to resolve CID 18:

Robust Management frame protection is valid only if RSNA is selected to protect data messages and dot11RSNAProtectedManagementFramesEnabled is set to true.

In an IBSS two 4-Way Handshakes exchange RSN information elements to establish the security of the link between two STAs. The Robust Management frame capabilities are determined by the RSN information elements exchanged in the 4-Way Handshake initiated by the Authenticator of the STA with the larger MAC address.  Table XX details all of the possibilities.

Add the following row to the end of  “Table XX Robust Management Frame Selection in an IBSS”  of 8.4.4.2 of P802.11w D2.1 also to resolve CID 18:

| dot11RSNAProtectedManagementFramesEnabled = false or not implemented | Any | The STA shall transmit Robust Management frames without protection, shall discard any unicast protected Management frames it receives, and shall ignore the MMIE in any broadcast/multicast protected Robust Management frame it receives. |
| --- | --- | --- |

To resolve CID 238, change the first sentence of 8.5.4 of P802.11w D1.01 as follows (changes highlighted in red):

The Authenticator uses the Group Key Handshake to send a new GTK, and, if Robust Management frame protection is enabled, a new IGTK, to the Supplicant.

Also change the third bullet of 8.5.4 of P802.11w D1.01 as follows:

IGTK[NIGTK], when present, denotes the IGTK encapsulated with its key identifier as defined in 8.5.2 using the KEK defined in 8.5.1.2 and associated IV.

Also change the second bullet of 8.5.4.1 of P802.11w D1.01 as follows:

When present, IGTK, IGTK’s key identifier, and sequence number (see 8.5.2)

Also change the bullet c) of 8.5.4.1 of P802.11w D1.01 as follows:

Uses the MLME-SETKEYS.request primitive to configure the temporal GTK, and, when present, IGTK into its IEEE 802.11 MAC.

Also change the second paragraph of 8.5.4.3 of P802.11w D1.01 as follows:

The state machines in 8.5.5 and 8.5.6 change the GTK, and, when present, IGTK in use by the network. See Figure 152.

Also change the text of bullet a) of 8.5.4.3 of P802.11w D1.01 as follows:

The Authenticator generates a new GTK, and, when Robust Management frame protection has been negotiated, a new IGTK. It encapsulates the GTK, and as necessary IGTK and sends an EAPOL-Key frame containing the GTK, and IGTK (Message 1), along with the last sequence number used with the GTK (RSC) and the last sequence number used with the IGTK (PN).

Also change the text of bullet b) of 8.5.4.3 of P802.11w D1.01 as follows:

On receiving the EAPOL-Key frame, the Supplicant validates the MIC, decapsulates the GTK, and, when present, the IGTK and uses the MLME-SETKEYS.request primitive to configure the GTK, IGTK,, RSC, and PN in its STA.

Also change the text of bullet d) of 8.5.4.3 of P802.11w D1.01 as follows:

On receiving the EAPOL-Key frame, the Authenticator validates the MIC. If the GTK, and, if present, IGTK is are not already configured into IEEE 802.11 MAC, after the Authenticator has delivered the GTK, and IGTK to all associated STAs, it uses the MLME-SETKEYS.request primitive to configure the GTK, and IGTK into the IEEE 802.11 STA.