IEEE P802.11
Wireless LANs

| LB97 CID 2285 EDCA Cleanup |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |
| Menzo Wentink | Conexant | Oudegracht 3a, Utrecht, Netherlands | +31 65 183 6231 | menzo.wentink@conexant.com |

Abstract

This document contains proposed resolutions for some LB97 CIDs that relate to the MAC adhoc category EDCA Cleanup. Specifically, CIDs 2285, 2288, 2851 and 3013 are addressed.

| 2285 | Stephens, Adrian | 113.52 | 9.9.1.2 | A | "A CF-End frame may be transmitted to cancel remaining NAV time if the prediction of the expected frame exchange duration was not accurate." You probably don't want CF-End transmissions if the remaining duration is < CF-End duration. | Replace with the following: "A CF-End frame may be transmitted to cancel remaining NAV time if the prediction of the expected frame exchange duration was not accurate, and the duration of the CF-End frame is less than the remaining NAV time.. | Accept. Replace the sentence starting on line 113.52 with: "A CF-End frame may be transmitted to cancel remaining NAV time if the prediction of the expected frame exchange duration was not accurate, and the duration of the CF-End frame is less than the remaining NAV time." |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2288 | Stephens, Adrian | 114.18 | 9.9.1.4 | C | "The HT STA may retransmit unacknowledged MPDUs within the same TXOP or in a subsequent TXOP." This is unnecessary. There's nothing to stop it doing this already. | Remove the quoted sentence or turn it into a NOTE-. | Countered by accepting submission 11-07-0636-11 by Tomoko Adachi. The submission specifies PIFS recovery when a NAV was set which has not expired yet. |
| 2851 | Trainin, Solomon | 113.48 | 9.9.1.2 | A | The explicit feedback should be mentioned clearly in the definition of TxOP=0 | After "… may be transmitted at any rate for each TXOP." add a sentence "An explicit feedback may be aggregated with any of control responses or sent once separately under TxOP limit value of 0" | Accepted per comment resolution for CID 2023, submission 11-07-520-02. The quoted section now reads "A TXOP limit value of 0 indicates that a single MSDU or MMPDU or A-MSDU or A-MPDU along with any frames required for beamforming or link adaptation, acknowledgements, and protection, including (#2023) a possible RTS/CTS exchange". |
| 3013 | Yee, James | 114.00 | 9.9.1.4 | C | The term "medium occupancy timer" does not appear elsewhere in this draft or 11ma-D9.0. If we mean to say NAV we should just say it. | As noted. | Countered by accepting submission 11-07-0636-11 by Tomoko Adachi. The submission defines a counter called TXNAV for this purpose. |

References: