IEEE P802.11
Wireless LANs

| LB97 CID 574 Various MAC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |
| Andreas Molisch | MERL |  |  | molisch@merl.com |
| Hongyuan Zhang | Marvell |  |  | hongyuan@marvell.com |
| Carlos Aldana | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94040 |  | caldana@broadcom.com |

Abstract

This document contains proposed resolutions for some LB97 CIDs that relate to various MAC topics including all of the comments from the Antenna Select Misc tab and some others.

| 574 | Dorsey, John | 184.29 | 11.1.2.1 | C | It is unclear whether the secondary beacon may be transmitted if the STBC encoder is not available. Clause 9.6.5 explains that when an STBC encoder is not used, the basic STBC MCS (used to transmit the secondary beacon) takes the value of the lowest rate in the Basic MCS Set. At such a MCS, the secondary beacon loses the range-extending property which was its raison d'être. | Add a sentence clearly stating that the secondary beacon may be generated, or shall not be generated, if an STBC encoder in unavailable. | Counter - replace on page 177 line 53 in D2.05 "The AP may transmit an STBC Beacon" with "An AP that is TX STBC capable may transmit an STBC Beacon" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2159 | Stephens, Adrian | 100.54 | 9.2.8a | R | "busy/idle state of the medium." The MAC doesn't know this. It does, however, know the CCA state. | Replace with "whether the last PHY-CCA.indication received from the PHY contained a busy or idle status". | Reject – see 9.2.1 of the baseline where the state of the medium is defined to be a combination of not only PHY-CCA.indication, but also NAV and TX state information: “The CS mechanism combines the NAV state and the STA’s transmitter status with physical CS to determine the busy/idle state of the medium.” |
| 834 | Lefkowitz, Martin | 102.05 | 9.4 | C | "The attribute dot11MaxTransmitMSDULifetime specifies the maximum amount of time allowed to transmit an MSDU or A-MSDU." what happens to the MSDU within the AMSDU? Which time should you go by the MSDU lifetime, or A-MSDU lifetime? | Remove A-MSDU from the sentence. Keep it an MSDU lifetime counter. This is best for the upper layers, and was the intent during fragmentation in the base standard. | Counter – see “9.7b A-MSDU operation” which describes how the lifetime timer operates for A-MSDU. See also the changes introduced by CID 3161 to this same subclause, which clarifies the case when there may be different values for dot11MaxTransmitMSDULifetime for the constituent MSDUs of an A-MSDU. |
| 326 | Chan2, Douglas | 159.00 | 9.18.1 |  | No need to constraint number of antennas to eight and RF chains to 4. | Remove these constraints. | Reject – the specification only defines MCS values up through 4 streams – if and when additional modes with additional streams are defined, the antenna selection procedures can also be extended at that time. Supporting more than 8 antennas would require additional sounding frames and CSI information to be passed back and forth, causing much increased overhead for the selection process to the point that its benefit would become questionable. |
| 714 | Kasher, Assaf | 160.59 | 9.18.2 |  | "...receiving nor the transmitting STA should switch…" This does not belong here but in 9.17. Antenna switching may occur even if antenna selection is not used. The time in which antenna switching is not allowed must be specified (after the last sounding or MRQ or whatever) | Add a no antenna switching subclause to 9.17 | Counter – language of the nature suggested is added to each of three subclauses of relevance as desired, as per editing instructions from document 11-07-2304r1. |
| 863 | Loc, Peter | 159.27 | 9.18.2 | C | The TX that sends out burst of sounding packets with TXASSI should be a TXOP holder. | change the sentence to "…separated by SIFS in a TXOP of itself ..." | Counter – in TGn draft D2.05 at page 157 about line 18 change the text “separated by SIFS in a TXOP” to “separated by SIFS in a TXOP of which it is the TXOP holder” – note that the concept of TXOP ownership and accompanying terms do not exist, only “TXOP holder” exists, although the use of “its TXOP” has precedent, and therefore, the editor may choose instead to substitute “separated by SIFS in its TXOP” |
| 864 | Loc, Peter | 159.00 | 9.18.2 | C | Both TX and RX may have multiple ASEL capabilities. There is no description on how to interop with different STAs with different capabilities. Further clarification is necessary. | Modify as: "d) If the ASEL Command field in the sounding frames is set to 6, then after receiving all the sounding PPDUs, the receiver shall explicitly feedback the full size channel state information. The +HTC frames with ASEL Command field set to 6 shall not be addressed to a station who does not declare Explicit CSI Feedback Capability, as determined by the Antenna Selection Capable field (see 7.3.2.49.7 (Antenna Selection Capability)). If the ASEL Command field in the sounding frames is set to 0, then after receiving all the sounding PPDUs, depending on the capabilities of the transmitter and receiver, as determined by the Antenna Selection Capable field (see 7.3.2.49.7 (Antenna Selection Capability)), the receiver may either explicitly feedback the full size channel state information or conduct antenna selection computation and feedback the selected antenna indices in a subsequent TXOP...." | Counter – only the first suggested change is needed, and it should appear in the bullet item b), where the ASEL transmitter behavior is described – editor shall make the changes shown for CID 864 in document 11-07-2304r1. |
| 1719 | Nanda, Sanjiv | 163.13 | 9.19.3. | C | Contradiction between the two dashed items. If the previous frame at the receiver was a transmission by the RD responder, then by the first item, the source of the NDP is equal to the TA. By the second item, the transmission was not be the TXOP holder, the source of the NDP is equal to the RA. | Needs to be fixed, so that the second item is "not the TXOP holder, nor the calibration responder, nor a RD responder"… | Counter – see the changes created by document 11-07-2056r1, which removes the confusion. |
| 2432 | Stephens, Adrian | 159.27 | 9.18.2 |  | "The transmitter sends out consecutive sounding PPDUs separated by SIFS in a TXOP using burst transmission with no ACK" There no such thing as "burst transmission" defined. Also, what goes in the PPDUs? Does it matter? | Define fully what MPDUs are transmitted in these PPDUs. | Counter: Delete the words "using burst transmission” from the sentence at about line 18 of page 157 of TGn draft D2.05 (in subclause 9.18.2, first item b)). Note that the contents of the PPDUs does not matter as long as they are sounding frames and do not expect immediate ACK. |
| 2433 | Stephens, Adrian | 159.34 | 9.18.2 |  | "If the transmitter allows antenna indices feedback (by setting the ASEL command field to 0)" I don't see how the control of feedback is provided by the definition for the ASEL comment field set to 0 value, which is TXASSI. | Relate to the names defined in table n13. | Counter – change the words “command field to 0” to “command field to TXASSI” at about line 27 of page 157 of TGn draft D2.05 – and also note the changes to the first item b) of this subclause due to CID 864 which clarify that specific command values allow or require specific receiver behavior, including the allowance to send antenna indices as feedback. |
| 2435 | Stephens, Adrian | 159.58 | 9.18.2 |  | "When providing channel information, an Action No Ack +HTC frame (defined in 7.2.3.12a (Action No Ack frame format)) is used to carry the MIMO CSI Matrices frame feedback defined in 7.4.9.6 (MIMO CSI Matrices frame format)." I see no reason to prevent antenna selection from using an acknowleged frame to carry feedback if the transmitter so wishes. This is particularly important because the report can be segmented across multiple large frames. The probability of error somewhere in the report is high, and the whole report is useless if one segment is missing. | Replace with: "Channel state information is transported using the MIMO CSI Matrices frame defined in 7.4.9.6 (MIMO CSI Matrices frame format)." | Counter - Replace the sentence that begins with “When providing channel information” at about line 53 page 157 of TGn draft D2.05 with: "Channel state information is transported using the MIMO CSI Matrices frame defined in 7.4.8.6 (CSI Matrices frame format) contained within either an Action No Ack +HTC or Action frame." |
| 2436 | Stephens, Adrian | 160.01 | 9.18.2 |  | "When providing antenna indices an Action No Ack +HTC frame (defined in 7.2.3.12a (Action No Ack frame format)) is used to carry antenna selection indices feedback as defined in 7.4.9.9 (Antenna Selection Indices Feedback frame format). One octet of the antenna selection indices field is used to carry the selected antenna indices feedback." Why do we need to constrain to the no-ack case. Also this is inconsistent with 7.4.9.8 that permits either frame type. Also, why do we care that one octet of the antenna selection indices field carries the feedback? This is structural information provided adequately in clause 7 and not relevant here. | Reword thus: "Antenna indices feedback is carried in the Antenna Selection Indices Feedback frame, defined in 7.4.9.9." | Counter: Replace the sentence that begins with “When providing channel information” at about line 61 page 157 of TGn draft D2.05 with: "Antenna indices feedback is carried in the Antenna Selection Indices Feedback frame, defined in 7.4.8.9. and contained within either an Action No Ack +HTC or Action frame." |
| 2437 | Stephens, Adrian | 160.07 | 9.18.2 |  | "In the case that the receiver does not correctly receive the sounding PPDUs, or the current feedback becomes stale" I'm not sure it makes sense to overload these reasons onto one code. There is a reserved code that could be used for "ASEL feedback stale" | Add in table n3 a code for "ASEL feedback stale" Reword the sentence starting line 7 to describe the use of this code as well as the "did not correctly receive" as follows: "In the case that the receiver does not correctly receive the sounding PPDUs, the receiver transmits a +HTC MPDU with the MAI field set to ASELI and the ASEL Command field set to ASEL Training Failure. The receiver notifies the transmitter that the ASEL feedback is stale by transmitting a +HTC MPDU with the MAI field set to ASELI and the ASEL Command field set to ASEL Feedback Stale" | Counter – similar to what was suggested, but with some modifications as shown in document 11-07-2304r1 – editor to make changes shown in 11-07-2304r1 for CID 2437. |
| 2438 | Stephens, Adrian | 160.09 | 9.18.2 |  | +HTC setting MAI to 14 - magic numbers | Replace with "+HTC setting MAI to the value ASELI (see Table n1)." | Accept |
| 2439 | Stephens, Adrian | 160.09 | 9.18.2 |  | "and the command part in ASELC to 6 to indicate the failure of antenna selection training process." But ASELC=6 means something else. Probably it should be 5 (ASEL training failure) I don't like embedding these numbers because they're invariably wrong. | Replace all ASEL magic numbers in 9.18 with names from table n13. | Accept |
| 2442 | Stephens, Adrian | 160.41 | 9.18.2 |  | "The receiver sends out a frame + HTC setting RX ASEL sounding request in the command part of ASELC subfield in HT Control Field, and the data part in ASELC to indicate the number of total sounding PPDUs required." Wrong terminology. | reword thus: "The receiver transmits a +HTC frame with the MAI field set to ASELI, with the ASEL Command field set to RXASSR and the ASEL Data field set to the number of sounding PPDUs required." | Accept |
| 2443 | Stephens, Adrian | 160.50 | 9.18.2 |  | "The transmitter responds with the corresponding number of sounding PPDUs or NDPs in its subsequent TXOP, using burst transmission with no ACK, with setting RX ASEL sounding indication, and sounding frame format." Issues: 1. An NDP *is* a sounding PPDU 2. "Burst transmission" is not a defined term 3. "with setting" is ungrammatical 4. The names do not relate to table n3. 5. There's no such thing as "sounding frame format" | Replace with the following: "The transmitter responds with the corresponding number of sounding PPDUs in its subsequent TXOP. These PPDUs are separated by SIFS. When using non-NDP sounding, each PPDU contains a +HTC frame in which the MAI field is set to ASELI, the ASEL Command field is set to RXASSI and the ASEL Data subfield is set to the remaining number of sounding PPDUs to be transmitted. When using NDP sounding, the PPDU that precedes the first NDP contains a +HTC frame in which the NDP Announce field is set to 1, the MAI field is set to ASELI, the ASEL Command field is set to RXASSI and the ASEL Data field is set to the remaining number of sounding PPDUs to be transmitted" | Accept |
| 2444 | Stephens, Adrian | 160.55 | 9.18.2 |  | "these sounding PPDUs or NDPs," - an NDP is a sounding PPDU | reword: "these sounding PPDUs," Ditto change line 56 | Accept |
| 2447 | Stephens, Adrian | 160.64 | 9.18.2 |  | "and TX and RX antenna selection training shall be done alternately." What does this mean? The statement is ambiguous. Also this appears to be a normative requirement placed jointly on the STA at each end of a link. This is not possible. | Reword in terms that define the normative requirements for each end of the link independently. | Counter – Delete the phrase “with “and Tx and Rx antenna selection training shall be done alternately” that appears at the end of the last paragraph of subclause “9.18.2 Procedure” at about line 11 of page 159 of TGn Draft D2.05 – note that the restriction intended by the deleted phrase is implicit in the descriptions of transmitter and receiver behavior, where TXOP holder or its TXOP are mentioned. See also CID 863 and its resolution. |
| 2448 | Stephens, Adrian | 161.19 | 9.18.2 |  | "A STA shall use the Control Wrapper frame (7.2.1.9 (Control Wrapper frame)) to announce NDP in non-HT PPDU control frames." Seeing as both HT and non-HT must use the control wrapper frame for +HTC control frames, this statement is incomplete and possibly misleading. | Delete it. | Accept |
| 2452 | Stephens, Adrian | 161.28 | 9.18.2 |  | "If the reception immediately preceding (the frame ending at SIFS from the start of the NDP) the NDP has an FCS error, the NDP shall be dropped by the receiver." "shall be dropped". This is not an adquately precise normative requirement. It's also unnecessary, because the STA cannot determine that any NDP was addressed to it. So it will ignore the NDP in the same way that it will ignore NDPs sent to other STA. | Remove the quoted sentence. | Accept |
| 3235 | Erceg, Vinko | 159.00 | 9.18.2 |  | This paragraph apparently does not allow for delayed CSI feedback response. | Please add such a feature. | Accept – see CID 2435, which makes the behavior explicit. |
| 3236 | Erceg, Vinko | 160.01 | 9.18.2 |  | Bullet item 2) says that the feedback must be in an Action No Ack frame -- this seems to contradict the description on page 159, lines 54-55, where it says that feedback may either "explicitly feedback" ... or "feedback...in a subsequent TXOP" - I think that it should say "immediately feedback" in stead of "explicitly feedback" on page 159, line 54 and the suggestion of a new TXOP for feedback implies that the feedbacker will initiate a TXOP. As such, one would expect that this feedback in the new TXOP initiated by the feedbacker would be suitably transmitted within a normal, ack-able action frame. | Change item 2) to allow a regular action mgmt frame to be used in addition to the action no ack frame for feedback purposes, with a dependency on whether the feedback is immediate (as a SIFS response) or in a new TXOP | Counter - see CID 2435. |
| 3237 | Erceg, Vinko | 159.00 | 9.18.2 |  | In the antenna selection case, receiver may want to initiate antenna selection sequence retry by the transmitter in the case the receiver determines that training is not valid (usable) because of the bursty interference , for example. | Allow for all zeros to be returned as antenna selection feedback after an antenna selection training sequence - this may effectively generate a retry on the part of the transmitter because the receiver determined that none of the selections provided a descent signal, perhaps due to an interference event that occurred during the training. Basically the need here is to establish that an all zeros value is not an error, and to establish what should be done when it occurs (see 7.3.1.33, pg 52). | Reject – training failure error code already exists. See also CID 2437 and its resolution. |
| 3238 | Erceg, Vinko | 159.00 | 9.18.2 |  | If TX antenna selection fails up to the kth packet, there is no way for the receiver to signal the transmitter to start from the (k+1)th packet. The way the current spec is written the transmitter would have to start from the beginning. | Allow for starting from (k+1)th packet either with a single "restart value" perhaps as an ASEL DATA value associated with the ASEL command "failure" or by providing for selective sounding packet failure indication by using, for example a bit field in a new HT PHY action frame with 16 bits indicating which of the possible 16 sounding frames was NOT received properly. | Counter – see CID 2437 and associated changes as found in 11-07-2304r1. |
| 3386 | Madhavan Pillai, Krishna Sankar | 160.43 |  |  | Typo: "…command part in ASELC to 6 to indicate the failure of antenna selection training process." Reason:Per Table n3, ASEL Command 5 indicates ASEL Training failure | Change to "…command part in ASELC to 6 5 to indicate the failure of antenna selection training process." | Counter – see CID 862 which addresses the same error. |

Comments being readdressed:

| 433 | Chaplin, Clint | 160.60 | 9.18.2 | C | "should switch its antennas.When both" missing space | "should switch its antennas. When both" | New resolution: Counter: First sentence of the pair is modified and moved to a different location as per CID 714. Old resolution: EDITOR: 2007-04-02 13:22:34Z Accept |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 2430 | Stephens, Adrian | 159.24 | 9.18.2 | C | "sending a TX ASEL sounding request." - this does not relate to the nomenclature of table n3 | For all terms defined in table n3, review 9.18 and subclauses and replace with one of the terms defined in table n3 - using the full form for the first use and the abbreviated form thereafter. | New resolution: Counter – see CID 2443, the resolution of which effectively includes the suggested change from this CID. Old resolution: EDITOR: 2007-04-02 11:08:19Z Accept |
| --- | --- | --- | --- | --- | --- | --- | --- |

CID 714:

TGn Editor: Delete the first sentence of the last paragraph of subclause “9.18.2 Procedure” of TGn Draft D2.05 on about page 159 line 6.

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of subclause “9.17.2.2 Unidirectional implicit transmit beamforming” of TGn Draft D2.05 at about page 148 line 37:

During the PPDU exchange, neither the receiving nor the transmitting STA should switch antennas.

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of subclause “9.17.2.3 Bidirectional implicit transmit beamforming” of TGn Draft D2.05 at about page 150 line 5:

During the PPDU exchange, neither the receiving nor the transmitting STA should switch antennas.

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of subclause “9.17.2.4.2 Procedure” of TGn Draft D2.05 at about page 154 line 5:

From the beginning of STEP 1 of the calibration procedure and continuing through the end of STEP 2 of the calibration procedure, neither the receiving nor the transmitting STA should switch antennas.

CID 864:

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of the first bulleted item b) within subclause “9.18.2 Procedure” of TGn Draft D2.05 at about page 157 line 40:

The ASEL transmitter shall not include the value of 6 in the command field of the sounding frame if the last received value of the Explicit CSI Feedback Capable subfield of the Antenna Selection Capability field (see 7.3.2.52.7 Antenna Selection Capability) from the receiver was zero.

CID 2437:

TGn Editor: Change the row of table “Table 7-6c – The ASEL Command and ASEL Data parts of the ASELC subfield” of subclause “7.1.3.5a HT Control field” of TGn Draft D2.05 at about page 21 line 24 that includes the value “5” for the “ASEL Command” column to appear as follows:

| 5 | No Feedback | 0-15 Training Failure - when the training process failed, for example, due to failure of the receiver to receive the training PPDUs this integer value indicates the point of failure, this integer value corresponds to the first sounding PPDU that was not received correctly |
| --- | --- | --- |

TGn Editor: Change the row of table “Table 7-6c – The ASEL Command and ASEL Data parts of the ASELC subfield” of subclause “7.1.3.5a HT Control field” of TGn Draft D2.05 at about page 21 line 29 that includes the value “7” for the “ASEL Command” column to appear as follows:

| 7 | ASEL Feedback Stale | Reserved |
| --- | --- | --- |

TGn Editor: Change what might be about paragraph 13 (depending on how you count paragraphs) of subclause “9.18.2 Procedure” of TGn Draft D2.05 at about page 158 line 1 as follows:

If the receiver does not correctly receive the sounding PPDUs, the receiver shall send a +HTC frame with the MAI subfield set to ASELI and the ASEL Command subfield set to No Feedback and the ASEL Data subfield set to the integer value corresponding to the number in sequence of the first sounding PPDU that was not properly received to indicate the failure of the antenna selection training process.

The receiver shall notify the transmitter that the ASEL feedback is stale by transmitting a +HTC MPDU with the MAI subfield set to ASELI and the ASEL Command subfield set to ASEL Feedback Stale.

References: