IEEE P802.11
Wireless LANs

| LB97 CID 574 Various MAC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |
| Andreas Molisch | MERL |  |  | molisch@merl.com |
| Hongyuan Zhang | Marvell |  |  | hongyuan@marvell.com |
| Carlos Aldana | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94040 |  | caldana@broadcom.com |

Abstract

This document contains proposed resolutions for some LB97 CIDs that relate to various MAC topics including all of the comments from the Antenna Select Misc tab and some others.

REVISION NOTES:

R2:

PINK comments reviewed but not yet resolved, review took place during August 15, 2007 TGn MAC adhoc conference call.

GREEN comments reviewed and resolved unanimously during August 15, 2007 TGn MAC adhoc conference call.

CIDs with no coloring have not yet been reviewed by the MAC adhoc.

Added CID 2180 and CID 562 in the readdressed CID section – both are the result of additional changes due to CID 574 proposed resolution.

R3:

A separate proposal was already prepared for CID 864, so it needs to be re-opened – it is marked in satanic RED for easy identification.

R4-6:

Some comment resolutions reworded.

HOT PINK now designates those CIDs that need re-review.

| 574 | Dorsey, John | 184.29 | 11.1.2.1 | C | It is unclear whether the secondary beacon may be transmitted if the STBC encoder is not available. Clause 9.6.5 explains that when an STBC encoder is not used, the basic STBC MCS (used to transmit the secondary beacon) takes the value of the lowest rate in the Basic MCS Set. At such a MCS, the secondary beacon loses the range-extending property which was its raison d'être. | Add a sentence clearly stating that the secondary beacon may be generated, or shall not be generated, if an STBC encoder in unavailable. | Counter – editor shall make the changes shown for CID 574 in document 11-07-2304r2. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2159 | Stephens, Adrian | 100.54 | 9.2.8a | R | "busy/idle state of the medium." The MAC doesn't know this. It does, however, know the CCA state. | Replace with "whether the last PHY-CCA.indication received from the PHY contained a busy or idle status". | Reject – see 9.2.1 of the baseline where the state of the medium is defined to be a combination of not only PHY-CCA.indication, but also NAV and TX state information: “The CS mechanism combines the NAV state and the STA’s transmitter status with physical CS to determine the busy/idle state of the medium.” |
| 834 | Lefkowitz, Martin | 102.05 | 9.4 | C | "The attribute dot11MaxTransmitMSDULifetime specifies the maximum amount of time allowed to transmit an MSDU or A-MSDU." what happens to the MSDU within the AMSDU? Which time should you go by the MSDU lifetime, or A-MSDU lifetime? | Remove A-MSDU from the sentence. Keep it an MSDU lifetime counter. This is best for the upper layers, and was the intent during fragmentation in the base standard. | Counter – see “9.7b A-MSDU operation” which describes how the lifetime timer operates for A-MSDU. See also the changes introduced by CID 3161 to this same subclause, which clarifies the case when there may be different values for dot11MaxTransmitMSDULifetime for the constituent MSDUs of an A-MSDU. |
| 326 | Chan2, Douglas | 159.00 | 9.18.1 |  | No need to constraint number of antennas to eight and RF chains to 4. | Remove these constraints. | Reject – the specification only defines MCS values up through 4 streams – if and when additional modes with additional streams are defined, the antenna selection procedures can also be extended at that time. Supporting more than 8 antennas would require additional sounding frames and CSI information to be passed back and forth, causing much increased overhead for the selection process to the point that its benefit would become questionable. |
| 714 | Kasher, Assaf | 160.59 | 9.18.2 |  | "...receiving nor the transmitting STA should switch…" This does not belong here but in 9.17. Antenna switching may occur even if antenna selection is not used. The time in which antenna switching is not allowed must be specified (after the last sounding or MRQ or whatever) | Add a no antenna switching subclause to 9.17 | Counter – language of the nature suggested is added to each of three subclauses of relevance as desired, as per editing instructions from document 11-07-2304r2. |
| 863 | Loc, Peter | 159.27 | 9.18.2 | C | The TX that sends out burst of sounding packets with TXASSI should be a TXOP holder. | change the sentence to "…separated by SIFS in a TXOP of itself ..." | Counter – in TGn draft D2.05 at page 157 about line 18 change the text “separated by SIFS in a TXOP” to “separated by SIFS in a TXOP of which it is the TXOP holder” – note that the concept of TXOP ownership and accompanying terms do not exist, only “TXOP holder” exists, although the use of “its TXOP” has precedent, and therefore, the editor may choose instead to substitute “separated by SIFS in its TXOP” |
| 864 | Loc, Peter | 159.00 | 9.18.2 | C | Both TX and RX may have multiple ASEL capabilities. There is no description on how to interop with different STAs with different capabilities. Further clarification is necessary. | Modify as: "d) If the ASEL Command field in the sounding frames is set to 6, then after receiving all the sounding PPDUs, the receiver shall explicitly feedback the full size channel state information. The +HTC frames with ASEL Command field set to 6 shall not be addressed to a station who does not declare Explicit CSI Feedback Capability, as determined by the Antenna Selection Capable field (see 7.3.2.49.7 (Antenna Selection Capability)). If the ASEL Command field in the sounding frames is set to 0, then after receiving all the sounding PPDUs, depending on the capabilities of the transmitter and receiver, as determined by the Antenna Selection Capable field (see 7.3.2.49.7 (Antenna Selection Capability)), the receiver may either explicitly feedback the full size channel state information or conduct antenna selection computation and feedback the selected antenna indices in a subsequent TXOP...." | Counter – only the first suggested change is needed, and it should appear in the bullet item b), where the ASEL transmitter behavior is described – editor shall make the changes shown for CID 864 in document 11-07-2304r2. NEED to re-open – see 11-07-0589r1 |
| 1719 | Nanda, Sanjiv | 163.13 | 9.19.3. | C | Contradiction between the two dashed items. If the previous frame at the receiver was a transmission by the RD responder, then by the first item, the source of the NDP is equal to the TA. By the second item, the transmission was not be the TXOP holder, the source of the NDP is equal to the RA. | Needs to be fixed, so that the second item is "not the TXOP holder, nor the calibration responder, nor a RD responder"… | Counter – see the changes created by document 11-07-2056r1, which removes the confusion. |
| 2432 | Stephens, Adrian | 159.27 | 9.18.2 |  | "The transmitter sends out consecutive sounding PPDUs separated by SIFS in a TXOP using burst transmission with no ACK" There no such thing as "burst transmission" defined. Also, what goes in the PPDUs? Does it matter? | Define fully what MPDUs are transmitted in these PPDUs. | Counter: Delete the words "using burst transmission” from the sentence at about line 18 of page 157 of TGn draft D2.05 (in subclause 9.18.2, first item b)). Note that the contents of the PPDUs does not matter as long as they are sounding frames and do not expect immediate ACK. |
| 2433 | Stephens, Adrian | 159.34 | 9.18.2 |  | "If the transmitter allows antenna indices feedback (by setting the ASEL command field to 0)" I don't see how the control of feedback is provided by the definition for the ASEL comment field set to 0 value, which is TXASSI. | Relate to the names defined in table n13. | Counter – change the words “command field to 0” to “command field to TXASSI” at about line 27 of page 157 of TGn draft D2.05 – and also note the changes to the first item b) of this subclause due to CID 864 which clarify that specific command values allow or require specific receiver behavior, including the allowance to send antenna indices as feedback. |
| 2435 | Stephens, Adrian | 159.58 | 9.18.2 |  | "When providing channel information, an Action No Ack +HTC frame (defined in 7.2.3.12a (Action No Ack frame format)) is used to carry the MIMO CSI Matrices frame feedback defined in 7.4.9.6 (MIMO CSI Matrices frame format)." I see no reason to prevent antenna selection from using an acknowleged frame to carry feedback if the transmitter so wishes. This is particularly important because the report can be segmented across multiple large frames. The probability of error somewhere in the report is high, and the whole report is useless if one segment is missing. | Replace with: "Channel state information is transported using the MIMO CSI Matrices frame defined in 7.4.9.6 (MIMO CSI Matrices frame format)." | Counter - Replace the sentence that begins with “When providing channel information” at about line 53 page 157 of TGn draft D2.05 with: "Channel state information is transported using the MIMO CSI Matrices frame defined in 7.4.8.6 (CSI Matrices frame format) contained within either an Action No Ack +HTC or Action frame." |
| 2436 | Stephens, Adrian | 160.01 | 9.18.2 |  | "When providing antenna indices an Action No Ack +HTC frame (defined in 7.2.3.12a (Action No Ack frame format)) is used to carry antenna selection indices feedback as defined in 7.4.9.9 (Antenna Selection Indices Feedback frame format). One octet of the antenna selection indices field is used to carry the selected antenna indices feedback." Why do we need to constrain to the no-ack case. Also this is inconsistent with 7.4.9.8 that permits either frame type. Also, why do we care that one octet of the antenna selection indices field carries the feedback? This is structural information provided adequately in clause 7 and not relevant here. | Reword thus: "Antenna indices feedback is carried in the Antenna Selection Indices Feedback frame, defined in 7.4.9.9." | Counter: Replace the sentence that begins with “When providing channel information” at about line 61 page 157 of TGn draft D2.05 with: "Antenna indices feedback is carried in the Antenna Selection Indices Feedback frame, defined in 7.4.8.9.” and modify 7.4.8.9. to state that this frame can be of either type/subtype Action No Ack or Action. |
| 2437 | Stephens, Adrian | 160.07 | 9.18.2 |  | "In the case that the receiver does not correctly receive the sounding PPDUs, or the current feedback becomes stale" I'm not sure it makes sense to overload these reasons onto one code. There is a reserved code that could be used for "ASEL feedback stale" | Add in table n3 a code for "ASEL feedback stale" Reword the sentence starting line 7 to describe the use of this code as well as the "did not correctly receive" as follows: "In the case that the receiver does not correctly receive the sounding PPDUs, the receiver transmits a +HTC MPDU with the MAI field set to ASELI and the ASEL Command field set to ASEL Training Failure. The receiver notifies the transmitter that the ASEL feedback is stale by transmitting a +HTC MPDU with the MAI field set to ASELI and the ASEL Command field set to ASEL Feedback Stale" | Counter – the response of the transmitter to either a training failure or an indication of stale feedback is the same – i.e. a new training session is initiated. As such, no new command code is needed to indicate stale feedback. However, the point should be made clear in the table that describes the use of each ASEL Command value in clause 7. Editor shall execute changes for CID 2437 as found in 11-07-2304r5. |
| 2438 | Stephens, Adrian | 160.09 | 9.18.2 |  | +HTC setting MAI to 14 - magic numbers | Replace with "+HTC setting MAI to the value ASELI (see Table n1)." | Accept |
| 2439 | Stephens, Adrian | 160.09 | 9.18.2 |  | "and the command part in ASELC to 6 to indicate the failure of antenna selection training process." But ASELC=6 means something else. Probably it should be 5 (ASEL training failure) I don't like embedding these numbers because they're invariably wrong. | Replace all ASEL magic numbers in 9.18 with names from table n13. | Accept |
| 2442 | Stephens, Adrian | 160.41 | 9.18.2 |  | "The receiver sends out a frame + HTC setting RX ASEL sounding request in the command part of ASELC subfield in HT Control Field, and the data part in ASELC to indicate the number of total sounding PPDUs required." Wrong terminology. | reword thus: "The receiver transmits a +HTC frame with the MAI field set to ASELI, with the ASEL Command field set to RXASSR and the ASEL Data field set to the number of sounding PPDUs required." | Accept |
| 2443 | Stephens, Adrian | 160.50 | 9.18.2 |  | "The transmitter responds with the corresponding number of sounding PPDUs or NDPs in its subsequent TXOP, using burst transmission with no ACK, with setting RX ASEL sounding indication, and sounding frame format." Issues: 1. An NDP *is* a sounding PPDU 2. "Burst transmission" is not a defined term 3. "with setting" is ungrammatical 4. The names do not relate to table n3. 5. There's no such thing as "sounding frame format" | Replace with the following: "The transmitter responds with the corresponding number of sounding PPDUs in its subsequent TXOP. These PPDUs are separated by SIFS. When using non-NDP sounding, each PPDU contains a +HTC frame in which the MAI field is set to ASELI, the ASEL Command field is set to RXASSI and the ASEL Data subfield is set to the remaining number of sounding PPDUs to be transmitted. When using NDP sounding, the PPDU that precedes the first NDP contains a +HTC frame in which the NDP Announce field is set to 1, the MAI field is set to ASELI, the ASEL Command field is set to RXASSI and the ASEL Data field is set to the remaining number of sounding PPDUs to be transmitted" | Counter – as suggested by the commentor, except that “Announce” shall be replaced by “Announcement” in the suggested replacement text. |
| 2444 | Stephens, Adrian | 160.55 | 9.18.2 |  | "these sounding PPDUs or NDPs," - an NDP is a sounding PPDU | reword: "these sounding PPDUs," Ditto change line 56 | Accept |
| 2447 | Stephens, Adrian | 160.64 | 9.18.2 |  | "and TX and RX antenna selection training shall be done alternately." What does this mean? The statement is ambiguous. Also this appears to be a normative requirement placed jointly on the STA at each end of a link. This is not possible. | Reword in terms that define the normative requirements for each end of the link independently. | Counter – Delete the phrase “and Tx and Rx antenna selection training shall be done alternately” that appears at the end of the last paragraph of subclause “9.18.2 Procedure” at about line 11 of page 159 of TGn Draft D2.05 – note that the restriction intended by the deleted phrase is implicit in the descriptions of transmitter and receiver behavior, where TXOP holder or its TXOP are mentioned. See also CID 863 and its resolution. |
| 2448 | Stephens, Adrian | 161.19 | 9.18.2 |  | "A STA shall use the Control Wrapper frame (7.2.1.9 (Control Wrapper frame)) to announce NDP in non-HT PPDU control frames." Seeing as both HT and non-HT must use the control wrapper frame for +HTC control frames, this statement is incomplete and possibly misleading. | Delete it. | Accept |
| 2452 | Stephens, Adrian | 161.28 | 9.18.2 |  | "If the reception immediately preceding (the frame ending at SIFS from the start of the NDP) the NDP has an FCS error, the NDP shall be dropped by the receiver." "shall be dropped". This is not an adquately precise normative requirement. It's also unnecessary, because the STA cannot determine that any NDP was addressed to it. So it will ignore the NDP in the same way that it will ignore NDPs sent to other STA. | Remove the quoted sentence. | Accept |
| 3235 | Erceg, Vinko | 159.00 | 9.18.2 |  | This paragraph apparently does not allow for delayed CSI feedback response. | Please add such a feature. | Accept – Editor shall make the changes shown in 11-07-2304r6 for CID 3235 that explicitly indicate that all feedback is in a subsequent TXOP. |
| 3236 | Erceg, Vinko | 160.01 | 9.18.2 |  | Bullet item 2) says that the feedback must be in an Action No Ack frame -- this seems to contradict the description on page 159, lines 54-55, where it says that feedback may either "explicitly feedback" ... or "feedback...in a subsequent TXOP" - I think that it should say "immediately feedback" in stead of "explicitly feedback" on page 159, line 54 and the suggestion of a new TXOP for feedback implies that the feedbacker will initiate a TXOP. As such, one would expect that this feedback in the new TXOP initiated by the feedbacker would be suitably transmitted within a normal, ack-able action frame. | Change item 2) to allow a regular action mgmt frame to be used in addition to the action no ack frame for feedback purposes, with a dependency on whether the feedback is immediate (as a SIFS response) or in a new TXOP | Counter - see CID 2435 and CID 3235. Only delayed feedback is allowed. |
| 3237 | Erceg, Vinko | 159.00 | 9.18.2 |  | In the antenna selection case, receiver may want to initiate antenna selection sequence retry by the transmitter in the case the receiver determines that training is not valid (usable) because of the bursty interference , for example. | Allow for all zeros to be returned as antenna selection feedback after an antenna selection training sequence - this may effectively generate a retry on the part of the transmitter because the receiver determined that none of the selections provided a descent signal, perhaps due to an interference event that occurred during the training. Basically the need here is to establish that an all zeros value is not an error, and to establish what should be done when it occurs (see 7.3.1.33, pg 52). | Reject – training failure error code already exists |
| 3238 | Erceg, Vinko | 159.00 | 9.18.2 |  | If TX antenna selection fails up to the kth packet, there is no way for the receiver to signal the transmitter to start from the (k+1)th packet. The way the current spec is written the transmitter would have to start from the beginning. | Allow for starting from (k+1)th packet either with a single "restart value" perhaps as an ASEL DATA value associated with the ASEL command "failure" or by providing for selective sounding packet failure indication by using, for example a bit field in a new HT PHY action frame with 16 bits indicating which of the possible 16 sounding frames was NOT received properly. |  |
| 3386 | Madhavan Pillai, Krishna Sankar | 160.43 |  |  | Typo: "…command part in ASELC to 6 to indicate the failure of antenna selection training process." Reason:Per Table n3, ASEL Command 5 indicates ASEL Training failure | Change to "…command part in ASELC to 6 5 to indicate the failure of antenna selection training process." | Counter – see CID 2439 which addresses the same error, but corrects by replacing the numerical value with the mnemonic value. |

Comments being readdressed:

| 433 | Chaplin, Clint | 160.60 | 9.18.2 | C | "should switch its antennas.When both" missing space | "should switch its antennas. When both" | New resolution: Counter: First sentence of the pair is modified and moved to a different location as per CID 714. Old resolution: EDITOR: 2007-04-02 13:22:34Z Accept |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 2180 | Stephens, Adrian | 108.04 | 9.6.5 | C | "The basic STBC MCS is the MCS of lowest rate in the Basic MCS Set used with STBC encoder with one spatial stream and two space time streams" - ungrammatical | Reword thus: "The basic STBC MCS is the MCS of lowest rate in the Basic MCS Set STBC encoded with one spatial stream and two space time streams" | New Resolution: Wording is modified further, to avoid confusion over whether an STBC MCS is actually present in the basic MCS set subfield, as per changes from 11-07-2304r5 arising from CID 574. Old Resolution: EDITOR: 2007-03-28 09:44:57Z Counter - Reworded the proposal slightly thus: "The term “basic STBC MCS” refers to the MCS of the lowest rate in the Basic MCS Set, which is also STBC encoded with one spatial stream and two space time streams. " |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 562 | Dorsey, John | 108.06 | 9.6.5 | C | The meaning of the phrase "the basic STBC MCS value is replaced by the MCS of the lowest rate…" is unclear. (Later clauses refer to the basic STBC MCS value, so it can't be "replaced".) | Change to "the basic STBC MCS value is defined to be the MCS of the lowest rate…." | New Resolution: Editor shall also mention receive STBC capability and the non-nullness of the basic MCS set, as per changes from 11-07-2304r5 arising from CID 574. Old Resolution: EDITOR: 2007-03-28 09:56:44Z - Counter. Replace the whole sentence with the following: "This term is only defined when the transmit capabilities indicated in the HT Capabilities info field allow the use of an STBC encoder, otherwise it is undefined." |
| --- | --- | --- | --- | --- | --- | --- | --- |

CID 574:

TGn Editor: Insert the following text to become the new last paragraph of subclause “9.6.2 Basic MCS Set” of TGn Draft D2.05 at about page 107 line 23 as follows:

An AP that transmitts the value one for the TX STBC subfield of the HT Capabilities info field of the HT Capabilities element in a frame shall include at least one MCS that has only one spatial stream in the Basic MCS Set field of the HT Information element of that frame, if that element is present.

TGn Editor: Change the editing instruction and the row that contains “BSSBasicMCSSet (#1884)” for the “Name” column in the table that is found in subclause “10.3.10.1.2 Semantics of the service primitive” (for the service primitive MLME-START.request) of TGn Draft D2.06 at about page 180 line 1 as follows:

Insert the following seven entries in the table in 10.3.10.1.2 immediately before the row for VendorSpecificInfo:

| BSSBasicMCSSet(#1884) | Set of integers | Each member of the set takes a value in the range 0 to 76, representing an MCS index value | The set of MCS values that shall be supported by all HT STAs to join this BSS. The STA that is creating the BSS shall be able to receive and transmit at each of the MCS values listed in the set. If the HT Capabilities parameter includes a value of “1” for the TX STBC subfield, then the BSSBasicMCSSet parameter shall include at least one integer value in the range 0 to 7. |
| --- | --- | --- | --- |

TGn Editor: Change the text of subclause “9.6.3 Basic STBC MCS” of TGn Draft D2.05 at about page 107 line 27 as follows:

The term “basic STBC MCS” refers to the MCS of the lowest rate in the Basic MCS Set, which is subsequently STBC encoded with one spatial stream and two space time streams. (#2180)

This term is only defined when the transmit capabilities indicated in the HT Capabilities info field transmitted by the AP allow the use of an STBC encoder and the receive capabilities indicated in the HT Capabilities info field transmitted by the AP allow the use of an STBC decoder and the Basic MCS Set is not empty, otherwise it is undefined. (#562)

TGn Editor: Change the sentence that begins with “The AP may transmit an STBC Beacon” found in subclause “11.1.2.1 Beacon generation in infrastructure networks” of TGn Draft D2.05 at about page 177 line 53 as follows:

An AP whose last transmitted values for the TX STBC subfield and RX STBC subfield of the HT Capabilities info field of the HT Capabilities element are both one may transmit an STBC Beacon (#1191) frame and broadcast/multicast traffic using the basic STBC MCS, as defined in 9.6.3 (Basic STBC MCS (#2806))

CID 714:

TGn Editor: Delete the first sentence of the last paragraph of subclause “9.18.2 Procedure” of TGn Draft D2.05 on about page 159 line 6.

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of subclause “9.17.2.2 Unidirectional implicit transmit beamforming” of TGn Draft D2.05 at about page 148 line 37:

During the PPDU exchange, neither the receiving nor the transmitting STA should switch antennas.

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of subclause “9.17.2.3 Bidirectional implicit transmit beamforming” of TGn Draft D2.05 at about page 150 line 5:

During the PPDU exchange, neither the receiving nor the transmitting STA should switch antennas.

TGn Editor: Insert the following text as a new paragraph to appear after the last paragraph of subclause “9.17.2.4.2 Procedure” of TGn Draft D2.05 at about page 154 line 5:

From the beginning of STEP 1 of the calibration procedure and continuing through the end of STEP 2 of the calibration procedure, neither the receiving nor the transmitting STA should switch antennas.

CID 3235:

TGn Editor: Change the text in paragraph d) of subclause “9.18.2 Procedure” of TGn Draft D2.06 at about page 165 line 45 as follows:

If the ASEL Command field in the sounding frames is set to 6, then after receiving all the sounding PPDUs, the receiver shall respond with (#2434) the full size channel state information in a subsequent TXOP. If the ASEL Command field in the sounding frames is set to 0, then after receiving all the sounding PPDUs, the receiver may either respond with (#2434) the full size channel state information in a subsequent TXOP, or conduct antenna selection computation and provide (#2434) the selected antenna indices in a subsequent TXOP.

(#2434)

References: