IEEE P802.11
Wireless LANs

| LB97 CID 1945 Dual CTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Conexant | Oudegracht 3a, Utrecht, Netherlands | +31 65 183 6231 | menzo.wentink@conexant.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains proposed resolutions for some LB97 CIDs that relate to the CIDs from the Dual CTS tab of the MAC spreadsheet.

| 1945 | Smith, Matt | 99.08 | 9.2.2.5a | Dual CTS protection is only needed in the case where STBC-capable STAs associated to the HT BSS are unable to receive non-STBC frames (such as RTS/CTS). Given that STBC provides no increase in total range, but rather improvement only in what rates are possible at a given range, and given that all STBC-capable STAs are required to receive non-STBC frames, this mechanism is not needed, and the resulting overhead should be eliminated. | Remove the "Dual CTS protection" section and all references to it. Also remove the corresponding signalling bits in the HT Information element. | Reject - See 07/523r0 Dual CTS is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - It is optional, adds no complexity at STAs which do not support this, and little complxity at STAs which do support this |
| --- | --- | --- | --- | --- | --- | --- |
| 1656 | Myles, Andrew | 99.08 | 9.2.5.5a | Dual CTS protection provides no range improvement relative to DSSS at 2.4 GHz. It provides only 1 dB (or 7%) range improvement at 5 GHz. That feature does not justify all the complexity and airtime overhead. | Remove Dual CTS Protection from the spec. | Reject - See 07/523r0 Dual CTS is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - It is optional, adds no complexity at STAs which do not support this, and little complxity at STAs which do support this |
| 1654 | Myles, Andrew | 74.34 | 7.3.2.50 | Dual CTS protection is completely unnecessary at 2.4 GHz because the DSSS PHY modes are more robust than the 6.5 Mbps STBC mode. They only provide 1 dB of link gain at 5 GHz. | Remove Dual CTS Protection from the spec. | Reject - See 07/523r0 Dual CTS is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - It is optional, adds no complexity at STAs which do not support this, and little complxity at STAs which do support this |
| 841 | Lemberger, Uriel | 99.08 | 9.2.5.5a | The current definition of STBC does not extend the range so substantially that justify special measures to protect STBC stations; so there is no need to worry about protection of the STA that are in the range not covered by regular protection. | Remove the dual CTS protection feature from the spec | Reject - See 07/523r0 Dual CTS is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - It is optional, adds no complexity at STAs which do not support this, and little complxity at STAs which do support this |
| 646 | Ji, Lusheng | 99.54 | 9.2.5.5a | is this CF-end non-STBC? | Specify. | Accept. To reach the AP and for resetting the NAV to be symmetric with the setting the NAV, the CF-End must be sent with the same modulation as the RTS of the Dual CTS protection exchange. Modify as follows: At line 7 of page 98 of TGn draft D2.05, replace "lowest Basic Rate" with "same modulation as the RTS" and at line 8 of page 98 of TGn draft D2.05, replace "its CF-End frame duration at the lowest basic rate" with "the CF-End". |
| 1525 | Morioka, Yuichi | 98.07 | 9.2.5.4 | Unaware (of whether or not Dual CTS is required or not) STA including legacy STA would not know whether or not Dual CTS Protection is required or not. Therefore these STAs would reset the NAV after 2 x SIFS + CTS_Time + Margin, even when dual CTS is used. This would lead to collision, and is a crucial when there is legacy OBSS. | Remove the words "When Dual CTS Protection is not required", from line 7 of line 98, or explain what an unaware STA should do. Resolve the issue with unaware STAs resetting the NAV before the second CTS is being sent. | Counter – Editor shall make changes as shown for CID 1525 within document 11-07-2305r0. The issues raised by the comment are addressed by these changes. |
| 1655 | Myles, Andrew | 98.07 | 9.2.5.4 | Have we created a causality problem by adding the following text: "When Dual CTS Protection is not required by the AP of a BSS, then a". Non-HT devices have no concept of dual CTS protection. Therefore they would be unable to comply with the remainder of the paragraph. | Either: 1. Remove Dual-CTS protection from the spec, Or: 2. Make it a condition that an AP can require Dual-CTS protection only if every STA in the BSS is an HT STA. | Reject. No new requirements are imposed on legacy clients. |
| 1657 | Myles, Andrew | 99.39 | 9.2.5.5a | The paragraph "If dual CTS Protection ……………… to allow for a collision detect" does not seem to make sense | Double check the text and if it does make sense, provide some reasoning for the policy. | Counter. In TGn draft D2.05, at page 97 line 56 change "shall continue PIFS after the CTS" to "may continue PIFS after the CTS" to make the collision detect optional. |
| 2153 | Stephens, Adrian | 99.44 | 9.2.5.5a | "The protection frames shall set a NAV for the whole duration of the transmission, covering dual CTS transmissions." The rules for Duration/ID are specified elswhere. This creates an exception to those rules. Moreover "whole duration of the transmision" is ambiguous. | Modifiy the rules for Duration/ID setting in clause 7 to account for the Dual CTS case and remove the quoted sentence. | Counter. Remove the quoted sentence from page 97 line 59 of TGn draft D2.05 and change the text appearing at page 23, line 48 of TGn draft D2.05 (7.2.1.1 RTS frame format) from "If the NAV protection is desired for only the first” to “If the RTS is not part of a dual CTS exchange and if the NAV protection is desired for only the first” |
| 2155 | Stephens, Adrian | 99.54 | 9.2.5.5a | "the STA may indicate truncation of the TXOP by transmitting a CF-End" This is either a repetition of the TXOP Truncation rules or an exception to them. | Move the text starting page 99 line 53 ending page 100 line 13 (4 paras) to 9.13.6.2. | Accept. |
| 2156 | Stephens, Adrian | 99.60 | 9.2.5.5a | "The transmission of a CF-End frame shall be interpreted by the other STAs that are capable of receiving it as a NAV reset." This is stated elsewhere. This is a duplication. | Delete quoted text. | Accept. |
| 2157 | Stephens, Adrian | 99.64 | 9.2.5.5a | "On receiving a CF-End frame from a STA with a matching BSSID, an AP shall respond with dual CF-End frames, one CF-End frame that is an STBC frame at the basic STBC MCS and one CF-End frame that is a non-STBC frame at the lowest basic rate, after a SIFS duration." This is too general. It requres a non-HT AP to do this. It requires an HT AP to do this even if there are no STBC STA. | Make it a rule when dual CTS protection is enabled. | Counter. On page 98 line 16 of TGn draft D2.05, replace "an AP shall respond with dual CF-End frames" with "an AP whose last transmitted HT Information element contained the Dual CTS Protection field set to 1 shall respond with dual CF-End frames". |
| 2804 | Surineni, Shravan | 99.10 | 9.2.2.5a | Unnecessary burden on STAs to start every TXOP with an RTS when dual protection subfield is set to '1' | Remove this restriction | Reject. Dual CTS is enabled when there is a mix of STBC and non-STBC stations, and both sets of stations need to know when TXOPs are active and when not. |

CID 1525:

TGn Editor: Insert the following text as a new paragraph to appear after the paragraph that begins with the words “If dual CTS Protection is enabled, the AP shall protect STBC TXOPs with a CTS-to-self” and is found in subclause “9.2.5.5a Dual CTS protection” of TGn Draft D2.05 at about page 97 line 57:

To avoid the restting of NAV by stations which have set their NAV due to an RTS, but then do not hear the CTS2, the non-AP STA may create a non-resetable NAV at the receiving STAs by initiating the TXOP with a legacy CTS addressed to the AP. The AP does not set a NAV in this case, and will be able to respond to the following RTS. The NAV at receiving STAs will not be updated by the RTS because its duration does not exceed the duration of the preceding CTS, and subsequently, the NAV cannot be reset during CTS2. A CTS addressed to the AP may also be transmitted prior to an STBC RTS, to set a non-resetable NAV at receiving stations. This technique allows NAV protection to be established while preventing the AP from setting a NAV. A No-ACK-QoS-Null frame addressed to the AP may be used instead of a CTS in this case. If +HTC is supported at the AP, the STA may send the RTS inside of a control wrapper. A NAV update that is the result of the reception of a wrapped RTS cannot be reset by a STA, because it was not updated by an RTS.

References: