IEEE P802.11
Wireless LANs

| TGk D8.0 SB Resolution for Admission Capacity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-AUG-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | Joekwak @sbcglobal.net |

Abstract

This document presents the proposed resolution for TGk Sponsor Ballot comment CID#246 assigned to Joe Kwak. The included text is based on D8.0.  WORD track changes is used to show draft changes needed to resolve the above listed comments.

P54L31:  Modify paragraph in 7.3.2.43 as shown:

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the  amount of medium time available using explicit admission control for the corresponding UP or AC traffic, in units of 32 microsecond periods per 1 second. See 11.10.9 for further details.

Insert new clause as shown:

### 11.10.9 BSS Available Admission Capacity

BSS Available Admission Capacity provides a means for an AP to advertise admission capacity  available for explicit admission control of any UP or AC category.  This information may assist STAs in making BSS transition decisions.

The transmitted BSS Available Admission Capacity value represents a proportion of time on the wireless medium scaled linearly in units of 32usec/sec from 0 (0% available time) to 31,250 (100% available time). A STA may use the formula in K.2.2 to calculate medium time for TSPECs and for Available Admission Capacity values transmitted in the BSS Load and BSS Available Admission Capacity elements. Available Admission Capacity values are dynamic in a BSS and the transmitted values may not indicate the actual current values used by the AP for explicit admission control. An AP shall recalculate current Available Admission Capacity values as needed.  An AP should only transmit current or recent Available Admission Capacity values.  If an AP transmits a BSS Load element, the values for any transmitted BSS Available Admission Capacity values shall be less than or equal to the current BSS Load value.

An Available Admission Capacity value of 0 transmitted in the BSS Load element indicates that no admission capacity is available and that no explicit admissions will be granted by the AP.  An Available Admission Capacity value of 0 transmitted in the BSS Available Admission Capacity element indicates that no admission capacity is available and that no explicit admissions will be granted by the AP for that UP or AC category.  A STA should not request admission for any TSPEC at an UP or AC which requires more medium time than is available for the requested UP or AC.  An AP which receives a TSPEC admission request for medium time which exceeds the current available admission capacity for the requested UP or AC category may not admit the requested TSPEC.  An AP which receives a TSPEC admission request for medium time which is less than or equal to the current available admission capacity for the requested UP or AC category should admit the requested TSPEC and may recalculate the available admission capacities according to local admission policy.