IEEE P802.11
Wireless LANs

| TGk D8.0 SB Resolution for Admission Capacity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-Aug-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | Joekwak @sbcglobal.net |
| Brian Hart | Cisco Systems | 170 W Tasman Dr, San Jose, CA 95134 | 408-5253346 | brianh@cisco.com |

Abstract

This document presents the proposed changed text needed to satisfy Sponsor Ballot Resolution CID#246 assigned to Joe Kwak. The included text is based on D8.0.  WORD track changes is used to show draft changes needed to resolve the above listed comments.

P54L31:  Modify paragraph in 7.3.2.43 as shown:

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the  amount of medium time available using explicit admission control for the corresponding UP or AC traffic, in units of 32 microsecond periods per 1 second. See 11.10.9 for further details.

Insert new clause as shown:

IEEE P802.11
Wireless LANs

| TGk D8.0 SB Resolution for Admission Capacity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-AUG-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | Joekwak @sbcglobal.net |

Abstract

This document presents the proposed resolution for TGk Sponsor Ballot comment CID#246 assigned to Joe Kwak. The included text is based on D8.0.  WORD track changes is used to show draft changes needed to resolve the above listed comments.

P54L31:  Modify paragraph in 7.3.2.43 as shown:

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the  amount of medium time available using explicit admission control for the corresponding UP or AC traffic, in units of 32 microsecond periods per 1 second. See 11.10.9 for further details.

Insert new clause as shown:

### 11.10.9 BSS Available Admission Capacity

BSS Available Admission Capacity provides a means for an AP to advertise admission capacity  available for explicit admission control of any UP or AC category.  This information may assist STAs in making BSS transition decisions.

The transmitted BSS Available Admission Capacity value represents a proportion of time on the wireless medium scaled linearly in units of 32usec/sec from 0 (0% available time) to 31,250 (100% available time). If an AP transmits a BSS Load element, the values for any transmitted BSS Available Admission Capacity values shall be less than or equal to the current BSS Load value. If an AP transmits a BSS Available Admission Capacity element, the transmitted values shall be current or recently calculated. The AP shall recalculate Available Admission Capacity values according to local policy. An Available Admission Capacity value of 0 transmitted in the BSS Available Admission Capacity element indicates that no admission capacity is available at the calculation time and that no explicit admissions will be granted by the AP for that UP or AC category at the calculation time.  An AP which receives a TSPEC admission request for medium time which is less than or equal to the current available admission capacity for the requested UP or AC category local policy may apply additional local policy before admitting the requested TSPEC.

Note: Available Admission Capacity values are dynamic in a BSS and the transmitted values cannot always reflect the actual values currently used by the AP for explicit admission control. Thus an AP should recalculate the Available Admission Capacity values regularly or after changes in the environment or the admitted capacity.

Note: STAs are advised that requesting admission for any TSPEC at an UP or AC which requires more medium time than is reported as available for the requested UP or AC is possible yet unlikely to be successful.