IEEE P802.11
Wireless LANs

| MIB Descriptions for Annex Q |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Richard Paine | Boeing | 6115 72nd Dr NE Marysville, Wa 98270 | 206-854-8199 | richard.h.paine@boeing.com |

Abstract

This document contains the MIB desriptions for the new 11k MIB in Annex Q:

Editor: Remove the present MIB description (P125L3) and replace it with the following:

-- **********************************************************************

-- * dot11RRMRequest and dot11RRMReport Usage

-- *

-- * The dot11RRMRequest and dot11RRMReport portions of the SME MIB

-- * provide access to the Radio Measurement service. By performing

-- * SET operations on the various dot11RRMRequest MIB objects,

-- * radio measurements may be initiated directly on the local STA or

-- * on any peer station within the same BSS. Subsequently, by

-- * performing GET operations on the various dot11RRMReport MIB

-- * objects the results of the requested measurements may be

-- * retrieved.

-- *

-- * In the diagram below, a radio measurement could be initiated

-- * for STA x by performing a MIB.set operation on the SME MIB of

-- * STA x and specifying the MAC address of STA x in

-- * dot11RRMRqstTargetAdd. Additionally, it is possible to have STA x

-- * request a measurement from STA y by performing a MIB.set operation

-- * on the SME MIB of STA x and specifying the MAC address of STA y in

-- * dot11RRMRqstTargetAdd. In both cases the result of the measurements

-- * can be retrieved by performing a MIB.get operation on the SME MIB

-- * of STA x upon completion of the measurement.

-- *

-- *

-- * MIB.Set 						MIB.Set

-- * or 						or

-- * MIB.Get 						MIB.Get

-- * +---------|------	+ 				+--------|-----+

-- * | SME 	| 	| 				| SME 	| 	|

-- * | 		\ / 	| 				| 	\ / 	|

-- * | +---------+ 	| 				| +---------+ 	|

-- * | | MIB     | 	| 				| | MIB     | 	|

-- * | | 	  | 	| 				| | 	    | 	|

-- * | | 	  | 	| 				| | 	    | 	|

-- * | +---------+ 	| 				| +---------+ 	|

-- * | 			| 				| 		|

-- * | / \ 		| 				| / \ 		|

-- * | 	|     MREQUEST | 				| |   MREQUEST |

-- * +-- +-------------+ 				+----+---------+

-- * | |       MREPORT | 				| |    MREPORT |

-- * | \ /     MEASURE | Action Frames 		| \ / MEASURE 	|

-- * | 			| <--Measurement Request--> 	| 		|

-- * | 			| <--Measurement Report---> 	|		|

-- * | MLME 		| 				| MLME 		|

-- * +---------------- + 				+--------------+

-- * 	STA x 							STA y

-- *

-- *

-- * Each STA maintains a single dot11RRMRequestTable in the SME MIB

-- * used to initiate RM Measurement Requests. Each dot11RRMRequestEntry

-- * in the table represents an individual Measurement Request that

-- * makes up a complete Measurement Request Action frame.

-- * Multiple Measurement Requests may be concatenated into a single

-- * Measurement Request Action frame by setting the same

-- * dot11RRMRqstToken value into multiple dot11RRMRequestEntrys.

-- *

-- * Each row, dot11RRMRequestEntry, of the dot11RRMRequestTable

-- * provides read-create access for the initiation of a measurement

-- * request. The dot11RRMRequestNextIndex object can be used to

-- * determine which is the next row available. Each row corresponding to

-- * one measurement in the sequence is created with a dot11RRMRqstRowStatus

-- * set to notInService. Once the dot11RRMRequestEntry(s) have been

-- * created for a desired measurement sequence the corresponding

-- * dot11RRMRqstRowStatus(s) objects are set to active to indicate that

-- * the SME can trigger the appropriate MLME primitives. Upon processing

-- * the request, the SME returns the corresponding dot11RRMRqstRowStatus(s)

-- * object to notInService and are now available for additional

-- * measurement requests.

-- *

-- * After a radio measurement is complete the SME populates the RRMReport

-- * objects with the results of the measurement. Each STA maintains a set

-- * of RRMReport tables, one for each corresponding measurement type. The

-- * results of the entire measurement sequence are spread across the tables

-- * based on what types of measurements were requested. Each xxxReportEntry

-- * within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- * to the original dot11RRMRqstToken in the measurement request. So the

-- * results of the measurement can be collected by searching the appropriate

-- * xxxReportTables and retrieve any reports with the matching request

-- * token.

-- **********************************************************************