IEEE P802.11
Wireless LANs

| MIB Descriptions for Annex Q |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Richard Paine | Boeing | 6115 72nd Dr NE Marysville, Wa 98270 | 206-854-8199 | richard.h.paine@boeing.com |

Abstract

This document contains the MIB descriptions for the new RRM MIB in Annex Q:

Editor: Remove the present MIB description (P125L3) and replace it with the following:

-- **********************************************************************

-- * dot11RRMRequest and dot11RRMReport Usage

-- *

-- * The dot11RRMRequest and dot11RRMReport portions of the RRM MIB

-- * provide access to the Radio Measurement service. By performing

-- * SET operations on the various dot11RRMRequest MIB objects,

-- * radio measurements may be initiated directly on the local STA or

-- * on any peer station within the same BSS. Subsequently, by

-- * performing GET operations on the various dot11RRMReport MIB

-- * objects the results of the requested measurements may be

-- * retrieved.

-- *

-- * In the diagram below, a radio measurement could be initiated

-- * for STA x by performing a MIB.set operation on the RRM MIB of

-- * STA x and specifying the MAC address of STA x in

-- * dot11RRMRqstTargetAdd. Additionally, it is possible to have STA x

-- * request a measurement from STA y by performing a MIB.set operation

-- * on the SME MIB of STA x and specifying the MAC address of STA y in

-- * dot11RRMRqstTargetAdd. In both cases the result of the measurements

-- * can be retrieved by performing a MIB.get operation on the RRM MIB

-- * of STA x upon completion of the measurement.

-- *

-- *

-- * MIB.Set 						MIB.Set

-- * or 						or

-- * MIB.Get 						MIB.Get

-- * +---------|------	+ 				+--------|-----+

-- * | SME 	| 	| 				| SME 	| 	|

-- * | 		\ / 	| 				| 	\ / 	|

-- * | +---------+ 	| 				| +---------+ 	|

-- * | | RRM MIB | 	| 				| | RRM MIB | 	|

-- * | | 	  | 	| 				| | 	    | 	|

-- * | | 	  | 	| 				| | 	    | 	|

-- * | +---------+ 	| 				| +---------+ 	|

-- * | 			| 				| 		|

-- * | / \ 		| 				| / \ 		|

-- * | 	|     MREQUEST | 				| |   MREQUEST |

-- * +-- +-------------+ 				+----+---------+

-- * | |       MREPORT | 				| |    MREPORT |

-- * | \ /     MEASURE | Action Frames 		| \ / MEASURE 	|

-- * | 			| <--Measurement Request--> 	| 		|

-- * | 			| <--Measurement Report---> 	|		|

-- * | MLME 		| 				| MLME 		|

-- * +---------------- + 				+--------------+

-- * 	STA x 							STA y

-- *

-- *

-- * Each STA maintains a single dot11RRMRequestTable in the SME MIB

-- * used to initiate RM Measurement Requests. Each dot11RRMRequestEntry

-- * in the table represents an individual Measurement Request that

-- * makes up a complete Measurement Request Action frame.

-- * Multiple Measurement Requests may be concatenated into a single

-- * Measurement Request Action frame by setting the same

-- * dot11RRMRqstToken value into multiple dot11RRMRequestEntrys.

-- *

-- * Each row, dot11RRMRequestEntry, of the dot11RRMRequestTable

-- * provides read-create access for the initiation of a measurement

-- * request. The dot11RRMRequestNextIndex object can be used to

-- * determine which is the next row available. Each row corresponding to

-- * one measurement in the sequence is created with a dot11RRMRqstRowStatus

-- * set to notInService. Once the dot11RRMRequestEntry(s) have been

-- * created for a desired measurement sequence the corresponding

-- * dot11RRMRqstRowStatus(s) objects are set to active to indicate that

-- * the SME can trigger the appropriate MLME primitives. Upon processing

-- * the request, the SME returns the corresponding dot11RRMRqstRowStatus(s)

-- * object to notInService and are now available for additional

-- * measurement requests.

-- *

-- * After a radio measurement is complete the RRM populates the RRMReport

-- * objects with the results of the measurement. Each STA maintains a set

-- * of RRMReport tables, one for each corresponding measurement type. The

-- * results of the entire measurement sequence are spread across the tables

-- * based on what types of measurements were requested. Each xxxReportEntry

-- * within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- * to the original dot11RRMRqstToken in the measurement request. So the

-- * results of the measurement can be collected by searching the appropriate

-- * xxxReportTables and retrieve any reports with the matching request

-- * token.

Editor create new Annex Q (normative) RRM MIB

The primary interface to the Radio Resource Measurements is meant to be real-time information obtained through the request/response mechanisms of RRM.  A secondary interface to the measurements is through retention of information in the MIB.  The information, meant to be retained for later access, includes the MIB entries of Annex Q.  Non-SNMP requests for information are obtained via object IDs (OIDs) through the NDIS or “wireless” interfaces in the operating systems.  SNMP requests for information are obtained via SNMP sets and gets.

Editor move MIB text page 125 line 2 through page 165 line 36 to Annex Q.

Editor move MIB text page 171 line 40 through page 172 line 50 to Annex Q.

Editor delete dot11SMTRRMRequest, dot11SMTRRMReport and dot11SMTRRMConfig from page 171 line 32 and insert dot11RadioMeasurementCapable and dot11RadioMeasurementEnabled.