IEEE P802.11
Wireless LANs

| LB 101 comment resolution for CIDs 107, 108, 109, 1660, 1661, and 1148 – near field definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marc Emmelmann | Technical University Berlin | Einsteinufer 25 10587 Berlin, Germany | +49-30-314-24580 | emmelmann@ieee.org |

Abstract

Proposed comment resolution for LB 101 CIDs 107, 108, 109, 1660, 1661, 1148 (definition of near-field).

# Near-Field definition (3.2.29)

## Addressed CIDs:

107, 108, 109, 1660, 1661, 1148

## Comments:

| 107 | Foegelle, Michael | Y | 3.2.29 | 7,01 | MT | The definition of reactive near field is incorrect. The proposed formula has units of 1/distance as a distance value. There are a number of accepted definitions for the reactive region that depend on the size of the radiating object, etc. This definition would have to be considerably longer to properly cover the topic. | Define the reactive region as "typically within a few wavelengths of the DUT" for the most general guideline. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 108 | Foegelle, Michael | Y | 3.2.29 | 7,01 | MT | The radiating near field (r < 2 D^2/lambda) is known as the Fresnel region. | Define and label the radiating near field and remove the reference to "the extend of the Fraunhofer region", which is confusing. |
| 109 | Foegelle, Michael | Y | 3.2.29 | 7,01 | MT | This definition doesn't define the meaning of these regions, which is just as important as where they're located. | Expand the definition to indicate that objects in the reactive region actually become part of the radiating object and change its behavior, while in the Fresnel region, most of the energy does actually radiate away from the device, but is not yet coherent. |
| 1660 | Kobayashi, Mark | Y | 3.2.29 | 7,02 | T | definition of lambda in the equation should be defined | Add definition of lambda |
| 1661 | Kobayashi, Mark | Y | 3.2.29 | 7,04 | T | definition of lambda in the equation should be defined | Add definition of lambda |
| 1148 | Emmelmann, Marc | N | 3.2.29 | 7,05 | E | The term Fraunhofer region is not (directly) listed as a definition but its synonom "radiating far field" is. | Replace "Fraunhofer region" by "radiating far field". |

## Resolution:

Counter (accept in principle) CIDs: 107, 108, 109, 1660, 1661, 1148

New definition incorporated all suggested changes.

### Original (D1.0) Draft Text:

3.2.29 Near-field: The region of space bounded on the outside by a distance r from an antenna or radiating

object with a largest dimension of D such that r < 2D2/λ. This is divided into the reactive near-field, which is

the region of space immediately surrounding the antenna, bounded by a distance r from an antenna such that

0 < r < 2π/λ; and the radiating near-field, which extends from the boundary of the reactive near-field to the

extent of the Fraunhofer region.

### Proposed Change:

3.2.29 Near-field: The close-in region of an antenna / radiating object where the angular field distribution is dependent upon the distance from the antenna. It is typically bounded on the outside by a distance r from an antenna or radiating object with a largest dimension of D such that r < 2D^2/λ, where λ is the wave length of the emitted radiation pattern. The near-field is divided into the reactive near-field and the radiating near field. For the most general guideline, the reactive near-field is typically within a few wavelengths of the DUT, i.e. r < λ and objects within it seem to become part of the radiating object and change its (radiation) behavior. Accordingly the radiating near field is the most general case described
by λ < r < 2D^2/ λ.

References:

[1] Near Field Definition, Agilent, http://www.home.agilent.com/agilent/redirector.jspx?action=doc&lc=eng&cc=US&id=423949&ltype=External%20File

[2] Hargrave’s Communication Dictionary, IEEE Press 2001.