IEEE P802.11
Wireless LANs

| 802.11.2/D1.0 LB 101 Proposed Comment Resolution for Annex D Theoretical Throughput Limits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marc Emmelmann | Technical University Berlin | Einsteinufer 25 10587 Berlin, Germany | +49-30-314-24580 | emmelmann@ieee.org |
| Larry Green | Ixia | 402 E. Gutierrez Street Santa Barbara, CA 93101 | +1-818-444-2901 | lgreen@ixiacom.com |

Abstract

This document contains proposed comment resolutions for those CIDs of LB 101 addressing Annex D – Theoretical Throughput Limits.

This document contains the comments and proposed resolutions. It is accompanied by document 11-07/2345 which contains the draft text of 802.11.2-D1.0 Annex D and highlights the changes introduced by adopting the comment resolutions as proposed hereafter.

This submission addressed the following comment CIDs of LB 101:

987, 467, 240, 1356, 237, 932, 1357, 1599, 1358, 1359,

1360, 1361, 1362, 1363, 1364, 1365, 1366, 1367, 1368, 1369,

1370, 1371, 1372, 1373, 1374, 1375, 56, 1600, 227, 1601,

1376, 1377, 1378, 1379, 1380, 1381, 1382, 1383, 1384, 1385,

1386, 1387, 1602, 1388, 1389, 1390, 1391, 1392, 1393, 1394,

1396, 1603

# Comments suggesting to remove Annex D

| 987 | Kolze, Thomas | Y | D | 153,01 | T | This section is only pass/fail, which is out of scope. | Remove Annex D |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 467 | Victor, Dalton | Y | D | 153,01 | T | Is this section really necessary? It will just be used as pass/fail criteria, which this draft is not supposed to include. | Remove Annex D |

## Suggested Resolution:

Reject CIDs 987 and 467.

Annex D does not provide pass/fail criteria.  It is informative and provides a methodology for determining upper boundaries and limits to performance.   The TTL methodology is useful for performance prediction and deployment of IEEE 802.11a/b/g networks.  Numerous organiziations have contacted the authors for clarification and assistance with calculations, indicating strong industry acceptance of Annex D.

The introduction clearly states that maximum throughput cannot expected to be achieved (“In practice, MSDU data throughput will be significantly lower …”, p153 ll 18-19 of 802.11.2/D1.0).  Hence it is obvious that any tested device can hardly ever achieve this theretical limit and thus, Annex D does not provide pass/fail criteria.

Note: If the commenters belive that this intention is not clearly enough expressed in the introduction of Annex D, they are invited to provide suggest text changes expressing their concern.

# (2) MAC overhead not included

| 240 | Sharma, Neeraj | N | D | 153,05 | T | I wonder why MAC overhead is not included? As a user you cannot separate out the MAC from the PHY overhead. I understand it would be complicated to account for all the possible scenarios in the detailed TTL equations, but it seems like the appendix could at least include a table with the number of MAC bytes of overhead for various scenarios. Depending on the frame type, the MAC overhead can be 30 bytes plus 4 bytes of FCS, that is a pretty significant overhead, relative to the SIFS-ACK numbers in the PHY, and relative to packets below 128 bytes or so. | Separate out the MAC overhead. A good thing would be to have one table for all the PHYs. You just add the number of MAC bytes to the Length value and use the existing equations. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Suggested Resolution

Reject Comment 240.

Performance overhead of the MAC was not included in the TTL methodology because MAC performance is highly dependent on implementation choices, rather than on the 30+4 byte header overhead.  Design choices such as receive buffer depth, internal architecture, bus interrupt mechanisms, and host memory access method will influence MAC performance much more than header overhead.

The suggestion for a PHY Table is not reachable, due to the large volume of information required.

# (3) Section D.2.12 – D3.2 Comments

| 1356 | Malarkey, Alastair | N | D.2 | 153,36 | T | The term FR is not defined | Define the term FR as Frame Rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1357 | Malarkey, Alastair | N | D.2.12 | 154,07 | T | In the subsequent clauses the term Backoff slot is used. The term Slot time is not used. | Change to Backoff Slot |
| 237 | Sharma, Neeraj | N | D.3.1 | 154,36 | MT | The text states that the number of backoff slots is selected from the range of 1 to CWmin, but this should be between 0 and CWmin. | Change 1 to 0. |
| 932 | Wentink, Menzo | Y | D.3.1 | 154,36 | T | The text states that the number of backoff slots is selected from the range of 1 to CWmin, but this should be between 0 and CWmin. | Change 1 to 0. |
| 1599 | Cypher, David | Y | D.3.2 | 154,52 | T | With the publishing of 802.11-2007 | Replace 1999 with 2007 and make all other ammendment references directly to 802.11-2007 and update subclause headers according, as well. |

## Proposed resolution:

Accept CIDs: 1356, 237, 932

Counter CID 1357: Change to “backoff” as later on the term “backoff” is used in the calculations emphasizing the time interval wheras backoff slot emphasizes one possible part of this interval.

Accept CID 1599. (Note, this also applies to p153, l 33 where references to 802.11-REVma are given).

# (4) Section D.4.1 Comments

| 1358 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | Backoff is not the correct term in FS-to-FS interval. | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1359 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1360 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | There is a value error in the DIFS equation. Backoff slot has value 50 not 28. | Use correct value of Backoff slot in calculation |
| 1361 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1362 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | The term computed in D.4.1 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions

Accept CIDs: 1358, 1359, 1360, 1361, 1362

# (5) Section D.4.2 Comments

| 1363 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1364 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1365 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1366 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | The term computed in D.4.2 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions

Accept CIDs: 1363, 1364, 1365, 1366

# (6) Section D.4.3 Comments

| 1367 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1368 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1369 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1370 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | The term computed in D.4.3 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions

Accept CIDs: 1367, 1368, 1369, 1370

# (7) Section D4.4 Comments

| 1371 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1372 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1373 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1374 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | The term computed in D.4.4 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |
| 1375 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | The term Tack is not clearly listed in table 30 | Change table title and 3rd column to use word Tack |
| 56 | Stephens, Adrian | N | D.4.4 | 156,21 | MT | Calculation is specific to 20MHz channel width. | Either state this, or state this and also add calculation for 10MHz and 5MHz channel widths. |
| 1600 | Cypher, David | Y | D.4.4 | 156,36 | T | Table 29 what is it? Is it a reference from Clause 17 of 802.11a-1999? Is it a table for Annex D clause D.4.4? | If it is an referenced table from 802.11-2007 should mark as such. If it is an annex table, it shall follow IEEE standard style manual. |
| 227 | Sharma, Neeraj | N | D.4.4 | 157,01 | E | Reference to Table 1 - should be reference to Table 30 | Change Table 1 to 30 |
| 1601 | Cypher, David | Y | D.4.4 | 157,02 | T | 1) is table 30 an annex table or a reference table from 802.11-2007? 2) is the reference "table 1" referring to table 30? | Correct so that there is no question about what is the reference or the numbering |

## Proposed Resolutions

Accept CIDs: 1371, 1372, 1373, 1374

Accept CID 1375 --- Attention, this is NOT reflected in the text provided in 11-07/2345 but should be adopted by the editor accordingly. Instructions to the editor were added to 07/2345 to reflect this situation.

Accept CIDs: 56, 1600, 227

Counter CID 1601:

The reference “table 1” has been changed to “table 30” (same as CID 227).

The table is part of the Annex and not directly taken from the 802.11-2007 standard.

INSTRUCTIONS TO THE EDITOR: check if the format of the table is in accordance to IEEE style manual.

# (8) Section D4.5 Comments

| 1376 | Malarkey, Alastair | Y | D.4.5 | 157,23 | T | The definitions of FR previously do not require the sue of the floor function. Also the brackets are incomplete. | Delete "Floor(" |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Proposed Resolutions

Accept CIDs: 1376

# (9) Section D4.6 Comments

| 1377 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | The TxTime formulation uses "'..Length + PBCC.." but the terms below are PBCC Indicator and PBCC Mode | change to "TxTime = Tpream + PLCPtime + ceiling(((Length + PBCC Indicator) * 8) / PHYrate) + Clock Switch Time" and change PBCC Mode line to "Clock Switch Time = 1 μs @ PBCC Mode of 33 Mb/s, otherwise 0." |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1378 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| 1379 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1380 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1381 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | The term computed in D.4.6 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CIDs: 1377

Accept CIDs. 1378, 1379, 1380, 1381

# (10) Section D4.7 Comments

| 1382 | Malarkey, Alastair | Y | D.4.7 | 158,11 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1383 | Malarkey, Alastair | Y | D.4.7 | 158,11 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1384 | Malarkey, Alastair | Y | D.4.7 | 158,11 | T | The term computed in D.4.7 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CIDs: 1382, 1383, 1384

# (11) Section D4.8 Comments

| 1385 | Malarkey, Alastair | Y | D.4.8 | 158,34 | T | Shouldn't this be Best-case Backoff to be consistent | Replace "Backoff" with "Best-case backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1386 | Malarkey, Alastair | Y | D.4.8 | 158,34 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1387 | Malarkey, Alastair | Y | D.4.8 | 158,34 | T | The term computed in D.4.8 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |
| 1602 | Cypher, David | Y | D.4.8 | 158,48 | T | Table 31 what is it? Is it a reference from Clause 19.5 of 802.11g-2003? Is it a table for Annex D clause D.4.8? | If it is an referenced table from 802.11-2007 should mark as such. If it is an annex table, it shall follow IEEE standard style manual. |

## Proposed Resolutions:

Accept CIDs: 1385, 1386

Note to the Editor: p159 l7 of 802.11.2-D1.0 should also read “Best-case Backoff”. This change is reflected in 11-07/2345

Accept CIDs: 1387, 1602

# (12) Section D4.9 Comments

| 1388 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | the term PBCC is used to define itself ! | Change "PBCC" to "PBCC Indicator" in TxTime formulation lines and change "PBCC:.." to "PBCC indicator:…" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1389 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| 1390 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1391 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1392 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | The term computed in D.4.9 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CIDs: 1388, 1389

Reject CID 1390:

The Draft text already includes the term “Backoff Slots”.

This comment might be a copy and paste error as the comment applied to other sections.

Accept CIDs: 1391, 1392

# (13) Section 4.10 Comments

| 1393 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | There are a number of inconsistencies in this formulation. The headers are not inlcuded in the base formulation. TpreamDSSS is not defined. Pad is not defined … | Please correct formulation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1394 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| 1395 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1396 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | The term computed in D.4.10 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |
| 1603 | Cypher, David | Y | D.4.10 | 160,03 | T | Table 32 what is it? Is it a reference from Clause 19.7 of 802.11g-2003? Is it a table for Annex D clause D.4.10? | If it is an referenced table from 802.11-2007 should mark as such. If it is an annex table, it shall follow IEEE standard style manual. |

## Proposed Resolutions:

Accept CID: 1393

Formulation of TXTIME corrected. Naming of components in alignment

with Cls. 19.8.3.3 of 802.11-2007.

Accept CID: 1394, 1395, 1396

Accept CID 1603

Table 32 clearly shows the relationship between PHY data rate and number of data bits per symbol (Ndbps) that is used in the TTL calculation.  Table 32 is a simplified subset of data in Table 17-3, 802.11-2007.

Table 32 should follow the IEEE style manual.