IEEE P802.11
Wireless LANs

| 802.11.2/D1.0 LB 101 Proposed Comment Resolution for Annex D Theoretical Throughput Limits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marc Emmelmann | Technical University Berlin | Einsteinufer 25 10587 Berlin, Germany | +49-30-314-24580 | emmelmann@ieee.org |
| Larry Green | Ixia | 402 E. Gutierrez Street Santa Barbara, CA 93101 | +1-818-444-2901 | lgreen@ixiacom.com |

Abstract

This document contains proposed comment resolutions for those CIDs of LB 101 addressing Annex D – Theoretical Throughput Limits.

This document contains the comments and proposed resolutions. It is accompanied by document 11-07/2345 which contains the draft text of 802.11.2-D1.0 Annex D and highlights the changes introduced by adopting the comment resolutions as proposed hereafter.

This submission addressed the following comment CIDs of LB 101:

1356, 237, 932, 1357, 1599, 1358, 1359,

1360, 1361, 1362, 1363, 1364, 1365, 1366, 1367, 1368, 1369,

1370, 1371, 1372, 1373, 1374, 1375, 56,

1376, 1377, 1378, 1379, 1380, 1381, 1382, 1383, 1384, 1385,

1386, 1387, 1388, 1389, 1390, 1391, 1392, 1393, 1394,

1396

# Revision History

| Revision | Changes |
| --- | --- |
| R0 | Original contribution as discussed during the TGT telephone conference calls on August 30 and September 7, 2007. |
| R1 | - Included revision history - Removed proposed resolution for those comments, TGT could not agree on during the telephone conference calls, i.e.: - CIDs 987 and 467 (proposed resolution now in 11-07/2504) - CID 240 - Use the term “average collision free backoff” instead of “best case backoff” as agreed on during the September 7 telephone conference (this relates to CIDs 1389, 1390, 1391 and the same comments made to the subsequent sections of the draft). - Removed proposed resultions for the following CIDs relating to tables 29 – 32 as they are resoved by the editor: CID 1600, 1601, 227, 1602, and 1603 |

Editorial Note: Sections (1) and (2) were removed in revision 1 of this document.

# (3) Section D.2.12 – D3.2 Comments

| 1356 | Malarkey, Alastair | N | D.2 | 153,36 | T | The term FR is not defined | Define the term FR as Frame Rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1357 | Malarkey, Alastair | N | D.2.12 | 154,07 | T | In the subsequent clauses the term Backoff slot is used. The term Slot time is not used. | Change to Backoff Slot |
| 237 | Sharma, Neeraj | N | D.3.1 | 154,36 | MT | The text states that the number of backoff slots is selected from the range of 1 to CWmin, but this should be between 0 and CWmin. | Change 1 to 0. |
| 932 | Wentink, Menzo | Y | D.3.1 | 154,36 | T | The text states that the number of backoff slots is selected from the range of 1 to CWmin, but this should be between 0 and CWmin. | Change 1 to 0. |
| 1599 | Cypher, David | Y | D.3.2 | 154,52 | T | With the publishing of 802.11-2007 | Replace 1999 with 2007 and make all other ammendment references directly to 802.11-2007 and update subclause headers according, as well. |

## Proposed resolution:

Accept CIDs: 1356, 237, 932

Counter CID 1357: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

Accept CID 1599. (Note, this also applies to p153, l 33 where references to 802.11-REVma are given).

# (4) Section D.4.1 Comments

| 1358 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | Backoff is not the correct term in FS-to-FS interval. | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1359 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1360 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | There is a value error in the DIFS equation. Backoff slot has value 50 not 28. | Use correct value of Backoff slot in calculation |
| 1361 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1362 | Malarkey, Alastair | Y | D.4.1 | 155,13 | T | The term computed in D.4.1 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions

Accept CIDs: 1360, 1362

Counter CID 1358, 1359: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

Counter CID 1361: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

# (5) Section D.4.2 Comments

| 1363 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1364 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1365 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1366 | Malarkey, Alastair | Y | D.4.2 | 155,35 | T | The term computed in D.4.2 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions

Accept CIDs:  1366

Counter CID 1365: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1363 and 1364: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

# (6) Section D.4.3 Comments

| 1367 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1368 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1369 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1370 | Malarkey, Alastair | Y | D.4.3 | 156,01 | T | The term computed in D.4.3 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions

Accept CIDs: 1370

Counter CID 1369: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1367 and 1368: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

# (7) Section D4.4 Comments

| 1371 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1372 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1373 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1374 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | The term computed in D.4.4 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |
| 1375 | Malarkey, Alastair | Y | D.4.4 | 156,21 | T | The term Tack is not clearly listed in table 30 | Change table title and 3rd column to use word Tack |
| 56 | Stephens, Adrian | N | D.4.4 | 156,21 | MT | Calculation is specific to 20MHz channel width. | Either state this, or state this and also add calculation for 10MHz and 5MHz channel widths. |

## Proposed Resolutions

Accept CIDs: 1374

Counter CID 1373: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1371 and 1372: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

Accept CID 1375 --- Attention, this is NOT reflected in the text provided in 11-07/2345 but should be adopted by the editor accordingly. Instructions to the editor were added to 07/2345 to reflect this situation.

Accept CIDs: 56

# (8) Section D4.5 Comments

| 1376 | Malarkey, Alastair | Y | D.4.5 | 157,23 | T | The definitions of FR previously do not require the sue of the floor function. Also the brackets are incomplete. | Delete "Floor(" |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Proposed Resolutions

Accept CIDs: 1376

# (9) Section D4.6 Comments

| 1377 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | The TxTime formulation uses "'..Length + PBCC.." but the terms below are PBCC Indicator and PBCC Mode | change to "TxTime = Tpream + PLCPtime + ceiling(((Length + PBCC Indicator) * 8) / PHYrate) + Clock Switch Time" and change PBCC Mode line to "Clock Switch Time = 1 μs @ PBCC Mode of 33 Mb/s, otherwise 0." |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1378 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| 1379 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1380 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1381 | Malarkey, Alastair | Y | D.4.6 | 157,42 | T | The term computed in D.4.6 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CIDs: 1377

Accept CIDs. 1381

Counter CID 1380: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1378 and 1379: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

# (10) Section D4.7 Comments

| 1382 | Malarkey, Alastair | Y | D.4.7 | 158,11 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1383 | Malarkey, Alastair | Y | D.4.7 | 158,11 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1384 | Malarkey, Alastair | Y | D.4.7 | 158,11 | T | The term computed in D.4.7 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CIDs: 1384

Counter CID 1383: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1382: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

# (11) Section D4.8 Comments

| 1385 | Malarkey, Alastair | Y | D.4.8 | 158,34 | T | Shouldn't this be Best-case Backoff to be consistent | Replace "Backoff" with "Best-case backoff" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1386 | Malarkey, Alastair | Y | D.4.8 | 158,34 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1387 | Malarkey, Alastair | Y | D.4.8 | 158,34 | T | The term computed in D.4.8 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Counter CID 1386: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Note to the Editor: p159 l7 of 802.11.2-D1.0 should also read “Best-case Backoff”. This change is reflected in 11-07/2345

Accept CIDs: 1387

Counter CID 1385: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

# (12) Section D4.9 Comments

| 1388 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | the term PBCC is used to define itself ! | Change "PBCC" to "PBCC Indicator" in TxTime formulation lines and change "PBCC:.." to "PBCC indicator:…" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1389 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| 1390 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | Slots is not the correct term in DIFS | replace "..Slots..f" with "..Backoff slots..." |
| 1391 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1392 | Malarkey, Alastair | Y | D.4.9 | 159,13 | T | The term computed in D.4.9 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CIDs: 1388,

Accept CIDs: 1392

Counter CID 1391: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1389 and 1390: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.

# (13) Section 4.10 Comments

| 1393 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | There are a number of inconsistencies in this formulation. The headers are not inlcuded in the base formulation. TpreamDSSS is not defined. Pad is not defined … | Please correct formulation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1394 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | Backoff is not the correct term in FS-to-FS interval | replace "..Backoff" with "..Best-case Backoff" |
| 1395 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | Best-case Backoff should be in microseconds to use in FS-to_FS interval formulation | Replace with "Best-case backoff = CWmin / 2 * Backoff slot =…" |
| 1396 | Malarkey, Alastair | Y | D.4.10 | 159,38 | T | The term computed in D.4.10 c) is FR. | Either replace FR in clause c above with Frame Rate or replace Frame Rate in this formula with FR. |

## Proposed Resolutions:

Accept CID: 1393

Formulation of TXTIME corrected. Naming of components in alignment

with Cls. 19.8.3.3 of 802.11-2007.

Accept CID:  1396

Counter CID 1395: (accept comment in general) accept the comment but use the term “average collision free backoff” on the left side of the equation instead.

Counter CID 1394: (Accept the comment in general). The term is replaced by “average collision free backoff” which is more appropriate.