IEEE P802.11
Wireless LANs

| FT MLME Figure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206, Hillsboro OR 97124 | +1 503 264 3759 | kapil.sood@intel.com |

Abstract

This submission proposes a resolution to comments from D7.0 Sponsor Ballot that asks for clarification of which MLME interfaces are used for FT protocol over-the-DS.

Delete 11A.10.2, Page 87, line 1, Figure 11A-18

Clause 11A.10.2 Page 86, line 62

Change: “The message flow for FT protocol over the DS is given in Figure 11A-XX”

Clause 11A.5.3, Page 57, line 36

Insert the following Figure.  “Figure 11A-XX – MLME Interfaces for Fast BSS Transition Protocol messages over-the-DS”

Non-AP STA

Current AP

Target AP

SME

SME

SME

MAC

MAC

MAC

MLME-REMOTE_

REQUEST.request

FT Action Request (IEs)

MLME-REMOTE_

REQUEST.confirm

MLME-REMOTE_

REQUEST.indication

Remote Request (IEs)

Target AP Processes Request

Remote Response (IEs)

MLME-REMOTE_

REQUEST.request

FT Action Response (IEs)

MLME-REMOTE_

REQUEST.confirm

MLME-REMOTE_

REQUEST.indication

MLME-REASSOCIATE.request

Reassociate Request (IEs)

MLME-REASSOCIATE.indication

MLME-REASSOCIATE.response

Reassociate Response (IEs)

MLME-REASSOCIATE.confirm

Figure 11A-XX – MLME Interfaces for Fast BSS Transition Protocol messages over-the-DS

Clause 11A.5.3, Page 57, line 36

Insert:  Figure 11A-XX shows the MLME interfaces on the non-AP STA, current AP, and the target AP for executing the Fast BSS Transition Protocol messages over-the-DS.