IEEE P802.11
Wireless LANs

| TGy LB109 Submission for ECSA comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., San Jose, CA 95134 | 408-527-0815 | petere@cisco.com |

Abstract

This document is aligned with P802.11-2007, P802.11k/D8.0, P802.11r/D7.0 for baseline, P802.11y/D4.0, and  submissions 07/2364r0 (misc PHY comments), 07/2382r1 LB109 non-Editorial comments, 07/2383r1 LB109 Editorial comments, 07/2384r0 LB109 Submission for DSE comments, and addresses the following LB109 comments:

CIDs: 3005, 3009, 3010, 3011, 3012, 3013, 3036, 3037, 3038, 3040, 3041, 3042, 3043, 3044, 3045, 3046, 3050, 3056, 3058, 3059, 3060, 3065, 3070, 3071, 3072, 3077, 3078, 3079, 3088, 3089, 3090, 3097, 3111, 3112, 3136, 3137, 3138, 3146, 3147, 3149, 3150, 3151, 3152, 3154, 3155, 3162, 3172, 3173, 3174, 3175, 3176, 3177, 3178, 3179, 3180, 3181, 3182, 3183, 3184, 3185 and 3186

## Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGy Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGy Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGy amendment with the baseline documents).

TGy Editor:  Editing instructions preceded by “TGy Editor” are instructions to the TGy editor to modify existing material in the TGy draft.   As a result of adopting the changes, the TGy editor will execute the instructions rather than copy them to the TGy Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Clause 7

CIDs: 3005, 3040, 3045, 3046, 3060, 3111, 3112, 3136, 3137, 3146, 3147, 3149, 3150, 3151, 3152, 3154 and 3155

Discussion:

In 11y/D4.0:

| 3146 | 7.2.3.1 | 5 | 15 | T | Y | Table 8, "Supported Regulatory Classes" IE is included in the beacon frame. However, "Supported Channels" IE is not. To be consistent, Should "Supported Channels" IE be included in the beacon frame as well? | Suggest either including both or including none. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3147 | 7.2.3.4 | 5 | 43 | T | Y | Table 10. In 802.11-2007 base draft, the Supported Channels element is present if dot11SpectrumManagementRequired is true. I assume that the Supported Channels should also be included in the Association Request frame if dot11ExtendedChannelSwitchImplemented is true. | Change the editorial note to "Change order 5 entry and insert order 11 entry into table 10 as shown below, before VendorSpecific:". Add new row before "Supported Regulatory Classses" as: Order = "5"; Information = "Supported Channels"; Notes = "The Supported Channels element shall be present if dot11SpectrumManagementRequired is true or dot11ExtendedChannelSwitchImplemented is true.". |
| 3149 | 7.2.3.6 | 6 | 14 | T | Y | Table 12. In 802.11-2007 base draft, the Supported Channels element is present if dot11SpectrumManagementRequired is true. I assume that the Supported Channels should also be included in the Reassociation Request frame if dot11ExtendedChannelSwitchImplemented is true. | Change the editorial note to "Change order 8 entry and insert order 13-14 entries into table 10 as shown below, before VendorSpecific:". Add new row before "Supported Regulatory Classses" as: Order = "8"; Information = "Supported Channels"; Notes = "The Supported Channels element shall be present if dot11SpectrumManagementRequired is true or dot11ExtendedChannelSwitchImplemented is true.". |
| 3150 | 7.2.3.8 | 6 | 49 | T | Y | Table 14, "Supported Regulatory Classes" IE is included in the Probe Request frame. However, "Supported Channels" IE is not. To be consistent, Should "Supported Channels" IE be included in the Probe Request frame as well? |  |
| 3151 | 7.2.3.9 | 7 | 1 | T | Y | Table 15, "Supported Regulatory Classes" IE is included in the Probe Response frame. However, "Supported Channels" IE is not. To be consistent, Should "Supported Channels" IE be included in the Probe Response frame as well? |  |

Comment Resolution: “Regulatory Class signifies "Supported Channels.”

Propose Reject Comments 3146, 3150 and 3151.

Comment Resolution: “Regulatory Class signifies Supported Channels. CID 3176 changes Supported Channels.”

Propose Reject Comments 3147 and 3149.

| 3154 | 7.2.3.1 | 5 | 22 | T | Y | Table 8, order 32, it states: The Extended Channel Switch Announcement information element may be present only if dot11ExtendedChannelSwitchImplemented, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired are true. Why does it requires all of MIB varaibles are true? It seems to that it only requires that dot11ExtendedChannelSwitchImplemented is true. | Change to: The Extended Channel Switch Announcement information element may be present only if dot11ExtendedChannelSwitchImplemented is true. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3155 | 7.2.3.1 | 5 | 28 | T | Y | Table 8, order 33, it states: The Supported Regulatory Classes information element shall be present if dot11ExtendedChannelSwitchImplemented, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired are true. It seems to that it only requires that dot11RegulatoryClassesRequired is true. | Change to: The Supported Regulatory Classes information element shall be present if dot11RegulatoryClassesRequired is true. |

Comment Resolution: “Text being commented on is removed by CID 3176.”

Propose Accept in Principle Comments 3154 and 3155.

| 3111 | 7.2.3.1 | 5 | 24 | T | Y | dot11ExtendedChannelSwitchImplemented should not depend on dot11SpectrumManagementRequired. ECSA is usefull for channel switching at the 2.4GHz band were spectrum management is not required. | Separate DFS from spectrum management. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3112 | 7.2.3.1 | 5 | 24 | T | Y | The beginning of clause 11.9 (DFS procedures) hints that specturm management is used only to solve problems in the 5GHz band. This may imply that specturm management should not be used in other places. | Modify the text in 11.9 to refer to other bands or remove the limitations to the 5GHz band. |

Comment Resolution: “dot11SpectrumManagementRequired text being commented on is removed by CID 3176.”

Propose Accept in Principle Comments 3111 and 3112.

| 3005 | 7.2.3.1 | 5 | 33 | T | Y | Use of "shall" is not recommended for section 7 in "The Supported Regulatory Classes information element shall be present if" | change to "is present" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment Resolution: “Many persistent Beacon frame elements (11, 14, 17, 18 and 21) are Noted as "shall be present", and Supported Regulatory Classes is persistent when operating in a band.”

Propose Reject Comment 3005.

| 3060 | 7.3.2.50 | 12 | 56 | T | Y | Comment carried from LB94. (I have not voted since then.) Do not fix the length of this element. - your response to my previous comment on this matter was "Legacy STAs do not have rules for processing extensions to defined elements." -- this response is a self-fulfilling prophecy, or something. Since this draft will define legacy behavior, that problem of the lack of a defintion for legacy behavior can be specified right now. I.e. it does not have to be a broad behavior for all elements, it can be specified for this element alone. See suggested change. One day, if I can get anyone to do this in any task group and then to keep doing it, we might be able to start adding element information to existing elements, rather than always adding new ones.... | Change the length of this element to "The length of this element is variable, as new fields may be added by subsequent revisions to this standard. STAs that do not recognize the new fields shall process the information in the fields of which they are aware and discard the information contained in the new fields." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment Resolution: “as no additional field is proposed.”

Propose Reject Comment 3060.

| 3152 | 7.4.1.6 | 14 | 34 | T | Y | Figure 117a. The length of the Extended Channel Switch Announcement element field shall be 6 instead of 4. | change 4 to 6. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment Resolution: “the length is four because Element ID and Length are excluded.”

Propose Reject Comment 3152.

Propose Accept Comments 3040, 3045, 3046, 3136 and 3137 without discussion.

Proposed Resolution:

Accept: 3040, 3045, 3046, 3136 and 3137

Accept in Principle based on discussion and editorial instructions in 07/2365r1: 3111, 3112, 3154 and 3155

Reject based on discussion in 07/2365r1: 3005, 3060, 3146, 3147, 3149, 3150, 3151 and 3152

Editorial Instructions:

# Clause 9

CIDs: 3009, 3010, 3036, 3041, 3042, 3056, 3065, 3162 and 3180

Discussion:

In 11y/D4.0:

| 3042 | 9.8.1 | 18 | 17 | T | Y | The statement of information conveyed in the Country Information element of the Beacon frame does not include any Regulatory Class information or Coverage Class information, as they are of variable length, but contain at least one Regulatory class triplet if any are present. | Rewrite paragraph to indicate at least one Regulatory Class triplet shall be present if operating in a band when dot11SupportedRegulatoryClassesRequired is true, and optionally all regulatory information that would be returned in a Probe Response frame may be present on a periodic basis. Also specify that the Supported Regulatory Classes IE shall be present in every Beacon frame when dot11ExtendedChannelSwitchImplemented is true. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3162 | 9.8.1 | 18 | 16 | T | Y | 9.8.1: "Optionally, the Beacon frame may also include, on a periodic basis, the regulatory information that would be returned in a Probe Response frame." 7.2.3.1: "The Supported Regulatory Classes information element shall be present if dot11ExtendedChannelSwitchImplemented is dot11ExtendedChannelSwitchImplemented, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired are true." It is not clear how the "optionally" in 9.8.1 ties in with the "shall" in 7.2.3.1. | Modify one of them so that these two subclauses are consistent. |

Comments 3042 and 3162 note that Supported Regulatory Classes is required in Beacon frames when dot11ExtendedChannelSwitchImplemented is true, yet 9.8.1 says ‘Optionally, the Beacon frame may also include, on a periodic basis, the regulatory information that would be returned in a Probe Response frame.’ Suggested Remedy is to rewrite 9.8.1 sentence beginning ‘Optionally’ so that it is clear that it refers to the full Country Information element, not Supported Regulatory Classes.

Comment Resolution: “Will rewrite part of 9.8.1 to indicate that optionality refers to the Country Information element fields, not the presence of Supported Regulatory Classes, and will change second statement in 9.8.3 accordingly.”

Propose Accept Comments 3042 and 3162.

| 3065 | 9.8.1 | 18 | 12 | T | N | Text is not new for D4.0, but is a problem: And I know that Tgy did not create this, but maybe they could fix it. The sentence begins with "A STA", but it should be restricted to a "A STA that is enabled for operation across regulatory domains" | Modify the sentence as shown in the comment |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept in Principle Comment 3065.

| 3036 | 9.8.1 | 18 | 12 | T | N | "When a STA enters a regulatory domain, before transmitting, it shall passively scan to learn at least one valid channel, i.e., a channel upon which it detects IEEE 802.11 frames." Detecting any IEEE 802.11 frames is not sufficient. It needs to detect a beacon; otherwise it cannot get the information that is needed for the next sentence. OTOH, this isn't text that 802.11y is adding; it's text in the base standard. | "When a STA enters a regulatory domain, before transmitting, it shall passively scan for beacons to learn at least one valid channel, i.e., a channel upon which it detects IEEE 802.11 frames." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment Resolution: “Text is part of 802.11-2007 base, and the sense is unchanged by 802.11y D4.0.”

Propose Accept in Principle Comment 3036.

| 3056 | 9.8.1 | 18 | 13 | T | N | The sentence requires a STA (i.e. "any" 802.11 device) to scan passivly for a beacon before starting any transmission. What happens if such a beacon is not received? Can a STA transmit anyway (after how many passiv scan tries) or never at all? Doesn't this requirement lead to a "bootstrapping" problem, i.e. was is the requirement to allow the 1st STA to transmit such a beacon. (if I don't receive any beacon, I cannot transmit and hence cannot transmit a beacon allowing any other STA to transmit ....) | Please clarify and include information / requirements for the STA that would transmit such a beacon. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment Resolution: “Text is part of 802.11-2007 base, and the sense is unchanged by 802.11y D4.0.”

Propose Accept in Principle Comment 3056.

| 3009 | 9.8.4 | 18 | 65 | T | Y | "When dot11RegulatoryClassesImplemented is true, the Coverage Class field of the Country Information…" Why is this based on dot11RegulatoryClassesImplemented rather than dot11RegulatoryClassesRequired? The coverage class should be used only when required, rather than just if the feature is present. It is also clear from section 17.3.8.6 that coverage class is taken into account only when regulatory classes are required. | change "dot11RegulatoryClassesImplemented" to "dot11RegulatoryClassesRequired" |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3010 | 9.8.4 | 19 | 6 | T | Y | "When dot11RegulatoryClassesImplemented is true, and the Max Transmit Power Level is different from the Transmit Power limit indicated by the Regulatory Class" Why is this based on dot11RegulatoryClassesImplemented rather than dot11RegulatoryClassesRequired? | change "dot11RegulatoryClassesImplemented" to "dot11RegulatoryClassesRequired" |

Propose Accept in Principle Comments 3009 and 3010.

| 3041 | 9.8.3 | 18 | 45 | T | Y | The third rule should not require the Country Information element in Association and Reassociation frames. | Delete "the Country Information and" from the third rule and change "elements" to "element". |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3180 | 9.8.3 | 18 | 45 | T | Y | The rule that mandates including the Country Information and SupportedRegulatoryClasses elements in Association and Re-association frames when dot11RegulatoryClassesRequired is true contradicts the basic IEEE 802.11-2007 spec. This rule makes incompliant the legacy STAs that are compliant with the IEEE 802.11-2007 spec. This change breaks backward compatibility of the specification. | Remove the paragraph: "When dot11RegulatoryClassesRequired is true and a STA is capable of operating as specified in more than one Regulatory Class, the STA shall include the Country Information and SupportedRegulatoryClasses elements in Association frames and Reassociation frames;" |

Comments 3041 and 3180 note that legacy STAs do not include the Country Information element in (re)Association frames.

Comment Resolution: “Will delete ‘Country Information and’ from the third statement of 9.8.3.”

Propose Accept in Principle Comments 3041 and 3180.

Proposed Resolution:

Accept on discussion and editorial instructions in 07/2365r1: 3042 and 3162

Accept in Principle based on discussion and editorial instructions in 07/2365r1: 3009, 3010, 3036, 3041, 3056, 3065 and 3180.

Editorial Instructions:

Tgy Editor: change text in 9.8.1, pg 18 line 16, D4.0 to become as follows

“Optionally, the Beacon frame may also include in the Country Information element, on a periodic basis, the regulatory information that would be returned in a Probe Response frame.”

Tgy Editor: change second and third statement text in 9.8.3, pg 18, D3.0 as follows

### 9.8.3 Operation with Regulatory Classes

- When dot11RegulatoryClassesRequired is true, or where Regulatory Classes domain information is present in a STA, the STA shall indicate Current Regulatory Class information in the Country Information element and Supported Regulatory Classes information element;

- When dot11RegulatoryClassesRequired is true and a STA is capable of operating as specified in more than one Regulatory Class, the STA shall include the Country Information and SupportedRegulatoryClasses elements in Association frames and Reassociation frames;

# Clause 11.9.7 CSA procedures

CIDs: 3011, 3013, 3037, 3038, 3043, 3044, 3050, 3058, 3059, 3070, 3071, 3072, 3079, 3088, 3089, 3090, 3138, 3172, 3173, 3174, 3175, 3176, 3177, 3178, 3181, 3182, 3183, 3184, 3185, 3186

Discussion:

In 11y/D4.0:

| 3181 | 11.9.7.1 | 36 | 61 | T |  | "If dot11ExtendedChannelSwitchImplemented is false, the AP shall send the Channel Switch Announcement and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." According to this text, "dot11ExtendedChannelSwitchImplemented is false" does not mean "dot11ExtendedChannelSwitch is not Implemented" since the AP can still send the Extended Channel Switch element and frame. This is different from the naming convention of other dot11 MIB variables. | Change the name of the MIB variable "dot11ExtendedChannelSwitchImplemented" to be consistent with the naming convention of MIB variables in the dot11 spec. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3184 | 11.9.7.2 | 37 | 31 | T |  | "If dot11ExtendedChannelSwitchImplemented is false, the DFS owner shall send the Channel Switch Announcement and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." According to this text, "dot11ExtendedChannelSwitchImplemented is false" does not mean "dot11ExtendedChannelSwitch is not Implemented" since the AP can still send the Extended Channel Switch element and frame. This is different from the naming convention of other dot11 MIB variables. | Change the name of the MIB variable "dot11ExtendedChannelSwitchImplemented" to be consistent with the naming convention of MIB variables in the dot11 spec. |

Proposed Comment Resolution: “We change the name to dot11ExtendedChannelSwitchEnabled, named like dot11MultiDomainCapabilityEnabled.”

Propose Accept Comments 3181 and 3184.

Tgy Editor: Replace ‘dot11ExtendedChannelSwitchImplemented’ with ‘dot11ExtendedChannelSwitchEnabled’ in all of Draft 4.0 and any draft text accepted from any submissions addressing LB109 comments.

| 3011 | 11.9.7 | 36 | 34 | T | Y | It is unclear when "dot11ExtendedChannelSwitchImplemented" should be set. In section 11.14, page 36, line 18, there is a bland statement which says that "dot11DSERequired and dot11ExtendedChannelSwitchImplemented shall be true when regulatory authorities require DSE". What does this mean? Is dot11ExtendedChannelSwitchImplemented set at a dependant STA, or a enabling STA or a registered STA, or all or what?? | Provide clarification and conditions for setting this MIB attribute. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 3011 asks for clarification on how this MIB variable is set. Proposed Comment Resolution:

“The exact functions of the SME are not specified in this standard,” and in general, Station Management Entity operation is not specified in the 802.11 standard. Annex J.2 specifies setting DSE MIB and ECSA indications for operation in US 3650 MHz band. The existence of dot11ExtendedChannelSwitchImplemented in the MIB indicates that the STA can do ECSA.”

Propose Reject Comment 3011.

| 3043 | 11.9.7 | 36 | 34 | T | Y | The other DFS and TPC procedures are not needed to perform Channel Switch, so remove ", dot11Spectrum Management Required" from the first sentence. | Per comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3079 | 11.9.7 | 36 | 34 | T | Y | I think that this condition is too restrictive, in particular, because TGn desires to use the ECSA without also wanting to have to be forced into support for Spectrum Management. | Decouple Spectrum Management support from ECSA support, and instead, for the purposes of Tgy, where SMgmt support is a desired requirement, couple it to LCIDSERequired, or something else that is very specific to providing a definitive marker of Tgy functionality. |
| 3138 | 11.9.7 | 36 | 34 | T | N | "When dot11ExtendedChannelSwitchImplemented is true, dot11MultiDomainCapabilityEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired shall be true." This mandates DFS for use of E-CSA. In TGn, E-CSA is used in 2.4 GHz or 5.15-5.25GHz where DFS is not required. | Decouple E-CSA from DFS procedures |
| 3173 | 11.9.7 | 36 | 34 | T | Y | "When dot11ExtendedChannelSwitchImplemented is true, dot11MultiDomainCapabilityEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired shall be true" The current solution mandates that STA that wants to support the ECSA shall support the entire TPC and DFS. In case there is no need to follow the regulations for example in 2.4GHz band the channel switching may be still important as in .11n. The definition of ECSA should allow using it separately and as an extension of DFS as well. | Separate the definition of ECSA from the DFS. Define rules of use it separately w/o support of the Spectrum management. Define rules to allow using ECSA together with Spectrum management |
| 3177 | 11.9.7 | 36 | 34 | T | Y | "When dot11ExtendedChannelSwitchImplemented is true, dot11MultiDomainCapabilityEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired shall be true" The current solution mandates that STA that wants to support the ECSA shall support the entire TPC and DFS. In case there is no need to follow the regulations for example in 2.4GHz band the channel switching may be still important as in .11n. The definition of ECSA should allow using it separately and as an extension of DFS as well. | Separate the definition of ECSA from the DFS. Define rules of use it separately w/o support of the Spectrum management. Define rules to allow using ECSA together with Spectrum management |

Comment 3043 notes that The other DFS and TPC procedures are not needed to perform Channel Switch, so remove ", dot11Spectrum Management Required" from the first sentence. Propose Accept in Principle Comments 3043, 3079, 3138 and 3173

Proposed Comment Resolution: “Text will be rewritten to have implicit use of ECSA in US 3650 band, regardless of association, and explicit and advertised in the Extended Capabilities IE, independent of dot11SpectrumManagementRequired. The rules will make no distinction whether dot11SpectrumManagementImplemented is true or false.” Propose Accept in Principle Comment 3177.

There are legacy devices that do not know about channel switch in the 2.4 GHz and 5 GHz bands, and there are legacy devices that do not know about regulatory class in the 5 GHz band, and there are legacy devices that do not know about ECSA. In general, channel switch is best effort, and announcements are unacknowledged, and in particular, none of the legacy devices know how to switch regulatory classes. The AP knows from (re)Association frames whether a station supports ECSA.

| 3012 | 11.9.7.1 | 36 | 58 | T | Y | "If … dot11ExtendedChannelSwitchImplemented is true, then the AP shall send the Extended Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames. If dot11ExtendedChannelSwitchImplemented is false, the AP shall send the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." 1. Whether the MIB is true or not, the text says "send both Extended Channel Switch Announcement". 2. Why should the Extended Channel Switch Announcement be sent when the dot11ExtendedChannelSwitchImplemented is false?? | Please clarify and make necessary changes to text if it is wrong. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 3012 asks for clarification on how the AP decides to send both ECSA and CSA. Proposed Comment Resolution:

“The text says 'or both ', the text does not say "send both". In the 5 GHz band, legacy stations that only know Channel Switch Announcements might be in the same BSS as stations that know Extended CSA, so the AP might send both when changing channels if the AP wants all stations to change. The AP knows from (re)Association frames whether a station supports ECSA.”

Propose Reject Comment 3012.

| 3037 | 11.9.7.1 | 37 | 1 | E | Y | "In the following text, wherever Channel Switch Announcement is referred to both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1), 2) and 3)." This sentence can be read incorrectly. I'd prefer inserting a comma. | "In the following text, wherever Channel Switch Announcement is referred to, both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1) and 2)." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 3037 notes ambiguity in first sentence of third paragraph of 11.9.7.1, and suggests inserting a comma for clarification. Comment Resolution: “Per CID 3178, rewriting ECSA in new clause.” Propose Accept in Principle comment 3037.

| 3089 | 11.9.7.1 | 36 | 58-65 | T | Y | This paragraph is way too confusing. A statement such as "If dot11ExtendedChannelSwitchImplemented, an AP shall use Extended Channel Switch Announcement." would make things much clearer. | As suggested. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 3089 asks for the simplification of the second statement in the inserted second paragraph, and suggests a simplification which does not cover all cases.

Proposed Comment Resolutions: “The statement about switching within the same regulatory class and it's converse are each here, and each is necessary. CID 3058 suggests this presentation for the prior statement as well.” Propose Reject Comment 3089.

| 3088 | 11.9.7.1 | 36 | 51-53 | T | Y | This sentence defines a mandatory behavior: an AP switching to a new channel in a different regulatory class SHALL use the Extended Channel Switch Announcement element and frame. Legacy Aps routinely switch from one regulatory class to another, for instance from 5.15GHz to 5.25GHz. However, they do not have the capability to send out ECSA. Does that mean legacy APs won't be compliant with 802.11 any more? This statement needs to be modified to ensure backward compatibility. | Please insert "and dot11ExtendedChannelSwitchImplemented is ture" before "the the AP shall use the Extended Channel Switch Announcement…" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment Resolution: “Legacy APs that use Channel Switch Announcement are not switching Regulatory Class in a way that violates regulation. CID 3058 proposes the converse be stated in 1), and doing so will clear up the legacy AP situation.”

Propose Accept in Principle Comment 3088.

| 3172 | 11.9.7.1 | 36 | 53 | T | Y | The CSA is not optimized and contains substantial limitation for switching between regulatory classes. Due to support of ECSA is important for legacy stations that are associated in BSS that uses ECSA. For example the .11n compliant BSS may associate non .11n compliant STA as well. Support of Extended Channel Switching may be implemented as SW upgrade in the legacy STA. To make the support of ECSA in the legacy STA visible to other STA the ECSA capability should be signaled. This comment is relevant for behavior of DFS owner in 11.9.7.2 as well | Add the ECSA capability field to the Extended Capabilities information element |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3176 | 11.9.7.1 | 36 | 53 | T | Y | The CSA is not optimized and contains substantial limitation for switching between regulatory classes. Due to support of ECSA is important for legacy stations that are associated in BSS that uses ECSA. For example the .11n compliant BSS may associate non .11n compliant STA as well. Support of Extended Channel Switching may be implemented as SW upgrade in the legacy STA. To make the support of ECSA in the legacy STA visible to other STA the ECSA capability should be signaled. This comment is relevant for behavior of DFS owner in 11.9.7.2 as well | Add the ECSA capability field to the Extended Capabilities information element |

Comment Resolution: “Extended Capabilities text will be based on 07/2326r2 7.3.2.27, pages 9 and 10, modified to meet 11y baseline, which excludes HT.”

Propose Accept in Principle Comments 3172 and 3176.

| 3174 | 11.9.7.1 | 36 | 53 | T | Y | The definition is contradictional: If the CSA cannot be used for switching to new channel in a different regulatory class how to use both. If the CSA can be used for switching to a new channel there is no need to mandate the ECSA. If the CSA cannot be used the ECSA shall be used instead. So the problem is how to know that the ECSA is supported in the cases the CSA cannot be used. This comment is relevant for behavior of DFS owner in 11.9.7.2 as well | Define use of ECSA as function of the ECSA capability. Define this capability as implicit in some kind of networks like TGn or explicit and advertised in ECSA capability field |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3178 | 11.9.7.1 | 36 | 53 | T | Y | The definition is contradictional: If the CSA cannot be used for switching to new channel in a different regulatory class how to use both. If the CSA can be used for switching to a new channel there is no need to mandate the ECSA. If the CSA cannot be used the ECSA shall be used instead. So the problem is how to know that the ECSA is supported in the cases the CSA cannot be used. This comment is relevant for behavior of DFS owner in 11.9.7.2 as well | Define use of ECSA as function of the ECSA capability. Define this capability as implicit in some kind of networks like TGn or explicit and advertised in ECSA capability field |

Comment Resolution: “Text will be rewritten to have implicit use of ECSA in US 3650 band, regardless of association, and explicit and advertised in the Extended Capabilities IE. CSA cannot be used when changing Regulatory Class unless all STAs that act on the CSA meet the requirements signified by the new Regulatory Class. There is no contradiction, the AP knows from (re)Association frames whether STAs can do ECSA, and may attempt to switch channels with both ECSA and CSA if the AP expects the legacy STAs to be able to operate on the new channel and RC.”

Propose Accept in Principle Comments 3174 and 3178.

| 3058 | 11.9.7.1 | 36 | 55 | T | Y | The addition of the text: "when the requirements signified by the new regulatory class are met by all STAs that act on the Channel Switch Announcement" has a problem. Syntactically, is it not completely clear what is being modified by this clause. I first read it as modifying the primary clause of the sentence, and someone managed to straighten me out with that regard. Still, the construction of the sentence is not correct in this regard. Furthermore, for the transmitting STA to identify "all STA that act [on this frame]" might be difficult - how about changing this part to "all associated STA" | I'd recommend splitting the sentence into two sentences with the new clause appearing in one sentence and its converse in the other. Also change the phrase "all STA that act [on this frame]" to "all associated STA" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolutions: “We use ECSA with enabled STAs, independent of their association.” Propose Accept in Principle Comment 3058.

| 3175 | 11.9.7.1 | 36 | 58 | T | Y | The AP knows dot11ExtendedChannelSwitchImplemented value of itself, but no means are defined to know state of the dot11ExtendedChannelSwitchImplemented of other stations. It may happen that no one of the associated stations does support the Extended Channel switching. This comment is relevant for behavior of DFS owner in 11.9.7.2 as well | Define advertising of the Extended Channel switching support. Define ECSA capability field to allow upgrade of the legacy stations to support ECSA. Define the AP behavior to cover associated stations that part of them supports and part does not support ECSA. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3179 | 11.9.7.1 | 36 | 58 | T | Y | The AP knows dot11ExtendedChannelSwitchImplemented value of itself, but no means are defined to know state of the dot11ExtendedChannelSwitchImplemented of other stations. It may happen that no one of the associated stations does support the Extended Channel switching. This comment is relevant for behavior of DFS owner in 11.9.7.2 as well | Define advertising of the Extended Channel switching support. Define ECSA capability field to allow upgrade of the legacy stations to support ECSA. Define the AP behavior to cover associated stations that part of them supports and part does not support ECSA. |

Proposed Comment Resolutions: “The AP knows from (re)Association frames whether STAs support ECSA and Supported Regulatory Classes. We define advertising ECSA via Supported Regulatory Classes and will add an Extended Capabilities indication field.” Propose Accept in Principle Comments 3175 and 3179.

| 3013 | 11.9.7.2 | 37 | 20 | T | Y | What is this line doing here "In the following text:"? | Remove/correct as required. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolutions: “Per CID 3178, rewriting ECSA in new clause.” Propose Accept in Principle Comment 3013.

| 3070 | 11.9.7.2 | 37 | 25 | T | Y | The addition of the text: "when the requirements signified by the new regulatory class are met by all STAs that act on the Channel Switch Announcement" has a problem. The phrase "all STA that act [on this frame]" presents a serious problem in the IBSS case - the DFS owner has a very good chance of NOT knowing that list of STA. I.e. there is no concept of IBSS association, so the DFS owner is likely to not be aware of all of the STA that might or might not act on the channel switch frame. At best, one might word the requirement for the DFS owner with some flexibility, as in "all STA that are known to the DFS owner to be members of the IBSS" instead of "all STAs that act [on this frame]" | Change the wording of the identified phrasing to reflect the fact that the DFS owner probably is not aware of the entire list of STA that might act on the receipt of the channel switch frame. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept in Principle Comment 3070.

| 3044 | 11.9.7.2 | 37 | 36 | T | Y | The DFS owner does not maintain association information, and third numbered statement should be removed. | Delete subparagraph 3) and change the end of the next sentence to "as defined in 1) and 2)." |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3059 | 11.9.7.2 | 37 | 36 | T | N | Text is not new for D4.0, but it is a problem: "The DFS owner shall inform associated STAs that" - within the IBSS case (as we currently are), there is no association concept. | Change the language to reflect the relationship that exists for IBSS, which is: "STAs that the DFS owner is aware are members of the BSS" or something similar |

Propose Accept in Principle Comment 3059.

Proposed Comment Resolution: “CID 3059 resolution chosen.” Propose Accept in Principle Comment 3044.

| 3038 | 11.9.7.2 | 37 | 40 | E | Y | "In the following text, wherever Channel Switch Announcement is referred to both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1), 2) and 3)." This sentence can be read incorrectly. I'd prefer inserting a comma. | "In the following text, wherever Channel Switch Announcement is referred to, both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1), 2) and 3)." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Per CID 3178, rewriting ECSA in new clause.” Propose Accept in Principle Comment 3038.

Propose Accept Comments 3040, 3050, 3071, 3072, 3090, 3182, 3183, 3185 and 3186 without discussion

Proposed Resolution:

Accept: 3040, 3050, 3071, 3072, 3090, 3181, 3182, 3183, 3184, 3185 and 3186

Accept in Principle based on discussion in 07/2365r1: 3013, 3037, 3038, 3043, 3044, 3058, 3059, 3070, 3079, 3088, 3138, 3172, 3173, 3174, 3175, 3176, 3177, 3178 and 3179

Reject: based on discussion in 07/2365r1; 3011, 3012 and 3089

Editorial Instructions:

The following text and editing instructions from 07/2326r2 are modified for the comment resolutions above:

TGy Editor: Remove the editing instruction and following text at end of 11.9.7, TGyD4.0, pg 36, line 34:

When dot11ExtendedChannelSwitchImplemented is true, dot11MultiDomainCapabilityEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired shall be true.

TGy Editor: Change the editing instruction on TGy D4.0 pg 36, line 48 to:

Change second paragraph of 11.9.7.1 as follows:

TGy Editor: Remove the following text in the third paragraph of 11.9.7.1, TGyD4.0, pg 37, line 1:

In the following text, wherever Channel Switch Announcement is referred to both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1) and 2).

TGy Editor: Remove the paragraph in 11.9.7.1, TGyD4.0, pg 36, lines 50-65 that starts with “In the following text: …” and ends with

… or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.”

TGy Editor: Remove the editing instruction and paragraph in 11.9.7.2, TGyD4.0, pg 37, lines 20-42 that starts with “In the following text: …” and ends with

… Switch Announcement and Channel Switch Announcement should be used as defined in 1), 2) and 3).”

TGy Editor: Remove the editing instruction and heading “11.9.7.3 Advertising supported regulatory classes”in TGyD4.0, page 37 line 44,  create new subclause “11.9a.3 Advertising supported regulatory classes” after 11.9a.2.2, and move all that is inside of the former “11.9.7.3 Advertising supported regulatory classes” in TGyD4.0 to the new created “11.9a.3 Advertising supported regulatory classes”

TGy Editor: Add new subclause 11.9a Extended Channel Switching after 11.9 DFS procedures

Insert the following text after end of sublause 11.9:

  - Extended channel switching

Regulations that apply to the 3650 MHz band in the United States of America require RLANs operating in the band to implement a mechanism that allows licensed operators a means to control frequency, channel bandwidth and power of dependent stations. This standard describes such a mechanism, referred to as extended channel switching.

This subclause describes ECS procedures that can be used to satisfy these and similar future regulatory requirements. The procedures may also satisfy comparable needs in other frequency bands and may be

useful for RLANs to change operating channel and Regulatory Class while maintaining BSS operation. Enabling STAs (see 11.14) and APs and DFS owners are each stations that may construct and transmit frames containing Extended Channel Switch Announcement elements when dot11ExtendedChannelSwitchEnabled is true.

.

A STA shall use the extended channel switching procedures defined in this subclause if dot11ExtendedChannelSwitchEnabled is true. When dot11ExtendedChannelSwitchEnabled is true, dot11MultiDomainCapabilityEnabled and dot11RegulatoryClassesRequired shall be true.

A STA shall not transmit the Extended Capabilities information element with the Extended Channel Switching field set to 1 unless dot11ExtendedChannelSwitchEnabled is set to true.

An enabling STA may send frames containing Extended Channel Switch Announcement elements to dependent STAs (see 11.14.4). Dependent STAs, DFS owners and APs shall perform ECS procedures so as to switch at the time indicated by the Channel Switch Count, and shall not take alternative action.

11.9a.1 Association based on supported regulatory classes

A STA with dot11ExtendedChannelSwitchEnabled true shall provide an AP with a list of the Supported Regulatory Classes in which it can operate when associating or reassociating using a Supported Regulatory Classes element in Association Request frames or Reassociation Request frames.

An AP may use the Supported Regulatory Classes lists of associated STAs as an input into an algorithm that is used to select a new channel for the BSS. The specification of such an algorithm is beyond the scope of this standard.

An AP may reject an association or reassociation request from a STA based on information contained in the requesting STA’s Supported Regulatory Classes element.

11.9a.2 Selecting and advertising a new channel

An attempt may be made to move a BSS to a new operating channel using extended channel switching. It is an objective that disruption to the BSS is minimized in this process, although it should be recognized that a channel switch might not successfully move all STAs. A dependent STA that receives frames from the enabling STA which contain Extended Channel Switch Announcement elements shall perform the extended channel switch at the indicated time.

11.9a.2.1 Advertising a new channel in an infrastructure BSS

A dependent AP that receives frames from the enabling STA which contain Extended Channel Switch Announcement elements shall advertise a channel switch with the parameters received in the Extended Channel Switch Announcement elements.

The decision to switch to a new operating channel in an infrastructure BSS shall be made only by the AP. An AP may make use of the information in the Supported Channels, Supported Regulatory Classes elements, and the results of measurements undertaken by the AP and other STAs in the BSS to assist the selection of the new channel.The method to make the decision and to select a new channel are defined in 11.9.7.1.

In the following text:

1) If an AP is switching to a new or the same channel in a different regulatory class then the AP shall use the Extended Channel Switch Announcement element and frame instead of the Channel Switch

Announcement element and frame, or both the Extended Channel Switch Announcement and

the Channel Switch Announcement elements and frames when the requirements signified by

the new regulatory class are met by all STAs that act on the Channel Switch Announcement.

2) If an AP is switching to a new channel within the same regulatory class, and

dot11ExtendedChannelSwitchEnabled is true, then the AP shall send the Extended Channel

Switch Announcement element and frame, or both the Extended Channel Switch

Announcement and the Channel Switch Announcement elements and frames. If

dot11ExtendedChannelSwitchEnabled is false, the AP shall send the Channel Switch

Announcement element and frame, or both the Extended Channel Switch Announcement and

the Channel Switch Announcement elements and frames.

3) If dot11ExtendedChannelSwitchEnabled and dot11LCIDSERequired are true, only Extended Channel Switch Announcement elements shall be transmitted.

4) If both the Extended Channel Switch Announcement and the Channel Switch Announcement elements are transmitted, they shall be sent in separate frames.

An AP shall inform associated STAs that the AP is moving to a new channel and maintain the association by advertising the switch using Extended Channel Switch Announcement elements in Beacon frames, Probe Response frames, and Extended Channel Switch Announcement frames until the intended channel switch time. The AP may force STAs in the BSS to stop transmissions until the channel switch takes place by setting the Extended Channel Switch Mode field to 1 in the Extended Channel Switch Announcement element. If possible, the channel switch should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Extended Channel Switch Announcement element before the switch. The AP may send the Extended Channel Switch Announcement frame in a BSS without performing a backoff, after determining the WM is idle for one PIFS period.

When a STA that is not a dependent STA receives an Extended Channel Switch Announcement element, it may choose not to perform the specified switch, but to take alternative action.

A STA in a BSS that is not the AP shall not transmit the Extended Channel Switch Announcement element.

11.9a.2.2 Advertising a new channel in an IBSS

A dependent DFS owner that receives frames from the enabling STA which contain Extended Channel Switch Announcement elements shall advertise a channel switch with the parameters received in the Extended Channel Switch Announcement elements.

The DFS owner that advertises a channel switch shall follow the rules defined in 11.9.7.2 with the following extensions:

1) If a DFS owner is switching to a new or the same channel in a different regulatory class then the DFS

owner shall use the Extended Channel Switch Announcement element and frame instead of the

Channel Switch Announcement element and frame, or both the Extended Channel Switch

Announcement and the Channel Switch Announcement elements and frames when the requirements

signified by the new regulatory class are met by all STAs that act on the Channel Switch

Announcement.

2) If a DFS owner is switching to a new channel within the same regulatory class, and

dot11ExtendedChannelSwitchEnabled is true, then the DFS owner shall send the

Extended Channel Switch Announcement element and frame, or both the Extended Channel

Switch Announcement and the Channel Switch Announcement elements and frames. If

dot11ExtendedChannelSwitchEnabled is false, the DFS owner shall send the Channel

Switch Announcement element and frame, or both the Extended Channel Switch Announcement

and the Channel Switch Announcement elements and frames.

3) If dot11ExtendedChannelSwitchEnabled and dot11LCIDSERequired are true, only Extended Channel Switch Announcement elements shall be transmitted.

4) If both the Extended Channel Switch Announcement and the Channel Switch Announcement elements are transmitted, they shall be sent in separate frames.

TGy Editor: Add to the clause 3 definitions

3.52a extended channel switching (ECS): To simultaneously change either or both operating channel and Regulatory Class.

TGy Editor: Add to the clause 4 abbreviations

ECS	extended channel switching

7.3.2.27 Extended Capabilities information element

TGy Editor: Add text of subclause 7.3.2.27 as follows:

Change last paragraph of 7.3.2.27 as follows:

The Capabilities field is a bit field indicating the capabilities being advertised by the STA transmitting the

information element. There are no capabilities defined for this field in this revision of the standard.One octet of extended information has been defined. The format of that octet appears in Figure 7-76a (Capabilities field first octet).

Insert the Figure 7-76a in 7.3.2.27 as follows:

|  | B0 | B1 | B2 <this position is a placeholder until assigned by the ANA> | B3 B7 |
| --- | --- | --- | --- | --- |
|  | Reserved | Reserved | Extended channel switching | Reserved |
| Bits: | 1 | 1 | 1 | 5 |

Figure 7-76a—Capabilities field first octet

Insert the following at the end of 7.3.2.27:

The extended channel switching field indicates support for the Extended Channel Switch Announcement element and management frame type and its use. The extended channel switching field shall be set to 1 to indicate support for the communication of channel switching information through the transmission and reception of the Extended Channel Switch Announcement element and management frame. The extended channel switching field shall be set to 0 to indicate a lack of support for the communication of channel switching information through the transmission and reception of the Extended Channel Switch Announcement element and management frame.

If a STA does not support any of capabilities defined in the Extended Capabilities information element, then the STA is not required to transmit the Extended Capabilities Information element.

End of changes in 7.3.2.27

TGy Editor: Change the Table 8-Beacon frame body in the TGyD4.0, on page 5 as follows:

| Order | Information | Notes |
| --- | --- | --- |
| 31 | DSE registered location | The DSE registered location information element shall be present if dot11LCIDSERequired is true. |
| 32 | Extended Channel Switch Announcement | The Extended Channel Switch Announcement information element may be present only if dot11ExtendedChannelSwitchEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired are is true. |
| 33 | Supported Regulatory Classes | The Supported Regulatory Classes information element shall be present if dot11ExtendedChannelSwitchEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired are is true. |

TGy Editor: In the subclause 7.2.3.4 of Std 802.11-2007 on the page 82 change in the Table 7-10 as follows:

| 7 | Supported Channels | The Supported Channels element shall be present if dot11SpectrumManagementRequired is true and dot11ExtendedChannelSwitchEnabled is false |
| --- | --- | --- |

TGy Editor: In the subclause 7.2.3.6 of Std 802.11-2007 on the page 83 change in the Table 7-12 as follows:

| 8 | Supported Channels | The Supported Channels element shall be present if dot11SpectrumManagementRequired is true and dot11ExtendedChannelSwitchEnabled is false |
| --- | --- | --- |

TGy Editor: Change last sentence in third paragraph of 7.3.2.50 as shown:

An AP in a BSS or a STA in an IBSS sets the Channel Switch Mode field to either 0 or 1 on transmission as specified in 11.9.7.1 and 11.9.7.211.9a.2.1 and 11.9a.2.2.

TGy Editor: Change the first sentence of 7.4.1.6 as shown:

The Extended Channel Switch Announcement frame is transmitted by an enabling STA, an AP in an infrastructure BSS or a STA in an IBSS to advertise a channel switch.

Annexs J.2

CIDs: 3077, 3078 and 3097

| 3078 | J.2 | 70 | 22 | T | Y | "and shall not use Channel Switch Announcement" -- ouch. I thought that I saw specific allowances for this frame to be used back in 11.9.7.1 and 11.9.7.2 | So which is it? Is a STA allowed to use this or not? The draft is self-contradicting. I vote for 11.9.7.1 and 11.9.7.2, hence recommend removing the text that was added here in J.2. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Requirements of Annex J.2 apply for operation in US 3650 MHz band. TGy voted several times to only allow ECSA in US 3650, so text in 11.9.7 will be changed to disallow CSA when dot11LCIDESRequired is true.”

Proposed Accept in Principle Comment 3078.

| 3077 | J.2 | 70 | 10 | T | Y | Regarding: "Further requirements when dot11LCIDSERequired is true:" -- up to this point in J.2 there has been no mention of "dot11LCIDSERequired" - so it is unclear which, if any, of any previous requirements relates to dot11LCIDSERequired. | Please introduce LCIDSERequired as a condition for any requirements listed, and/or relate this MIB attribute to the discussion at hand, possibly eliminating the word ""further", but at least, providing the relationship between LCIDSERequired values and the current discussion - e.g. DSE is invoked when LCIDSERequired is set to true at a STA. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3097 | Annex J.2 | 70 | 12 | T | Y | Co-existence can be improved by the use of narrower channels. Period. Why is an outdoor environment different from indoors? | Replace "and outdoor operation are" with "is" |

Propose Accept Comments 3077 and 3097 without discussion.

Proposed Resolution:

Accept: 3077 and 3097

Accept in Principle based on discussion in 07/2365r1: 3078