IEEE P802.11
Wireless LANs

| LB97 CID83 DelayedBA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

Addressing the TGn LB97 CIDs from the MAC adhoc subgroup Block Ack Delayed, including CIDs 83, 84, 835, 843.

| 83 | Banerjee, Kaberi | 3.n17 | 2.20 |  | Conveys no further intelligence than the terms that are being defined | Some information to distinguish these from the non-HT Delayed BA may be required | Accept – editor shall make the changes found in 11-07-2407r0 under the heading CID 83. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 84 | Banerjee, Kaberi | 3.n18 | 2.24 |  | Conveys no further intelligence than the terms that are being defined | Some information to distinguish these from the non-HT Immediate BA may be required. | Accept – editor shall make the changes found in 11-07-2407r0 under the heading CID 84. |
| 835 | Lefkowitz, Martin | 9.10.8.2 | 125.21 |  | "HT-delayed BlockAck is an optional feature." Between the next sentence and the PICS this sentence really adds nothing to the specifucation | Remove the sentence. | Accept – remove the sentence “HT-delayed BlockAck is an optional feature." That is found in D2.07 at page 131 line 27. |
| 843 | Lemberger, Uriel | 9.10.8 | 125.04 |  | This is an old remain that suited the proposals before the down selection process. It has no value in the current content and it is used only after special request not after the packet. | Remove the HT-Delayed BlockAck from the spec | Reject – there are circumstances when a STA may be able to receive frames but is unable to provide an immediate acknowledgement, but is able to provide a delayed acknowledgement. Under those circumstances, this feature is quite useful. |

CID 83

TGn Editor: Change item 3.n17 of subclause “3 Definitions” of TGn Draft D2.07 on about page 3 line 23 as shown:

3.n17 high throughput delayed (HT-delayed) block ack: An optional Delayed Block Ack mechanism optimized for use by HT STAs that includes the mandatory use of the compressed BlockAck frame and the optional use of the NoAck ack policy setting within both the BlockAckReq and BlockAck frames. This Block Ack scheme is negotiated between two HT STAs that both support HT Delayed Block Ack.

CID 84

TGn Editor: Change item 3.n18 of subclause “3 Definitions” of TGn Draft D2.07 on about page 3 line 27 as shown:

3.n18 high throughput immediate (HT-immediate) block ack: An Immediate Block Ack mechanism optimized for use by HT STAs that includes the mandatory use of the compressed BlockAck frame, an optional implied block ack request by the originator and the option for partial state operation at the recipientt. This Block Ack scheme is negotiated between two HT STAs.

References: