IEEE P802.11
Wireless LANs

| Resolution of miscellaneous RFI comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd, Lake Mary, FL 32771 | +1-407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

As there are now 56 open RFI comments in 07-0023r46, we are very close to comment resolution completion.  This document resolves simple issues such as terminology (element vs message).

Resolution of comment 1437

| Comment | Resolution Notes |
| --- | --- |

| When the text states the "IE will be discarded," does that mean the entire frame containing the IE is discarded, or just the IE? | There are too many confusing "discarded" in the text. We recommend "the IE will not be processed" instead of discarded. |
| --- | --- |

Counter with the following resolution

| Page (D1.06) | Section | Original text | Replacement |
| --- | --- | --- | --- |
| 149 | 11A.6.2.3.1 Acceptance criteria | The Portal Announcement shall be discarded if | The Portal Announcement element shall not be accepted (and shall not be processed as described in 11A.6.2.3.2) if |
| 149 | 11A.6.2.3.2 Effect of receipt | The following applies only to Portal Announcements that were not discarded during the check described in 11A.6.2.3.1 (Acceptance criteria) above. | The following applies only to a Portal Announcement element that was accepted according to the acceptance criteria in 11A.6.2.3.1 (Acceptance criteria). |
| 158 | 11A.8.4.2 Destination Sequence Number (DSN) | then the information in the HWMP message is stale and shall be discarded. | then the information in the HWMP message is stale and shall not be processed. |
| 158 | 11A.8.4.2 Destination Sequence Number (DSN) | If they are the same, the outcome (information element discarded or not) | If they are the same, the outcome (information element processed or not) |
| 167 | 11A.8.5.3.1 Acceptance criteria | The PREQ is discarded if | The PREQ element shall not be accepted (and shall not be processed as described in 11A.8.5.3.2) if |
| 167 | 11A.8.5.3.2 Effect of receipt | The following applies only to a PREQ that has not been discarded: | The following applies only to a PREQ element that was accepted according to the acceptance criteria in 11A.8.5.3.1: |
| 172 | 11A.8.6.3.1 Acceptance criteria | The PREP is discarded if | The PREP element shall not be accepted (and shall not be processed as described in 11A.8.6.3.2) if |
| 172 | 11A.8.6.3.2 Effect of receipt | The following applies only to a PREP that has not been discarded | The following applies only to a PREP alement that was accepted according to the acceptance criteria in 11A.8.6.3.1 |
| 174 | 11A.8.7.3.1 Acceptance criteria | The PERR is not discarded if | The PERR shall be accepted (and shall be processed as described in 11A.8.7.3.2) if |
| 174 | 11A.8.7.3.2 Effect of receipt | The following applies only to PERR that has not been discarded. | The following applies only to a PERR element that was accepted according to the acceptance criteria in 11A.8.7.3.1 |
| 176 | 11A.8.8.3.1 Acceptance criteria | The RANN is discarded if | The RANN element shall not be accepted (and shall not be processed as described in 11A.8.8.3.2) if |
| 176 | 11A.8.8.3.2 Effect of receipt | The following applies only to RANN that has not been discarded. | The following applies only to a RANN element that was accepted according to the acceptance criteria in 11A.8.8.3.1 |

Resolution of comment 4140

| Comment | Proposed Change by Commenter | Resolution Notes |
| --- | --- | --- |
| RREQ and RANN are frames, not messages in the .11 context | Change from "message" to "frame" throughout. | "message" may be misleading, but "frame" is more misleading. Consider replacing with "IE" |

Indeed, the only “frame” is the action frame.  Counter with the following resolution

| Page (D1.06) | Section | Original text | Replacement |
| --- | --- | --- | --- |
| 151 | 11A.6.5.1.3 PU processing | A PU is a unicast message sent from an MP to a destination MP | A PU element is individually addressed from an MP to a destination MP |
| 153 | 11A.8.1.1 General | In order to propagate the metric information between MPs, a metric field is used in the PREQ, PREP and RANN messages. | In order to propagate the metric information between MPs, a metric field is used in the PREQ, PREP and RANN elements. |
| 154 | 11A.8.1.3 Proactive tree building mode | The first method uses a proactive Path Request (PREQ) message | The first method uses a proactive Path Request (PREQ) element |
| 154 | 11A.8.1.3 Proactive tree building mode | An MP configured as root MP would send either proactive PREQ or RANN messages periodically. | An MP configured as root MP would send either proactive PREQ or RANN elememts periodically. |
| 154 | 11A.8.1.3.1 Proactive PREQ mechanism | The PREQ tree building process begins with a proactive Path Request message | The PREQ tree building process begins with a proactive Path Request element |
| 155 | 11A.8.1.3.2 Proactive RANN mechanism | The root MP periodically propagates a RANN message into the network. | The root MP periodically propagates a RANN element through the network. |
| 157 | 11A.8.4 General rules for processing HWMP information elements | components of the HWMP messages and information elements | components of the HWMP information elements |
| 157 | 11A.8.4 General rules for processing HWMP information elements | It is assumed that the receiving MP only receives HWMP messages | It is assumed that the receiving MP only accepts HWMP elements |
| 157 | 11A.8.4 General rules for processing HWMP information elements | all HWMP messages received are presumed | all HWMP elements received are presumed |
| 158 | 11A.8.4.2 Destination Sequence Number (DSN) | the receiving MP compares its value of the stored sequence number with that in the incoming HWMP message | the receiving MP compares its value of the stored sequence number with that in the incoming HWMP element |
| 158 | 11A.8.4.2 Destination Sequence Number (DSN) | then the information in the HWMP message is stale | then the information in the HWMP element is stale |
| 160 | 11A.8.5.2 Conditions for generating and sending a PREQ | The MP has not sent more than (HWMP_PREQ_RATELIMIT – 1) path request messages | The MP has not sent more than (HWMP_PREQ_RATELIMIT – 1) path request elements |
| 173 | 11A.8.7.2 Conditions for generating and sending a PERR | The MP did not send more than HWMP_PERR_RATELIMIT – 1 PERR messages | The MP did not send more than HWMP_PERR_RATELIMIT – 1 PERR elements |
| 173 | 11A.8.7.2 Conditions for generating and sending a PERR | The MP has not sent or forwarded more than HWMP_PERR_RATELIMIT – 1 PERR messages during the last second. | The MP has not sent or forwarded more than HWMP_PERR_RATELIMIT – 1 PERR elements during the last second. |
| 195 | 11A.9.13 Interworking through Mesh Portal (MPP) | If an MP is not able to determine an inter-mesh path to the destination MAC address, the MP shall assume that the destination is outside the mesh and shall forwards the messages | If an MP is not able to determine an inter-mesh path to the destination MAC address, the MP shall assume that the destination is outside the mesh and shall forward the frames |
| Misc | Misc | Reference to 11A.8.4 provide the wrong title | Reference to 11A.8.4 provide the wrong title |

Resolution of comments 34 221 223 558 2207 2213 2214 3212 5687 5691 and 5692

For clarity, all variables will start with dot11MeshHWMP

| Page (D1.06) | Section | Original text | Replacement |
| --- | --- | --- | --- |
| 160 | 11A.8.5.2 Conditions for generating and sending a PREQ | DO flag, RF flag, as required | The DO flag is set to dot11MeshHWMPDestinationOnly and the RF flag is set to dot11MeshHWMPReplayAndForward |
| 160 | 11A.8.4.6 Repeated attempts at path discovery | HWMP_MAX_PREQ_RETRIES | dot11MeshHWMPmaxPREQretries |
| 160 | 11A.8.5.2 Conditions for generating and sending a PREQ | HWMP_MAX_PREQ_RETRIES | dot11MeshHWMPmaxPREQretries |

Replace the Recommended values section (Annex T2)

dot11MeshHWMPmaxPREQretries = 3

References:

doc.: IEEE 802.11-07/0023r46

IEEE P802.11s™/D1.06