IEEE P802.11
Wireless LANs

| Draft Text Changes for ‘Express Forwarding’ in a Mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mathilde Benveniste | Avaya Labs-Research | 233 Mt Airy Road Basking Ridge, NJ 07920, USA | 973-761-6105 | benveniste@ieee.org |

Abstract

Changes to TGs Draft 1.07 are presented in this document to enable inclusion of the optional feature ‘express forwarding’ for multi-hop transmissions.  A description of ‘express forwarding’ appears in Doc. IEEE 802.11-07-2452r2.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Inserted text appears in red underlined characters; deleted text appears in blue characters with a strike through line.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Modify subclause 7.1.3.5a.2 as follows:

- Mesh Flags field

The Mesh Flags field, shown in s5, is 8 bits in length and the flags therein are used to control mesh-specific header processing, e.g., for mesh address extension.

| B0 | B1 7 | B2 7 |
| --- | --- | --- |
| Address Extension (AE) | Time-sensitive QoS (TSQ) | Reserved |
| Bits: 1 | 1 | 7 6 |

The “Address Extension (AE)” flag is used to indicate the existence of — and thereby the use of — the mesh address extension field for mesh address extension based on 6-address format: If AE = 1, the mesh address extension field follows the Mesh Sequence Number field and provides two additional address fields for mesh address extension (see 11A.3.4 for the usage of address fields in 6-address format); if AE is 0, the mesh address extension field is not present. The “AE” flag can be set to 1 only when both “To DS” and “From DS” fields are set to 1.

The “Time-Sensitive QoS (TSQ)” flag is used to indicate that the Duration field value has been extended for Express Forwarding purposes.  If TSQ = 1 for a transmitted frame, an MP that supports Express Forwarding shall process the Duration field value of the frame when updating its NAV according to the rules given in subclause 9.22.2.  If TSQ = 0 for a transmitted frame, an MP shall process the Duration field value of the frame to update its NAV setting according to the rules of the coordination function.  An MP that does not support Express Forwarding shall always process the Duration field value of the frame to update its NAV setting according to the rules of the coordination function, regardless of the TSQ flag setting.  Only frames of type Extended with subtypes Mesh Data or Mesh Management.  The TSQ flag shall be set when the conditions in subclause 9.22.1 are met.

- The reserved field is set to zero.

- Inset text below after subclause 9.21

9.22   Express Forwarding (Optional)

Express Forwarding (EF) is an optional access method that allows MPs that support EF to access the channel with lower contention than would otherwise be possible when forwarding selected frames that traverse multi-hop paths.  A frame of type Extended with subtypes Mesh Data or Mesh Management may be express-forwarded if its TSQ flag in the Mesh_Flags field of the Mesh Header field is set.  Such a frame is referred to as a ‘TSQ frame’. An MP that supports EF shall extend the Duration field value when transmitting a TSQ frame to an MP other than the final destination of the frame. The intended receiver MP shall  access the channel after an AIFS idle period.  . If the intended receiver MP does not support EF, it shall forward the received TSQ frame according to the rules of the coordination function.

    - Behavior of an MP transmitting a TSQ frame

When an MP that supports EF has a frame of type Extended with subtypes Mesh Data or Mesh Management to transmit to an MP with Address 1 that does not match Address 3 of the frame, for which the following conditions are true:

- the frame’s user priority is dot11MeshEFUP or greater, and

- the value in the TTL subfield of the Mesh Header field is less than or equal to (dot11MeshTTL – dot11MeshEFNH ),

it shall set the frame’s TSQ flag to 1, and shall extend the Duration field value by the value of Dot11MeshDTC.   A frame that meets the conditions above is referred to as a “TSQ frame”.

- 9.22.2	Behavior of an MP receiving a TSQ frame

When an MP that does not support EF receives a TSQ frame, it shall process the Duration field value of the received frame when updating its NAV according the coordination function rules.

When an MP that supports EF receives a TSQ frame, it shall process the Duration field value of the received frame when updating its NAV.

If an MP that supports EF is the intended receiver of the transmitted TSQ frame (i.e., Address 1 matches its own MAC address), it shall send an ACK frame for a frame requiring acknowledgment, with Duration field value obtained from the Duration field of the received frame, minus the time, in microseconds, required to transmit the ACK frame and its SIFS interval.  The MP may transmit a frame once the channel has been idle for an AIFS interval.

If an MP that supports EF has transmitted a TSQ frame that requires acknowledgement/CTS does not get a PHY-RXSTART.indication during the ACKTimeout/CTSTimeout interval, the MP shall retransmit the TSQ frame by observing the rules in subclause 9.22.1.  The MP shall invoke the backoff procedure for the retransmission of the TSQ frame except for the first retransmission, when the MP may transmit without backoff after the channel has been idle for an AIFS time interval following expiration of the ACKTimeout/CTSTimeout timer.

- Inset text at the end of Appendix D

- ASN.1 encoding of MAC and PHY MIB

dot11MeshDTC

SYNTAX INTEGER (O..255)

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

”This object specifies the time value in microseconds added to the duration field value when transmitting a TSQ frame.  "

== { dot11MeshConfig Entry 11};

dot11MeshEFUP

SYNTAX INTEGER (O..7)

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

”This object specifies lowest user priority of frames that may use 	express forwarding"

== { dot11MeshConfig Entry 13};

dot11MeshEFNH

SYNTAX INTEGER (O..20)

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

”The object specifies the minimum number of Mesh Hops that a frame 	must transmit before engaging in express forwarding."

== { dot11MeshConfig Entry 14};

- References:

- Draft P802.11s_D1.07.pdf, Sept 2007

- M. Benveniste, “Express Forwardingin Multi-hop Wireless Network”, Doc. IEEE 802.11-07-0415, May 2007

- M. Benveniste, “Express Forwardingin for Single-Channel Wireless Mesh”, Doc. IEEE 802.11-07-2452, Nov 2007