IEEE P802.11
Wireless LANs

| SA Teardown Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-1-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Epstein | Meru Networks | 894 Ross Dr, Sunnyvale, CA 94089 | +1 408 215 5300 | jepstein@merunetworks.com |
| Kapil Sood | Intel | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 264 3759 | Kapil.Sood@intel.com |

Abstract

This document contains the normative text for introducing additional SA teardown protection with optional Lockout protection, into the RSN framework, as mentioned in the presentation 802.11-07/2441r2

(Contains protection by association reject with comeback time)

## Introduction

This proposal creates a Ping request/response method that is used to help determine whether an incoming Association Request is from an already associated STA that has lost state, or is rather from an attacker hoping to tear down the security associations.  Presentation 802.11-07/2441 describes this in more detail.

These changes are based off of 802.11w Draft 3.0 and its dependant documents.  Changes are shown in underlined green, unless they are pure additions or are otherwise mentioned.  Instructions to the TGw editor are highlighted in yellow.

## Editor’s Instructions

TGw Editor: Add the following row, and replace <ANA> with the value of the assigned status code in 7.3.1.9 below

7.2.3.5 Association Response frame format

Insert the following new row into table 7-11:

Table 7-11 – Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Association Comeback Time | Association Comeback Time is present whenever the Association is rejected with status code <ANA> |

TGw Editor: Add the following row, and replace <ANA> with the value of the assigned status code in 7.3.1.9 below

7.2.3.7 Reassociation Response frame format

Insert the following new row into table 7-13

Table 7-13 – Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Association Comeback Time | Association Comeback Time is present whenever the Association is rejected with status code <ANA> |

TGw Editor: Add the following

7.3.1.9 Status Code

Insert the following new row into table 7-23:

Table 7-23 – Status Codes

| Code | Meaning |
| --- | --- |
| <ANA> | Association request rejected temporarily; Try again later |

TGw Editor: Add the following

7.3.1.11 Action Field

Insert the following new row into table 24:

Table 24 – Category values

| Code | Meaning | See subclause |
| --- | --- | --- |
| <ANA> | Ping | 7.4.8 |

TGw Editor: Add the following row of Table 7-26

7.3.2 Information Elements

Insert the following new row into table 24:

Table 7-26 – Information Elements

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Association Comeback Time (see 7.3.2.55) | <ANA> | 6 |

7.3.2.25.3 RSN capabilities

Change as follows:

Insert in Fig 7-74: B8 – Lockout Protection Enabled

Insert after Bit 7 description:

- Bit 8: Lockout Protection Enabled.  A STA sets this bit to 1 when dot11RSNALockoutProtectionEnabled is set to TRUE to advertise that it supports mechanisms for evading Lockout protection.

TGw Editor: Add the following

7.3.2.55 Association Comeback Time information element

The Association Comeback Time information element may be present in non-successful Association Response and Reassociation Response frames, to indicate to the requester the earliest time at which the AP is willing to possibly accept the association. The Comeback Duration field contains the delay—in TU—from the receipt of the frame containing the information element, after which the STA should try to associate.  The Association Comeback Time information element format is shown in Figure 7-3w.

|  | Element ID | Length | Comeback Duration (TU) |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 4 |

Figure 7-3w—Ping Response frame details

TGw Editor: Add the following, and assign table and figure numbers appropriately

Insert the following before 7.5

7.4.8 Ping action details

Two Action frame formats are defined for the Ping action. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table 7-1w.

Table 7-1w – Ping action fields

| Action field value | Description |
| --- | --- |
| 0 | Ping Request |
| 1 | Ping Response |

7.4.8.1 Ping Request frame

The Ping Request frame is used to request a Ping Response from the receiving STA.  The format of the frame is shown in Figure 7-12w.

|  | Category | Action | Transaction Identifier | Vendor Specific Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 16 | variable |

Figure 7-2w—Ping Request frame details

The Category field is set to the value indicating the Ping category, as specified inTable 24 in 7.3.1.11.

The Action field is set to the value indicating Ping Request frame, as specified in Table 7-1w in 7.4.8.

The Transaction Identifier field is set to a 128-bit random value by the STA sending the Ping request to identify the request/report transaction.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

7.4.8.2 Ping Response frame

The Ping Response frame is used to answer a Ping request from another STA.  The format of the frame is shown in Figure 7-3w.

|  | Category | Action | Transaction Identifier | Vendor Specific Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 16 | variable |

Figure 7-3w—Ping Response frame details

The Category field is set to the value indicating the Ping category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Ping Response frame, as specified in Table 7-1w in 7.4.8.

The Transaction Identifier field is set to the same value as the Transaction Identifier field in the corresponding Ping Request frame.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

8.4.3 RSNA policy selection in an ESS

TGw Editor:  Change as follows:

An RSNA-enabled AP shall use Table 8-1a and the Robust Management Frame Protection Capable and Robust Management Frame Protection Required advertised in the RSN IEs to determine if it may associate with a non-AP STA. An RSNA enabled non-AP STA shall use Table 8-1a and the values of the Robust Management Frame Protection Capable and Robust Management Frame Protection Required bits advertised in the RSN IEs to determine if it may associate with an AP.  If management frame protection is negotiated, then Lockout Protection shall be enabled when the non-AP STA and the AP set the Lockout Protection Enabled bit in the advertised RSN IE.  If frame protection is not negotiated, then Lockout Protection Enabled bit shall be set to zero and ignored on reception.

TGw Editor: Change 8.4.10 with the following, with changes to the draft shown in green

8.4.10 RSNA security association termination

Change the first paragraph as follows:

When a non-AP STA SME receives a successful MLME Association or Reassociation confirm primitive that is not part of a Fast BSS Transition or receives or invokes an MLME Disassociation or Deauthentication primitive, it will delete some security associations. Similarly, when an AP SME

- receives an MLME Association or Reassociation indication primitive, from a non-AP STA that has not negotiated management frame protection, or

- receives an MLME Association or Reassociation indication primitive, from a non-AP STA that has negotiated management frame protection, that a) has resulted in an MLME Association or Reassociation response that is successful, and b) is not part of a Fast BSS Transition, or

- if Lockout Protection is negotiated, completes a failed Ping procedure, as specified in 11.3.2.2 or 11.3.2.4, or

- receives or invokes an MLME Disassociation or Deauthentication primitive,

it will delete some security associations. In the case of an ESS the non-AP STA’s SME shall delete the PTKSA, GTKSA, IGTKSA, SMKSA, and any STKSA, and the AP’s SME shall delete the PTKSA, and invoke an STSL application teardown procedure for any of its STKSAs. An example of an STSL application teardown procedure is described in 11.7.3. In the case of an IBSS, the STA’s SME shall delete the PTKSA and the receive GTKSA and IGTKSA. Once the security associations have been deleted, the SME then invokes MLME-DELETEKEYS.request primitive to delete all temporal keys associated with the deleted security associations.

TGw Editor: Add the following before Clause 11

Insert at the end of sub clause 10.3 a new sub clause as follows:

10.3.39 Ping support

10.3.39.1 MLME-PING.request

10.3.39.1.1 Function

This primitive requests that a Ping Request frame be sent to the peer STA to which the STA is associated.

10.3.39.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-PING.request	(
PeerSTAAddress,

TransactionIdentifier,

VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity for the Ping . |
| TransactionIdentifier | 16 octets | N/A | The Transaction Identifier to identify the Ping Request and Response transaction. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the Ping Request frame. |

10.3.39.1.3 When Generated

This primitive is generated by the SME to request that a Ping Request frame be sent to the peer STA with which the STA is associated.

10.3.39.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Ping Request action management frame. The STA then attempts to transmit this to the peer STA with which it is associated.

10.3.39.2 MLME-PING.confirm

10.3.39.2.1 Function

This primitive reports the result of a Ping procedure.

10.3.39.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- PING.confirm	(
PeerSTAAddress
TransactionIdentifier,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity that the Ping Reponse was received from. |
| TransactionIdentifier | 16 octets | N/A | The TransactionIdentifier to identify the Ping Request and Response transaction. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters from the Ping Response frame. |

10.3.39.2.3 When Generated

This primitive is generated by the MLME as a result of the receipt of a Ping Response action management frame.

10.3.39.2.4 Effect of Receipt

On receipt of this primitive, the SME may use the response as a sign of liveness of the peer STA.

10.3.39.3 MLME-PING.indication

10.3.39.3.1 Function

This primitive indicates that a Ping Request frame was received from a STA.

10.3.39.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-PING.indication	(
PeerSTAAddress,
TransactionIdentifier,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity that the Ping Request was received from. |
| TransactionIdentifier | 16 octets | N/A | The TransactionIdentifier to identify the Ping Request and Response transaction. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters from the Ping Request frame. |

10.3.39.3.3 When Generated

This primitive is generated by the MLME when a valid Ping Request action management frame is received.

10.3.39.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.

10.3.39.4 MLME-PING.response

10.3.39.4.1 Function

This primitive is generated in response to an MLME-PING.indication requesting a Ping Response frame be sent to a  STA.

10.3.39.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-PING.response	(
PeerSTAAddress,
TransactionIdentifier,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity for the Ping . |
| TransactionIdentifier | 16 octets | N/A | The TransactionIdentifier to identify the Ping Request and Response transaction. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the Ping Response frame. |

10.3.39.4.3 When Generated

This primitive is generated by the SME, in response to an MLME-PING.indication, requesting a Ping Response be sent to a STA.

10.3.39.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Ping Response action management frame. The STA then attempts to transmit this to the STA indicated by the PeerSTAAddress parameter.

TGw Editor: Insert the following

11.3.1.1 Authentication—Originating STA

Change the last paragraph to as follows

The STA’s SME shallmay delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) before invoking MLME-AUTHENTICATE.request primitive

11.3.1.2 Authentication—Destination STA

Change the second to the last paragraph to as follows.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLMEAUTHENTICATE.indication primitive if and only if dot11RSNAProtectedManagementFramesEnabled  is false.

11.3.2.2 AP Association procedures

Insert the following after item b and reletter the remainder of the list.

c) If the STA is associated and has a valid security association, and has negotiated management  frame protection and Lockout Protection, the AP shall reject the Association Request with status code “Association request rejected temporarily; Try again later”, shall not modify any association state for the non-AP STA, and shall include in the Association Response an Association Comeback Time information element, specifying a comeback time no smaller than dot11AssociationPingResponseTimeOut multiplied by dot11AssociationMaximumPingAttempts.  Following this, if the AP is not already engaging in Pings as a part of a dot11AssociationPingResponseTimeOut, the AP shall issue one MLME-PING.requests to the STA, and wait for up to dot11AssociationPingResponseTimeOut for an MLME-PING.confirm.  If an MLME-PING.confirm with a matching transaction identifier does not come within the timeout, the Ping Request process shall be repeated for up to dot11AssociationMaximumPingAttempts.  If no MLME-PING.confirm whose transaction identifier matches that of any request comes by end of the Ping Response timeout, the AP shall terminate the security association, as specified in 8.4.10.

Change the last paragraph to as follows:

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLME-ASSOCIATE.indication primitive that results in a successful MLME-ASSOCIATE.response primitive.

11.3.2.4 AP Reassociation procedures

Insert the following after item b and reletter the remainder of the list.

c) If the STA is associated and has a valid security association, has negotiated management frame protection and lockout protection,, and the reassociation is not a part of a Fast BSS Transition, the AP shall reject the Reassociation Request with status code “Association request rejected temporarily; Try again later”, shall not modify any association state for the non-AP STA, and shall include in the Reassociation Response an Association Comeback Time information element, specifying a comeback time no smaller than dot11AssociationPingResponseTimeOut multiplied by dot11AssociationMaximumPingAttempts.  Following this, if the AP is not already engaging in Pings as a part of a dot11AssociationPingResponseTimeOut, the AP shall issue one MLME-PING.requests to the STA, and wait for up to dot11AssociationPingResponseTimeOut for an MLME-PING.confirm.  If an MLME-PING.confirm with a matching transaction identifier does not come within the timeout, the Ping Request process shall be repeated for up to dot11AssociationMaximumPingAttempts.  If no MLME-PING.confirm whose transaction identifier matches that of any request comes by end of the Ping Response timeout, the AP shall terminate the security associations, as specified in 8.4.10.

Change the last paragraph to as follows:

Except when the association is part of a Fast BSS Transition, the STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLME-REASSOCIATE.indication primitive that results in a successful MLME-REASSOCIATE.response primitive.

TGw Editor: Insert the following

Insert at the end of Clause 11 a new sub clause as follows:

11.20 Ping procedures

To send a Ping Request to a peer STA, the STA shall issue an MLME-PING.request.  A Ping Request shall not be sent to a peer STA unless the peer STA is associated to the sending STA and the peer STA supports Ping services.  A STA that supports Ping services and receives an MLME-PING.indication shall respond with an MLME-PING.response when all of the following are true: the receiving STA is currently associated to the sending STA, and no pending MLME-ASSOCIATE.request or MLME-REASSOCIATE.request primitives are outstanding for the STA that receives the Ping indication.

If dot11RSNAProtectedManagementFramesEnabled is true, then the STA shall support Ping services.

Annex A

TGw Editor: Insert the following at Annex A, at the end of the table already present in the draft

Insert the following row to end of table in A.4.4.1:

| PC 35 | Ping Services | 7.4.8, 11.20 | PC34:M | Yes No |
| --- | --- | --- | --- | --- |

Annex D

TGw Editor: Insert the following at Annex D, page 31, line 14, after adding a comma to the end of the preceding line

dot11AssociationPingResponseTimeOut		Unsigned32,

dot11AssociationMaximumPingAttempts		INTEGER,

dot11RSNALockoutProtectionEnabled	TruthValue

TGw Editor: Insert the following at Annex D, page 31, line 55

dot11AssociationPingResponseTimeOut				OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of time units (TUs)

that an AP waits, from the scheduling of the first Ping Request, as a part of security-association verification, to drop the security associations for the STA if a successful Ping Response is not received."

::= { dot11StationConfigEntry 66 }

DEFVAL (50)

dot11AssociationMaximumPingAttempts 				OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of unresponded-to Ping Requests that may be sent by an AP, for a given Association Request or Reassociation Request, before the association can proceed."

::= { dot11StationConfigEntry 67 }

DEFVAL (2)

dot11RSNALockoutProtectionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This variable indicates whether or not this STA supports Lockout protection mechanisms in RSNA networks.”

DEFAULT { FALSE }

::= { dot11StationConfigEntry 68}

TGw Editor: Thank you!