IEEE P802.11
Wireless LANs

| Venue Type Assisted Network Selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503-264-8014 | necati.canpolat@intel.com |

Abstract

This document is a normative text proposal which adds venue type and venue name to the work of network-type assisted network selection (cf. 11-07-2219r2).  This document describes proposed changes to resolve CIDs: 1892, 1903.

This document is based on IEEE P802.11u-d1.0 and IEEE 802.11-2007.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Management frames

      - Beacon frame format

Add the following to the contents of Table 8 as shown below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | Interworking Capability | Interworking Capability is present if dot11InterworkingImplemented is true. |
| 26 | Generic Advertisement Service Capability | Generic Advertisement Service Capability is present if dot11AdvertisingServiceImplemented is true. |
| 27 | Generic Advertising Service Traffic Indication Map | Present in Beacon if any of the supported Advertisement Protocols are configured for multicast delivery. |
| 28 | HESSID | HESSID is present if dot11InterworkingImplemented is present |
| 29 | Network Information | Present if dot11InterworkingImplemented is true. |

EDITORIAL NOTE— These order values need to be allocated before sponsor ballot (ABSB) by ANA

and are only tentative values at this time..

      - Probe Response frame format

Add the following to the contents of Table 15 as shown:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | Interworking Capability | Interworking Capability is present if dot11InterworkingImplemented is true. |
| 24 | Generic Advertisement Service Capability | Generic Advertisement Service Capability is be present if dot11AdvertisingServiceImplemented is true. |
| 25 | HESSID | HESSID is present if dot11InterworkingImplemented is present |
| 26 | Network Information | Present if dot11InterworkingImplemented is true. |
| 26 27 | SSID Container IE | Optional (shall be present if “Use SSIDC IE in Probes” bit in the Interworking Capability IE is set to 1 and the SSID in the corresponding Probe Request is not the default SSID.) |

EDITORIAL NOTE— These order values need to be allocated before sponsor ballot (ABSB) by ANA

and are only tentative values at this time..

- 7.3.2	Information elements

Insert the following to the contents of Table 26 as shown below:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Interworking Capability | <ABSB> (see note below) |
| GAS Capability | <ABSB> (see note below) |
| Advertisement Protocol | <ABSB> (see note below) |
| GAS Request | <ABSB> (see note below) |
| GAS Response | <ABSB> (see note below) |
| GAS Traffic Indication Map | <ABSB> (see note below) |
| GAS Comeback Delay | <ABSB> (see note below) |
| HESSID | <ABSB> (see note below) |
| Network Information | <ABSB> (see note below) |
| QoS Map Set | <ABSB> (see note below) |
| Expedited bandwidth request | <ABSB> (see note below) |
| SSID Container element | <ABSB> (see note below) |
| Default Emergency Services NAI | <ABSB> (see note below) |
| Reserved | <ABSB> (see note below) |

EDITORIAL NOTE—Assignment of values for these information elements needs to be approved by IEEE 802.11 ANA. Until that time, these values are marked as Allocate By Sponsor Ballot (ABSB). Final values will be requested from IEEE 802.11 ANA once this amendment reaches the 75% approval threshold in Sponsor Ballot.

- 7.3.2.43	HESSID Information element

The Homogenous Extended Service Set ID (HESSID) Information element contains description of HESSID. The format of the HESSID element is shown in Figure u1.

|  | Element ID | Length | HESSID | Network Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure u1—HESSID Information element format

The Element ID field is equal to the HESSID value in Table 26.

The value of the Length field is 6.

The HESSID field specifies the definition of HESSID.

In the infrastructure mode, the HESSID definition includes the BSSID value of one of the group of APs. The HESSID and the SSID together to provide is a unique value that is advertised in beacons and probe responses so that the non-AP STA is aware of continued applicability of previously discovered interworking and advertising services when moving from one AP to another within the scope of the HESSID.

The Network Type field is defined in Figure u13a.  This field is used to advertise the type of network for every SSID included in the HESSID set.

|  | b0 | b2 | b3 | b4 | b5 | b6 | b7 | b3 | b7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Type Code |  | NASR | Intranet/ Internet | Advertisement Policy |  | Reserved | Reserved |  |
| Bits: | 3 |  | 1 | 1 | 2 |  | 1 | 5 |  |

Figure u13a—HESSID Network Type Field

Bits 0-2 are the Network Type Code bits.  These bits are set with the value from Table u1a.

Table u1a—Network Type Code bit assignments

| Network Type Code Bits (b0-b1-b2) | Meaning | Description |
| --- | --- | --- |
| 0x0 | Unspecified | Network type has not been configured. |
| 000 0x1 | Private network | Network requires user accounts for access |
| 001 0x2 | Private network with guest access | Network requires user accounts for access, but guest accounts area available |
| 010 0x3 | Chargeable | Access to the network requires payment. Further information on types of charges may be available through other methods (802.21, UAM, etc). |
| 011 0x4 | Free | No charges apply for network use. Bits 4-6 are also used in this case. |
| 100 0x5 | Emergency network | The network supports emergency services. Further information is available from a GAS native query (see clause 7.3.2.53) |
| 101-111 0x6 to 0x7 | Reserved | Not used. |

Bits 3 through 7 are reserved and are set to 0 by the AP and ignored by the non-AP STA upon reception.

Bit 3 is the Next Authentication Step Required (NASR) bit.  This bit shall be set to one to indicate that the network requires a further authentication step, as defined in clause 7.3.2.54.

Bit 4 is the Internet/Intranet bit.  This bit is only used when the Network Type Code in Table u1a is set to 011.  It is set to 1 to indicate that the network provides Internet access, and 0 to indicate the network provides access only to local networks.  If the Network Type Code is not 011, these bits shall be set to 0.

Bits 5 and 6 are the Advertisement Policy field, which has values set according to Table u1b.  These bits are only used when the Network Type Code in Table u1a is set to 011  If the Network Type Code is not 011, these bits shall be set to 0.

Table u1b—Advertisement Policy bit assignments

| Network Type Code Bits (b0-b1-b2) | Meaning | Description |
| --- | --- | --- |
| 00 | No Advertisements | The network does not require the end user to view commercial advertisements |
| 01 | Advertisements present | The network requires end users to view advertisements to support the service |
| 10 | Mixed advertisement policy | The network may require end users to view advertisements, but only for certain services. |
| 11 | Reserved | Not used. |

Bit 7 is reserved and shall be set to zero.

Editor: insert a new element after HESSID element and renumber clauses.

- 7.3.2.44	Network Information element

The Network Information element contains descriptive data for the BSS.  The format of the Network Information element is shown in Figure u1a.

|  | Element ID | Length | Network Metadata | Venue Group | Venue Type |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure u2a—Network Information element format

The Element ID field is equal to the Network Information value in Table 26.

The value of the Length field is 3.

The Network Metadata field provides information about the BSS.  The Network Information field is defined in Figure u13a.  This field is used to advertise metadata for the default and non-default SSIDs included in the BSSID.  If the Network Information element is also included in an SSIDC element which is part of the same BSSID, it supersedes this element.

The Venue Group and Venue Type are both 1-octet values selected from Table 1c and Table 1d respectively.  These fields are used to advertise venue information for the default and non-default SSIDs included in the BSSID.  If the Network Information element is also included in an SSIDC element which is part of the same BSSID, it supersedes this element.

|  | b0 | b1 | b2 | b3 | b4 | b5 | b7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NASR | Internet/ Intranet |  | Advertisement Policy |  | Reserved |  |
| Bits: | 1 | 2 |  | 2 |  | 3 |  |

Figure u13a—Network Metadata Field

Bit 0 is the Next Authentication Step Required (NASR) bit.  This bit shall be set to one to indicate that the network requires a further authentication step, as defined in clause 7.3.2.54.

Bits 1 and 2 are the Internet/Intranet bits, whose values are set according to Table u1ab.  These bits are only used when the Network Type is free.  If the Network Type is not free, these bits are set to 0.

Bits 3 and 4 are the Advertisement Policy field, whose values are set according to Table u1b.  These bits are only used when the Network Type Code in the HESSID element indicate a “free” network type; otherwise these bits are set to 0.

Bits 5 - 7 are reserved and are set to zero by the AP and ignored upon reception.

Table u1ab—Internet/Intranet bit assignments

| Internet/Intranet Bits | Description |
| --- | --- |
| 0x0 | Unspecified |
| 0x1 | Internet access provided |
| 0x2 | Intranet access only (i.e., access is only provided to local networks) |
| 0x3 | Reserved |

Table u1b—Advertisement Policy bit assignments

| Advertisement Policy Bits | Meaning | Description |
| --- | --- | --- |
| 0x0 | Unspecified | This value has not been configured on the AP |
| 00 0x1 | No Advertisements | The network does not require the end user to view commercial advertisements |
| 01 0x2 | Advertisements present | The network requires end users to view advertisements to support the service |
| 10 0x3 | Mixed advertisement policy | The network may require end users to view advertisements, but only for certain services. |
| 11 | Reserved | Not used. |

Table u1c—Venue Group assignments

| Venue Group Code | Venue Group Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Assembly |
| 2 | Business |
| 3 | Educational |
| 4 | Factory and Industrial |
| 5 | Institutional |
| 6 | Mercantile |
| 7 | Residential |
| 8 | Storage |
| 9 | Utility and Miscellaneous |
| 10 | Vehicular |
| 11 | Outdoor |
| 12 – 255 | Reserved |

Table u1d—Venue Type assignments

| Venue Group | Venue Type Code | Venue Description |  |
| --- | --- | --- | --- |
| 0 | 0 | Unspecified |  |
| 0 | 1 - 255 | Reserved |  |
| 1 | 0 | Unspecified Assembly |  |
| 1 | 1 | Arena |  |
| 1 | 2 | Stadium |  |
| 1 | 3 | Passenger Terminal (e.g., airport, port, bus station, ferry terminal, train station) |  |
| 1 | 4 | Amphitheater |  |
| 1 | 5 | Amusement Park |  |
| 1 | 6 | Church |  |
| 1 | 7 | Convention Center |  |
| 1 | 8 | Library |  |
| 1 | 9 | Museum |  |
| 1 | 10 | Restaurant |  |
| 1 | 11 | Theater |  |
| 1 | 12 | Zoo or Aquarium |  |
| 1 | 13 - 255 | Reserved |  |
| 2 | 0 | Unspecified Business |  |
| 2 | 1 | Doctor or Dentist office |  |
| 2 | 2 | Bank |  |
| 2 | 3 | Fire Station |  |
| 2 | 4 | Police Station |  |
| 2 | 5 | Post Office |  |
| 2 | 6 | Professional Office |  |
| 2 | 7 | Research and Development Facility |  |
| 2 | 8 | Attorney Office |  |
| 2 | 9 – 255 | Reserved |  |
| 3 | 0 | Unspecified Educational |  |
| 3 | 1 | School, Primary |  |
| 3 | 2 | School, Secondary |  |
| 3 | 3 | University or College |  |
| 3 | 4-255 | Reserved |  |
| 4 | 0 | Unspecified Factory and Industrial |  |
| 4 | 1 | Factory |  |
| 4 | 2 – 255 | Reserved |  |
| 5 | 0 | Unspecified Institutional |  |
| 5 | 1 | Hospital |  |
| 5 | 2 | Long-Term Care Facility (e.g., Nursing home, Hospice, etc.) |  |
| 5 | 3 | Alcohol and Drug Re-habilitation Center |  |
| 5 | 4 | Group Home |  |
| 5 | 5 | Prison or Jail |  |
| 5 | 6 – 255 | Reserved |  |
| 6 | 0 | Unspecified Mercantile |  |
| 6 | 1 | Retail Store |  |
| 6 | 2 | Grocery Market |  |
| 6 | 3 | Automotive Service Station |  |
| 6 | 4 – 255 | Reserved |  |
| 7 | 0 | Unspecified Residential |  |
| 7 | 1 | Hotel or Motel |  |
| 7 | 2 | Dormitory |  |
| 7 | 3 | Boarding House |  |
| 7 | 4 – 255 | Reserved |  |
| 8 | 0 – 255 | Reserved |  |
| 9 | 0 – 255 | Reserved |  |
| 10 | 0 | Unspecified Vehicular |  |
| 10 | 1 | Automobile or Truck |  |
| 10 | 2 | Airplane |  |
| 10 | 3 | Bus |  |
| 10 | 4 | Ferry |  |
| 10 | 5 | Ship or Boat |  |
| 10 | 6 | Train |  |
| 10 | 7 – 255 | Reserved |  |
| 11 | 0 | Unspecified Outdoor |  |
| 11 | 1 | Muni-mesh Network |  |
| 11 | 2 | City Park |  |
| 11 | 3 – 255 | Reserved |  |

      - SSID Container Information element

The SSID Container Information element is used to contain an Index field, an SSID IE and optional RSN IE for use by Interworking-capable STAs in accordance with the scanning procedures defined in clause 11.1.3.  The format of the SSID Container information element is shown in Figure u3.

|  | Element ID | Length | Index | SSID IE | RSN IE (optional) | Network Information IE (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | 5 |

Figure u3—SSID Container (SSIDC) element description

The Element ID field is equal to the SSID Container element value in Table 26.

The Length field is shall be set to 1 plus the sum of the number of octets in the SSID and RSN IEs if the Network Information IE is not present.  The Length field is set to 6 plus the sum of the number of octets in the SSID and RSN IEs if the Network Information IE is present.

The index field is used to identify within the TIM element which bit in the Partial Virtual bitmap indicates broadcast or multicast frames are buffered at the AP for this SSID.  The values which the Index field can take on are provided in clause 7.3.2.6.

The SSID IE is defined in clause 7.3.2.1.

The RSN IE is defined in clause 7.3.2.25.

The Network Information  IE is defined in 7.3.2.44.  If this optional field is present, then the information provided is specific to this SSID and overrides the information provided by the Network Information IE in the Beacon frame.

Insert a new Native Query Type in Table u5 in clause 7.3.2.48:

Table u5—Native Query Info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 0 | 7.3.2.51 |
| mSSID List | 1 | 7.3.2.52 |
| Emergency Networks List | 2 | 7.3.2.53 |
| Network Authentication Type | 3 | 7.3.2.54 |
| Venue Name | 4 | 7.3.2.55 |
| Reserved | 5 – 255 |  |

Insert the following new subclause, renumbering figures and tables as appropriate:

      - Venue Name Information element

The Venue Name Information Element provides the venue name associated with the BSS.  The format of the Venue Name IE is shown in Figure u27d.  The Venue Name element may be used in cases where the SSID does not provide sufficient description of the venue’s name.

|  | Info ID (4) | Length | Status Code | Venue Name |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable |

Figure u27d—Native Info Venue Name element format

The Info ID field is equal to 4, which corresponds to the Venue Name element per Table u2.

The Length field is the length of the Venue Name element and is equal to 2 plus the size of the venue name field.

The Status Code is a 2-octet field with values drawn from Table 23.

The Venue Name field is a UTF-8 formatted field containing the venue’s name.  The maximum length of this field is 256 octets.  UTF-8 format shall be per RFC-3629.

  - MLME SAP Interface

    - Scan

      - MLME-SCAN.request

      - MLME-SCAN.confirm

        - Function

        - Semantics of the service primitive

Add the following to the BSSDescription elements:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | The values from the Interworking Capability information element if such an element was present in the Probe Response or Beacon. |
| GASCapability | As defined in GAS Capability element | As defined in frame format | The values from the GAS Capability information element if such an element was present in the Probe Response or Beacon. |
| HESSID | As defined in HESSID element | As defined in frame format | The values from the HESSID information element if such an element was present in the Probe Response or Beacon. |
| Network Information | As defined in Network Information element | As defined in frame format | The values from the Network Information element if such an element was present in the Probe Response of Beacon. |

- MLME

- 11.10 WLAN Interworking with External Networks Procedures

Insert the following new subclauses, renumbering figures and tables as appropriate:

- 11.10.4	Homogeneous Extended Service Set

A Homogenous ESS is a collection of BSSs wherein the set of reachable networks, defined by their SSIDs, and services provided by those networks available at any one BSS are available at all BSSs.  The HESSID is a 6-octet MAC address which identifies the homogeneous ESS.  All BSSs identified by an HESSID shall also be in the same mobility domain, if one is defined.  The HESSID value shall be identical to one of the BSSIDs in the homogeneous ESS.  Thus it is a globally unique identifier, which in conjunction with the SSID, is used to provide network identification for an SSPN.

- 11.10.5	Network Information

The Network Information element provides metadata, venue name and venue type information per 7.3.2.44 in Beacon and Probe Response frames to a non-AP STA to assist the non-AP STA with network selection.  When provided in Beacon and Probe Response frames, this information applies to all SSIDs configured on BSSID transmitted in those frames.

The Network Information element is optionally included in the SSIDC element (cf. 7.3.2.47).  When included in an SSIDC element, the information applies to only the SSID in the SSIDC and overrides the information provided in Beacon and Probe Response frames for that BSSID.