IEEE P802.11
Wireless LANs

| Proposed Text Change for Traffic Generation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Moo Ryong Jeong | DoCoMo USA Labs. | 3240 Hillview Ave, Palo Alto, CA 94304-1201, USA | +1-650-496-4761 | jeong@docomolabs-usa.com |

Abstract

This document contains a proposed text change to Draft P802.11v_D1.02 to resolve the LB 108 comments on Traffic Generation and AC Station Count

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 7.3.2.27 Extended Capabilities information element

##### Insert new rows into table 35a as follows:

- Table 35a—Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| … | … | … |
| <ANA>+12 | Traffic Generation | The Traffic Generation bit set to 1 indicates the STA supports Traffic Generation as described in 11.20.10. The Traffic Generation bit set to 0 indicates that the STA does not support this capability. |
| <ANA>+13 | AC Station Count | The AC Station Count bit set to 1 indicates the STA supports AC Station Count as described in 11.20.13. The AC Station Count bit set to 0 indicates that the STA does not support this capability. |
| <ANA>+14-n*8 | Reserved | - All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

7.3.2.74 AC Station Count element

##### Replace the last paragraph with the following:

The Station Count field is 2 octets long and contains an unsigned integer, encoded according to 7.1.1, that specifies the number of associated QoS STAs that have indicated the traffic generation of the AC. The AC Station Count procedures is described in 11.20.13.

- 7.4.11 Wireless Network Management action details

##### Insert a new row into table v48 as follows:

Table v48—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| … | … |
| 23 | Traffic Generation Update |
| 24-255 | Reserved |

##### Insert a new clause as follows:

- 7.4.11.24 Traffic Generation Update frame format

The Traffic Generation Update frame is sent by a non-AP STA to the AP to update Traffic Generation information. The format of the frame is shown in Figure xxx1.

|  | Category | Action | Traffic Generation Flags |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

- Figure xxx1—Traffic Generation Update frame format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Traffic Generation Update frame, as specified in Table v48 in .

The Traffic Generation Flags field indicates whether there is expectation of generating traffic for each of the eight user priorities (UP). The format of the Traffic Generation Flags field is defined in 7.3.2.73.

- 11.20.10 Traffic Generation Procedure

##### Replace the descrption of this clause with the following:

If the MIB attribute dot11MgmtOptionTrafficGenerationImplemented is true, a non-AP QoS STA shall be able to construct the Traffic Generation Flags field as specified in 7.3.2.73 . Traffic Generation Flags are constructed at the SME of the non-AP QoS STA, from application requirements supplied via the SME. The Traffic Generation Flags are constructed from two application requirements: whether traffic generation is required for applications and whether a specific UP is required for the generated traffic. If such requirements are known to an application, the application supplies them to the SME on its initial configuration or on any changes of configuration. Note that the requirements may be known before the traffic is actually generated. For example, a phone application may be configured to generate UP 6 traffic upon the initiation of a voice session.

If there is insufficient information available to the SME, the corresponding Flag bit shall be set to 0. When provided with the traffic generation aspects, the SME updates the Traffic Generation Flags accordingly.

If the MIB attribute dot11MgmtOptionTrafficGenerationEnabled is true, a non-AP QoS STA shall include the Traffic Generation element in an Association Request frame or in a Reassociation Request frame when it is sending such a frame to associate or reassociate with an AP. If there is any change in Traffic Generation Flags while associated with an AP, the non-AP STA shall send to the AP a Traffic Generation Update frame including the updated Traffic Generation element, as specified in 7.4.11.24.

If the MIB attribute dot11MgmtOptionTrafficGenerationImplemented is true, a QoS AP shall be able to understand the Traffic Generation Flags in an Association Request frame,  an Reassocation Request frame, and a Traffic Generation Update frame.

If the MIB attribute dot11MgmtOptionTrafficGenerationEnabled is true, a QoS AP shall include the Traffic Generation Flags of the associated non-AP QoS STAs as one of the factors that may affect the outcome of the BSS load management. It is implement specific how the Traffic Generation Flags affect the outcome of the BSS load management. However, an informative description is given in the follwoing. An AP may determine the station count for a user priority (UP) based on the number of associated STAs that indicate the traffic generation of the UP. Similary, an AP may determine the station count for an access category (AC) as specified in 11.20.13. Furthermore, based on the station counts for UPs and other information that is available for an application, an AP may determine the station count for the application. Those station counts may be interpreted as the number of STAs that are expected to access the channel to transmit MSDUs of the corresponding UP, AC, or the application. Accordingly, if needed, the AP and network may use those station count information for the purpose of BSS load management, for example, using the procedures of 11.20.7 and 11.20.13.

##### Insert a new clause as follows:

- 11.20.13 AC Station Count

If the MIB attribute dot11MgmtOptionACStationCountImplemented is true, a QoS AP shall be able to construct the AC Station Count field value of the AC Station Count sub-element (7.3.2.37) and the AC Station Count element (7.3.2.74). The AP shall construct the AC Station Count field value based on the UP-to-AC mappings as defined in Table 65 and the Traffic Generation Flags of the non-AP STAs that are currently associated with it. If the MIB attribute dot11MgmtOptionACStationCountEnabled is true, a QoS AP shall include the AC Station Count element and the AC Station Count sub-element respectively in a probe response frame and in a Neighbor Report element within a BSS Transition Management Request frame.

If the MIB attribute dot11MgmtOptionACStationCountImplemented is true, a non-AP QoS STAs shall be able to understand the AC Station Count field value in the AC Station Count sub-element and the AC Station Count element. If the MIB attribute dot11MgmtOptionACStationCountEnabled is true, a non-AP QoS STA shall include the AC Station Count field values as one of the factors when determining association, reassociation, and the BSS transition.

# Annex A

- A.4 PICS proforma-IEEE Std. 802.11, 1999 Edition

- A.4.18 Wireless Network Management extensions

Insert this entry to the end of following table:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME16 | Traffic Generation | 7.2.3.4, 7.2.3.6, 7.3.2.74, 11.20.10 | (CFv AND CF12):O | Yes, No, N/A |
| RME17 | AC Station Count | 7.2.3.9, 7.3.2.37, 7.3.2.75, 11.20.13 | (CFv AND CF12):O | Yes, No, N/A |

# Annex D

Insert the following entries at the end of the Dot11WirelessMgmtOptionsEntry sequence list:

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

...

dot11MgmtOptionTrafficGenerationImplemented	TruthValue,

dot11MgmtOptionTrafficGenerationEnabled	TruthValue,

dot11MgmtOptionACStationCountImplemented	TruthValue,

dot11MgmtOptionACStationCountEnabled		TruthValue}

On page 187, as part of Dot11WirelessMgmtOptionsEntry, add the followingMIB objects:

dot11MgmtOptionTrafficGenerationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Traffic Generation when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 14 }

dot11MgmtOptionTrafficGenerationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that Traffic Generation is enabled. Traffic Generation is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 15 }

dot11MgmtOptionACStationCountImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting AC Station Count when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 16 }

dot11MgmtOptionACStationCountEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that AC Station Count is enabled. AC Station Count is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 17 }