IEEE P802.11
Wireless LANs

| TGp LB 110 Comment Resolution Landscape Form |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Vinuth Rai | VSC-2 | 4009 Miranda Avenue, Palo Alto, CA, 94304 | 650-251-0530 | Vinuth@us.toyota-itc.com |

Abstract

This document proposes accepting comments # 77,78,79,80,81,82,83 and passes a motion to accept the proposed change to the document

|  | LB110 Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| 77 | Stephens, Adrian | 5.2.1 | 2 | 43 | T | T | "WBSS uses ad hoc mode specifications where not explicitly defined otherwise" "Ad-hoc" is the informal term. IBSS (independent BSS) is the formal term and should be preferred. Also it's not clear what "uses ... specifications" means. | Be clear about what you mean and avoid use of informal terminology. | Accepted | This paragraph will be removed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 78 | Myles, Andrew | 5.2.1 | 2 | 47 | TR | TR | The text states, "WBSS uses ad hoc mode specifications where not explicitly defined otherwise" This weakly implies that WBSS has some commonality with IBSS, more so that a BSS. However, a review of the draft reveals that WBSS has little commonality with an IBSS because the special features of an IBSS (eg synchronisation and power management) are not used in a WBSS | Remove the sentence, and define a WBSS is terms of what it does use from the 802.11 standard, which appears to be EDCA and a limited number of management frames | Accepted | This paragraph will be removed |
| 79 | Durand, Roger | 5.2.1 | 2 | 47 | TR | TR | This paragraph implies ad-hoc operation for a WAVE environment. This simply as I see it can not and will not work. The environments are not compatible and a lot of detail is missing to make it compatible. Ad-hoc as it presently exists has nothing to do with WAVE so either don't say anything or do the work to make it compatible | remove the paragraph. | Accepted | This paragraph will be removed |
| 80 | Rai, Vinuth | 5.2.1 | 2 | 47 | TR | TR | the statement “WBSS uses ad hoc specifications where not explicitly defined otherwise” is, nor in 11.1.1. There is an explicit qualification in Section 11.1 regarding synchronization problematic. How explicitly and in how close a proximity to an ad hoc specification will the WBSS alternate specification be? Consider, for example, this statement in Section 11.1.1.2: “The TSF in an IBSS shall be implemented via a distributed algorithm that shall be performed by all of the members of the BSS.” That statement does not apply to a WBSS. There is no explicit alternate specification for a WBSS in Section 11.1.1.2, i.e. “All STAs within a single BSS shall be synchronized to a common clock using the mechanisms defined herein, unless the BSS is a WAVE BSS.” Does that constitute an “explicitly defined otherwise statement” with respect to 11.1.1.2? A similar issue arises with IBSS specifications in 11.1.2.2. Other examples of potential issues: • Section 5.2.3.2 includes: “In an IBSS, each STA implements both the Authenticator PAE and Supplicant PAE roles and both EAP Authenticator and EAP peer roles. … In certain applications, the AS may be integrated into the same physical device as the AP, or into a STA in an IBSS.” (These statements are also repeated in similar words in Section 5.8) These de not apply to the WBSS. • Section 5.4.3.1 includes the following: “IEEE 802.11 authentication is an SS. This service may be used by all STAs to establish their identity to STAs with which they communicate, in both ESS and IBSS networks. If a mutually acceptable level of authentication has not been established between two STAs, an association shall not be established.” This does not apply to WBSS. • Section 5.4.3.2 contains the following: “In an RSN IBSS, Open System authentication is optional, but a STA is required to recognize Deauthentication frames.” • Section 5.4.4.2 includes the following: “The DFS service provides for the following: … Selecting and advertising a new channel to assist the migration of a BSS or IBSS after radar is detected.” This does not apply to WBSS. • Section 5.6 includes the following: “In an IBSS, only Class 1 and Class 2 frames are allowed because there is no DS in an IBSS.” Does this also apply to WBSS? I believe a WBSS can connect to a DS. • Section 5.8 includes the following: “The IEEE 802.1X access control mechanisms apply to the association between a STA and an AP and to the relationship between the IBSS STA and STA peer.” This does not apply to WBSS. • Section 5.8.3 starts with: “5.8.3 IBSS functional model description This subclause summarizes the system setup and operation of an RSNA in an IBSS.” This section does not apply to WBSS. • Section 7.2.3.1 (Table 7-8) states that: “The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS.” This does not apply to WBSS. A similar comment applies to the Probe Response frame defined in Section 7.2.3.9 • Later in Table 7-8 it says: “IBSS DFS element shall be present if dot11SpectrumManagementRequired is true in an IBSS.” I believe that if that attribute is true in a WBSS the IBSS DFS element shall not be present, and it would be an error condition. A similar comment applies to the Probe Response frame defined in Section 7.2.3.9 • Section 7.3.1.4 includes the following: “STAs within an IBSS set the ESS subfield to 0 and the IBSS subfield to 1 in transmitted Beacon or Probe Response management frames.” This does not apply to WBSS. • Section 7.3.2.7 defines the IBSS Parameter Set element, including the following: “The IBSS Parameter Set element contains the set of parameters necessary to support an IBSS.” This does not apply to WBSS, and it is unclear if the optional WSIE constitutes an explicit definition to the contrary. A similar comment applies to the IBSS DFS element defined in Section 7.3.2.24. • Section 7.3.2.13 includes the following: “In an IBSS, the setting of the Use_Protection bit is left to the STA.” It is unclear if this applies to WBSS. • Section 7.3.2.20 includes the following: “The Channel Switch Announcement element is used by an AP in a BSS or a STA in an IBSS to advertise when it is changing to a new channel and the channel number of the new channel. … A STA in an IBSS may treat a Channel Switch Mode field set to 1 as advisory.” It is unclear if this applies to WBSS. A similar comment applies to the Channel Switch Announcement frame defined in Section 7.4.1.5. • Section 7.5, Table 7-58 lists the frame subtypes that are transmitted and received by a MAC in an IBSS. It is unclear whether all of these apply to WBSS (e.g. authentication, deauthentication), or whether exceptions are sufficiently “explicitly defined otherwise.” • Section 8 includes many specifications regarding security in an IBSS. It is unclear whether any of these apply to a WBSS. This is only a sample of instances where specifications related to IBSS do not apply (or it is unclear if they apply) to WBSS, and for which an explicit definition to the contrary is lacking. | Delete line 47: "WBSS uses ad hoc mode specifications where not explicitly defined otherwise" | Accepted | This paragraph will be removed |
| 81 | Jiang, Daniel | 5.2.1 | 2 | 47 | TR | TR | the statement “WBSS uses ad hoc specifications where not explicitly defined otherwise” is dangerous and opens this amendment to many inconsistences with the main 802.11 standard. Problems include but not necessarily are limited to the following: • Section 5.2.3.2 includes: “In an IBSS, each STA implements both the Authenticator PAE and Supplicant PAE roles and both EAP Authenticator and EAP peer roles. … In certain applications, the AS may be integrated into the same physical device as the AP, or into a STA in an IBSS.” (These statements are also repeated in similar words in Section 5.8) These de not apply to the WBSS. • Section 5.4.3.1 includes the following: “IEEE 802.11 authentication is an SS. This service may be used by all STAs to establish their identity to STAs with which they communicate, in both ESS and IBSS networks. If a mutually acceptable level of authentication has not been established between two STAs, an association shall not be established.” This does not apply to WBSS. • Section 5.4.3.2 contains the following: “In an RSN IBSS, Open System authentication is optional, but a STA is required to recognize Deauthentication frames.” • Section 5.4.4.2 includes the following: “The DFS service provides for the following: … Selecting and advertising a new channel to assist the migration of a BSS or IBSS after radar is detected.” This does not apply to WBSS. • Section 5.6 includes the following: “In an IBSS, only Class 1 and Class 2 frames are allowed because there is no DS in an IBSS.” Does this also apply to WBSS? I believe a WBSS can connect to a DS. • Section 5.8 includes the following: “The IEEE 802.1X access control mechanisms apply to the association between a STA and an AP and to the relationship between the IBSS STA and STA peer.” This does not apply to WBSS. • Section 5.8.3 starts with: “5.8.3 IBSS functional model description This subclause summarizes the system setup and operation of an RSNA in an IBSS.” This section does not apply to WBSS. • Section 7.2.3.1 (Table 7-8) states that: “The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS.” This does not apply to WBSS. A similar comment applies to the Probe Response frame defined in Section 7.2.3.9 • Later in Table 7-8 it says: “IBSS DFS element shall be present if dot11SpectrumManagementRequired is true in an IBSS.” I believe that if that attribute is true in a WBSS the IBSS DFS element shall not be present, and it would be an error condition. A similar comment applies to the Probe Response frame defined in Section 7.2.3.9 • Section 7.3.1.4 includes the following: “STAs within an IBSS set the ESS subfield to 0 and the IBSS subfield to 1 in transmitted Beacon or Probe Response management frames.” This does not apply to WBSS. • Section 7.3.2.7 defines the IBSS Parameter Set element, including the following: “The IBSS Parameter Set element contains the set of parameters necessary to support an IBSS.” This does not apply to WBSS, and it is unclear if the optional WSIE constitutes an explicit definition to the contrary. A similar comment applies to the IBSS DFS element defined in Section 7.3.2.24. • Section 7.3.2.13 includes the following: “In an IBSS, the setting of the Use_Protection bit is left to the STA.” It is unclear if this applies to WBSS. • Section 7.3.2.20 includes the following: “The Channel Switch Announcement element is used by an AP in a BSS or a STA in an IBSS to advertise when it is changing to a new channel and the channel number of the new channel. … A STA in an IBSS may treat a Channel Switch Mode field set to 1 as advisory.” It is unclear if this applies to WBSS. A similar comment applies to the Channel Switch Announcement frame defined in Section 7.4.1.5. • Section 7.5, Table 7-58 lists the frame subtypes that are transmitted and received by a MAC in an IBSS. It is unclear whether all of these apply to WBSS (e.g. authentication, deauthentication), or whether exceptions are sufficiently “explicitly defined otherwise.” • Section 8 includes many specifications regarding security in an IBSS. It is unclear whether any of these apply to a WBSS. This is only a sample of instances where specifications related to IBSS do not apply (or it is unclear if they apply) to WBSS, and for which an explicit definition to the contrary is lacking. | Delete line 47: "WBSS uses ad hoc mode specifications where not explicitly defined otherwise" | Accepted | This paragraph will be removed |
| 82 | Bai , Fan | 5.2.1 | 2 | 48 | TR | TR | WBSS using ad hoc mode (i.e., IBSS mode) is not defined either in master document or in 11p amendment. Given that IBSS mode is not the key focus of 11p amendment and related technical regulations/descriptions are not elaborated in 11p amendment, 11p amendment should avoid discussing and regulating it unless very necessary. The users should refer to the IBSS mode regulated in master document for their implementation. | delete this sentence "WBSS uses ad hoc mode specifications where not explicitly defined otherwise." | Accepted | This paragraph will be removed |
| 83 | Roy, Richard | 5.2.1 | 2 | 48 | TR | TR | The proposed insertion: "WBSS uses ad hoc mode specifications where not explicitly defined otherwise." is not requried of STAs in WAVE mode and should be removed. | Remove this insertion. | Accepted | This paragraph will be removed |

- Background, Explanation, Discussion, etc.: Explained in comments above

- Recommended Resolution of the Comment:

Remove line 47/48 in page 2 of clause 5.2.1 in 11p document, specifically delete the following line “WBSS uses ad hoc mode specifications where not explicitly defined otherwise.”

- Motion (if technical and/or significant):

Move to Remove line 47/48 in page 2 of clause 5.2.1 in 11p document, specifically delete the following line “WBSS uses ad hoc mode specifications where not explicitly defined otherwise.” And intstruct the 802.11p editor to make the required change to the document

Motion by: Vinuth Rai Date: 10/10/2007

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References: