IEEE P802.11
Wireless LANs

| TGp LB 110 Comment Resolution – Extended Capabilities Information Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Francois Simon | DOT | ARINC | 410-266-4531 | fsimon@arinc.com |

Abstract

This paper addresses the comments addressing Extended Capabilities Information Element.  It include responses to CR#: 192, 193, 195, 194, 196, 219, 336, 220, 223, and 224

|  | LB110 Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| ID | Commenter | Clause | Pg | Ln | E or T | Yes or No | Comment | Suggested Remedy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 192, 193 195 | Farncois Simon Francois Simon | 7.3.2.27 7.2.3.1 | 5 5 | 45 12 | T T |  | An Extended Capabilities IE is added to the Beacon. However, it appears to be mandatory ---Make it optional Addition of Extended Capabilities IE into the Beacon frame body (Table 7-8) seems to claim that this IE would be included in all Beacon frames, not just the ones used for WAVE on-demand beacon. This would make all existing implementation not comply with the standard which is not really acceptable. This IE should only be added when needed --- Make addition of Extended Capabilities IE conditional--something like “... is present when dot11WAVEServicesEnabled is TRUE and this is an on-demand beacon frame” The On-demand beacon field within the Extended Capabilities information element indicates if a beacon is an On-demand beacon or not. The inclusion of the Extended Capability in a beacon is not Exclusive to 11p. The consequence are as follow: - If the Extended Capabilities is not included in the beacon, the beacon is NOT an On-demand beacon. - If the Extended Capabilities is included in the beacon AND the On-demand beacon field is set to ‘0’ then the beacon is not an On-demand beacon; the Extended Capibilities is inserted for other reasons outside WAVE mode. - If the Extended Capabilities is inserted in the beacon AND the On-demand beacon field is set to ‘1’ then the beacon is an On-demand beacon but may not be necessarily used in WAVE mode. There are 2 conditions required to create a WAVE mode On-demand beacon: - The MIB attribute dot11WAVEServiceEnabled = true; AND - The On-demand beacon fiels in the Extended Capabilities IE (B1) is set to ‘1’. | In P802.11p/D3.0 make the following changes: Insert the first paragraph of sub-clause 7.3.2.27 (from base standard) with the indicated changes: “The Extended Capabilities information element carries information about the capabilities of an IEEE 802.11 STA to intended to augment the Capability Information field (CIF) when these bits are fully allocated. The format of this information element is shown in Figure 7-76. Note: this is not WAVE mode exclusive but required in WAVE mode.” Replace Figure 7-76a as follow: Replace the current figure with an 8 bit octet representation. All bits are marked “Reserved” except for B1 which is marked “On-demand beacon”. Each field of the octet is 1 bit wide. Above the octet, indicate each bit position with “B0 to B7” Replace the text below the figure with the following: “--- In WAVE mode, the On-demand beacon field (B1) in the Extended Capabilities information element, capabilities fields, first octet is set to 1 if the MIB attribute dot11WAVEServicesEnabled = true.” |
| 194 | Francois Simon | 7.2.3.1 | 5 | 9 | T |  | In Table 7-8, instructions for when to include the extended capabilities IE in beacons are missing. Not every STA should need to include this IE when transmitting a beacon ----Add instructions about when to include the extended capabilities IE in beacons It is correct that not every STA should need to include the Extended Capabilities IE in a beacon. The inclusion of this information element in a beacon is not exclusive to WAVE mode. Other 802.11 WGs may require its inclusion in a beacon. In WAVE mode it is mandatory. | Changes specified for CR# 192 apply |
| 196 219 336 | Francois Simon | 7.2.3.1 | 5 | 12 | T |  | The functionality and purpose of extended capabilities should be clearly stated and illustrated. Also, what is the difference between 'capabilities' and 'extended capabilities"? ---- give the explicit reference to the applicable locations of definitions of 'capabilities' and 'extended capabilities' in master document The reference given in 7.3.2.27 P802.11/D3.0 IS the reference of the Extended Capabilities information element in the base standard. The base standard specifies the purpose of this IE: “augments the Capability Information field (CIF)”. The Capability element is specified in 7.3.1.4 of the base document: IEEE Std 802.11-2007. | Changes specified for CR# 192 apply |
| 220 | Francois Simon | 7.3.2.27 | 5 | 50 | T |  | I would like to see entire figure of Extended Capabilities field --- Add or change the Figure 7-76a to show entire field of Extended Capabilities. The Extended Capabilities element format is shown in sub-clause 7.3.2.27, Figure 7-76 of the base standard. The first octet (out of 255) is shown in sub-clause 7.3.2.27, Figure 7-76a of P802.11p/D3.0. B0 is reserved by TGn, B1 is reserved for TGp, and B2 is reserved for Tgy. The remaining 5 bits of this first octet are reserved for future use as well as each bit of the 254 remaining octets. | Changes specified for CR# 192 apply. |
| 223 | Francois Simon | 7.3.2.27 | 6 | 2 | T |  | Confusing or incorrect editing instructions for the extended capabilities field: “HT Information Exchange Support” is underlined. Is it really added by 802.11p? The underlined “field first octet..” has two periods in the end. Typo? ----Underline only the text that was added in 802.11p Agree with comments. See remedy | The propose remedy for CR# 192 applies. |
| 224 | Francois Simon | 7.3.2.27 | 6 | 3 | E |  | The bits in the Extended Capabilities field are unlabelled ----Apply a label of B0 above HT Information Exchange Support and apply a label of B1 above On-demand Beacon if that is the correct bit number Agree with comments. See remedy | The propose remedy for CR# 192 applies. |

- Background, Explanation, Discussion, etc.:

Covers in comments: CR associated with the use of Extended Capabilities Information Element in the On-demand Beacon in WAVE mode as well as other modes.  The text provided above provides explanation of the use of the Extended Capabilities information element in and outside WAVE mode.

- Recommended Resolution of the Comment:

7.3.2.27 Extended Capabilities information element

Insert the first paragraph of sub-clause 7.3.2.27 with the following changes:

The Extended Capabilities information element carries information about the capabilities of an IEEE 802.11 STA, intended to augment the Capability Information field (CIF) when these bits are fully allocated. The format of this information element is shown in Figure 7-76

Replace Figure 7-76a as follow:

B0                      B1                     B2                  B3                  B4                    B5                   B6                  B7

| Reserved | On-demand beacon | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bits		     1                          1                       1                    1                  1                            1                   1                     1

Figure 7-76A – Extended Capabilities field, first octet

Change the text below the figure as follow:

– In WAVE mode, the On-demand beacon field (B1) in the Extended Capabilities information element, Capabilities fields, first octet is set to 1 if the MIB attribute dot11WAVEServicesEnabled = true.

- Motion (if technical and/or significant):

Move to accept the proposed changed in text noted above

Move to xxxxxxxxxxxxxxxxxx     …  .

Motion by: ___Francois Simon________________Date:

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References: