IEEE P802.11
Wireless LANs

| TGp LB 110 Comment Resolution for Annex I |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jeremy Landt | TransCore | 8600 Jefferson St NE Albuquerque, NM 87113 | 505-856-8015 | jerry.landt@transcore.com |
| Alastair Malarky | Mark IV | 6030 Ambler Drive, Mississauga, Ontarion, Canada L4W 2P1 | 905-624-3020 | amaalrky@ivhs.com |

Abstract

This document addresses the LB 110 comments submitted for Annex I.  It includes responses to Comments 526 to 536.

|  | LB110 Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Comment Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 526 | Roy, Richard | Annexes I and J | 28 | 30 | TR | The European frequency and channel allocations specified in previous submissions are still missing. | Add the European channelization and freq1uency allocations to the annexes as proposed in previous submission 11-07-2228-00-000p-regulatory-language-change.doc. | (Counter) Table I.2 to be updated as recommended and corrected with appropriate information on European channels. Table I-2 to be updated by addition of “ITS radio service” as prefix fo Classes A through D and 20 MHz fo clarity and to reflect FCC nomenclature. No modification will be made to the frequency channels. 5 MHz and 40 MHz WAVE are not approved in the FCC regulations. |
| 527 | Ecclesine, Peter | I.1 | 28 | 41 | E | Editing instruction is to change United States entry, but changes to Europe are also present. Remove changes to Europe. | per comment | Accepted (ETS 300-328 included in 802.11kD9) |
| 528 | Ecclesine, Peter | I.2.2 | 30 | 20 | E | Table I.4 was changed by 11y to have two US columns, and is not shown properly here. | per comment | (Accepted) The missing column is to be added and the appropriate data entered in the missing column. |
| 529 | Goodall, David | I.2.2 | 30 | 36 | TR | The centre label for the USA defines the maximum output power with a 6 dBi antenna but the value of 760 below the label is quoted with a 13 dBi antenna, which is contradictory. | Clarify the maximum output power for the USA. | (Accepted) For Table I.4, a missing column was added (see Comment 528). Additional information was added to table entries for clarification. |
| 530 | Kolze, Thomas | I.2.3 | 30 | 40 | TR | Spectrum mask is too tight for the Class C and D devices, is not justified, and makes it too hard to implement. | Use same spectrum mask as for the Class A or Class B devices. | (Declined) Comment is out of scope. Informative Annex I reports national regulations that bear on implementation of equipment. IEEE 802.11 cannot change these regulations. Change of national spectrum masks must be done within the rule and regulation change process for the country of interest. |
| 531 | Wang, Qi | I.2.3 | 30 | 40 | TR | Spectrum mask is too tight for the Class C and D devices, hard to implement. | Use same spectrum mask as for the Class A or Class B devices. | (Declined) Out of scope. See resolution of Comment 530 |
| 532 | Young, Christopher | I.2.3 | 30 | 40 | TR | Spectrum mask is too tight for the Class C and D devices, hard to implement. | Use same spectrum mask as for the Class A or Class B devices. | (Declined) Out of scope. See resolution of Comment 530 |
| 533 | Kavner, Doug | Table I.7 | 31 | 16 | E | There is no class E defined | Change to "Class A thru D" | Accepted (Editorial) |
| 534 | Hart, Brian | I.2.3 | 31 | 16 | E | No Class E in table | Terminate at Class D | Accepted (Editorial) |
| 535 | Caam-Winget, Nancy | I.2.3 | 31 | 16 | E | No Class E in table | Terminate at Class D | Accepted (Editorial) |
| 536 | Bai , Fan | I.2.3 | 31 | 16 | ER | The caption of Table I.7 seem to be "Class A thru D…", rather than "Class A thru E…" | Change it to "Class A thru D" | Accepted (Editorial) |

- Background, Explanation, Discussion, etc.:

Annex I changes are based on IEEE Std 802.11TM-2007 as amended by P802.11k-D9.0, P802.11n-D3.0, P802.11r-D8.0 and P802.11y-D5.0.  (P802.11w and P802.11u have no changes to Annex I)

Updated Table I.1 per comment 527 and correcting for referenced prior amendments (90.1301-90.1337 added by amendment y-D5.0)

Comment 526 and correcting for referenced prior amendments

Comments 528 and 529 accepted and clarifying language added to table

Comments 530, 531 and 532 rejected as out of scope.

- Recommended Resolution of the Comment:

Table I.1 – change the editor instruction to be  :

In Table I.1- Regulatory requirement list, Change the list of documents for the United States  as shown:

| - Regulatory requirement list |  |  |  |
| --- | --- | --- | --- |
| Geographic area | Approval standards | Documents | Approval authority |
| United States | Federal Communications Commission (FCC) | FCC 47 CFR [B8], Part 15, Sections 15.205, 15.209, and 15.247; and Subpart E, Sections 15.401-15.407, Section 90.210, Section 90.371-383, Section 90.1201-90.1217, 90.1301-90.1337, Section 95.639, Section 95.1501-1511 | FCC |

Change Table I.2 instruction to be :

Insert 5 new Emissions limits sets for WAVE in Table I.2- Emissions limits sets, starting at the first available set (currently#7 – kD9.0  adds #6), as shown below, and changing the reserved row accordingly:

| - Emissions limits sets |  |  |  |
| --- | --- | --- | --- |
| Emissions limits set | USA | Europe | Japan |
| 7 ITS radio service Class A | FCC CFR 47, Sections 90.375, 90.377, 95.639 and 95.1511 | Reserved | Reserved |
| 8 ITS radio service Class B | FCC CFR47, Sections 90.375, 90.377, 95.639 and 95.1511 | Reserved | Reserved |
| 9 ITS radio service Class C | FCC CFR47, Sections 90.375, 90.377, 95.639 and 95.1511 | Reserved | Reserved |
| 10 ITS radio service Class D | FCC CFR47, Sections 90.375, 90.377, 95.639 and 95.1511 | Reserved | Reserved |
| 11 ITS radio service 20 MHz Channels | FCC CFR47, Sections 90.375, 90.377, 95.639 and 95.1511 | Reserved | Reserved |
| 12 -255 | Reserved | Reserved | Reserved |

Change Table I.3 instructions to be :

Insert 2 new behavior limits sets for WAVE in Table I.3- Behavior limits sets, starting at the first available set (currently#15, nD3.0 ends at #14), as shown below, and changing the reserved row accordingy:

| - Behavior limits sets |  |  |  |
| --- | --- | --- | --- |
| Behavior limits sets | USA | Europe | Japan |
| 15 ITS radio service Coverage Class 1 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 1 | Reserved | Reserved |
| 16 ITS radio service Coverage Class 0 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 0 | Reserved | Reserved |
| 17-255 | Reserved | Reserved | Reserved |

Change Table I.4 instruction to be :

Insert a new entry for WAVE into Table I.4 - Transmit Power by Regulatory Domain as shown below:

| - Transmit power level by regulatory domain |  |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency band (GHz) | USA (Maximum output power with up to 6 dBi antenna gain) (mW) | USA EIRP | Europe (EIRP) |  |
| 5.85–5.925 | 760 Power level at antenna input. Antenna gain greater than 6 dBi is allowed although additional limitations apply per FCC 47CFR [B8], 90.375 | 44.8 dBm (30 W) Additional limitations apply per FCC 47CFR [B8], 90.375 | — |  |

Change the title of Table I.7 to be :

Table I.7 --  Class A thru Class D spectrum masks

See the right column above for the resolutions of the other individual comments.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D3.0 noted above and instruct the editor to make these changes to P802.11p D3.0.

Motion by: ____________________Date: _________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References: