| Native GAS Update to 802.11u-D1.0 (LB 107) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This document describes proposed changes to Native GAS (Generic Advertisement Service transporting Native query protocol).  This document (if approved) will resolve the following review comments (cf. 11-07-2204r12) CID #’s: 36, 37, 282, 284, 287, 1266, 1274, 1276, 1280, 1282, 1289, 1797, 1902, 2108, 2111, 2159, 2317.

This document is based on IEEE P802.11u-d1.0 as modified by 11-07-2493r1 and IEEE 802.11-2007.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <Error! Hyperlink reference not valid.>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Overview

- Normative references

- Definitions

- Abbreviations and acronyms

- General description

  - Overview of the services

- MAC Service Definition

- Frame formats

  - MAC frame formats

      - Native Query Information element

The Native Query information element is used by the GAS protocol using the procedures defined in clause 11.10.2.  The format of the Native Query information element is provided in Figure .

|  | Element ID (0) | Length | Native Query Info |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure u2—Native Query IE

Table u2—Native Query Info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 0 | 7.3.2.50 |
| mSSID List | 1 | 7.3.2.51 |
| Emergency Public Network Access Information | 2 | 7.3.2.52 |
| Network Authentication Type | 3 | 7.3.2.53 |
| Reserved | 3 – 255 |  |

The Element ID field is equal to 0 for Native Query IE.  Note that the Element ID space for GAS Native Advertisement protocol is distinct from the number space for standard 802.11 information elements.

The Length field is the length of the Native Query Information element and is determined by the number of 1-octet values in the Native Query Info field.

The Native Query Info field is a list one-octet values which are chosen from Table u3 corresponding to the information a non-AP STA is querying from an AP.

      - Native Query Response Information element

The Native Query Response information element is used by the GAS Native protocol using the procedures defined in clause 11.10.2.  The format of the Native Query information element is provided in Figure .

|  | Element ID (1) | Length | Native Info IE #1 | Native Info IE #2 (Optional) | … | Native Info IE #N (Optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | variable | variable | … | variable |

Figure u4—Native Query Response Information element format

The Element ID field is equal to 1 for Native Query Response IE.  Note that the Element ID space for GAS Native Advertisement protocol is distinct from the number space for standard 802.11 information elements.

The Length field is the length of the Native Query Response Information element and is determined by the determined by the number and size of the Native Info IEs.Each Native Info IE is one of the Native Info elements identified by the Info IDs in Table u2.

      - Native Info Capabilities List element

The Native Info Capabilities List element provides a list of information/capabilities that has been configured on an AP.  This IE may be returned in response to a GAS Native Query Request.  The format of the Native Info Capabilities List element is provided in Figure .

|  | Info ID (0) | Length | Status Code | Info ID #1 (0) | Info ID #2 (Optional) | … | Info ID #N (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 1 | 1 | … | 1 |

Figure u6—Native Info Capabilities List element format

The Info ID field is equal to 0, which corresponds to the Capabilities List element per Table u2.

The Length field is 2-octet length of the Capabilities List element and is equal to the number of Info IDs present plus 2.

The Status Code is a 2-octet field whose value is drawn from Table 23.

Each Info ID included in the Capabilities List element declares that the corresponding capability is either configured or supported in the BSS.  In other words, a Native query for that Info ID would return a non-trivial response (i.e., a trivial response being a response with status code indicating failure).  Each Info ID returned must be a member of the Info IDs in Table u2.

      - Native Info mSSID List element

The Native Info mSSID List element provides a list of non-default SSID(s) that have been configured on an AP.  This IE may be returned in response to a GAS Native Query Request.  The format of the Native Info mSSID List element is provided in Figure .

|  | Info ID (1) | Length | Status Code | SSIDC IE #1 (optional) | SSIDC IE #2 (optional) | … | SSIDC IE #N (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable | Variable | … | variable |

Figure u8—Native Info mSSID List element format

The Info ID field is equal to 1, which corresponds to the mSSID List element per Table u2.

The Length field is 2-octect the length of the mSSID List element and is the determined by the number and size of the SSIDC IEs.

The Status Code is a 2-octet field whose value is drawn from Table 23.

Each SSIDC IE in the mSSID List provides information for one of the non-default SSIDs defined in the BSS.

      - Native Info Emergency Public Network Access Information element

The Native Info Emergency Public Network Acccess Information element provides information related to network access for public emergency use.  This IE may be returned in response to a GAS Native Query Request that can be iniitated by non-AP STA to the AP that supports Emergency Services.  The format of the Public Emergency Access Information element is provided in Error: Reference source not found.  It includes zero or more duples of Emergency Subtype code and Emergency Information data elements.

|  | Info ID (2) | Length | Status Code | Emergency Subtype Code #1 | Emergency Information Data #1 | Emergency Subtype Code #2 | Emergency Information Data #2 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 1 | variable | 1 | variable | … |

Figure u10—Native Info Emergency Public Network Access Information element format

The Info ID field is equal to 2, which corresponds to the Emergency Public Network Access Information element per .

The Length field is the 2-octet length of the information element and is the determined by the number and size of the following IEs.

The Status Code is a 2-octet field whose value is drawn from Table 23.

The Emergency Subtype codes are as defined in Table u6.

Table u4—Emergency Public Network Access Information Subtypes

| Subtype Name | Subtype Code |
| --- | --- |
| Not used | 0 |
| Open Association – Emergency Services Only | 1 |
| Public Credentials | 2 |
| Reserved | 2 – 255 |

The Emergency Information Data for the Open Association subtype is the SSID IE of variable length that is configured to provide an open association for emergency access only.

The Emergency Information Data for the Public Credential subtype is shown in Figure u12—Emergency Information Data format using Public Credentials. It includes

- An Emergency Service Public Credential IE as shown in Figure u27

- SSID IE as defined in the base specification.

|  | Emergency Service Public Credentials IE | SSID IE |
| --- | --- | --- |
| Octets: | Variable | variable |

Figure u12—Emergency Information Data format using Public Credentials

      - Native Info Network Authentication Type Information element

The Native Info Network Authentication Type Information Element provides a list of authentication types that are used on the indicated SSID.

|  | Info ID (3) | Length | Status Code | Network Authentication Type Unit #1 | Network Authentication Type Unit #2 | … | Network Authentication Type Unit #N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable | variable | … | Variable |

Figure u14—Native Info Network Authentication Type element format

The Info ID field is equal to 3, which corresponds to the Network Authentication Type element per Table u2.

The Length field is the length of the Network Authentication Type element and is the determined by the number and size of the Network Authentication Type Units, as described in this subclause.

The Status Code is a 2-octet field with values from Table 23.

Each Network Authentication Type Unit has the structure shown in Figure u29.

|  | Network Authentication Type Indicator Value | Network Authentication Type Unit Length | Network Authentication Type Indicator data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure u16—Network Authentication Type Unit

The Network Authentication Type indicator value has one of the values show in Table u5.

The Network Authentication Type Unit Length field is set to the number of octets in the Network Authentication Type Unit.

Each indicator value defines additional information that may be communicated.

Table u6—Network Authentication Type indicator value

| Value | Meaning |
| --- | --- |
| 0 | Acceptance of legal terms and conditions |
| 1 | On-line enrollment supported |
| 2 | UAM |

If the Network Authentication Type indicator value is 0, the network requires the user to accept terms and conditions.  No Network Authentication Type data is present.

If the Network Authentication Type indicator value is 1, the network supports on-line enrolment.  Therefore, higher-layer software on the non-AP STA may indicate to the user that accounts may be created.  No Network Authentication Type Data is present.

If the Network Authentication Type indicator value is 2, the network supports the browser-based Universal Access Method.  In this case, the Network Authentication Type data is used to indicate the version of UAM in use, as shown in Figure u30.

|  | Network Authentication Type Indicator Value (2 - UAM) | Network Authentication Type Unit Length (2) | UAM Version |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure u18—Network Authentication Type Unit for UAM

The UAM Version is a one-octet field indicating the version of UAM in use.  When the version number is equal to 1, there is no further data required.

Editors note: insert the following subclause, renumber when necessary:

    - Native query protocol Information elements

Native query protocol information elements are defined to have a common general format consisting of a 1-octet Info ID field, a 2-octet length field, and a variable-length element-specific information field. Each element is assigned a unique Info ID as defined in this standard. The Length field specifies the number of octets in the Information field. See Figure u22a.

Info ID

Length

Information

1

2

Length

Octets:

Figure u19a—Native Query protocol Information element format

The set of valid Info IDs is defined in Table u5.

Table u5—Native query protocol info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 0 | 7.3.3.1 |
| multiple SSID Set | 1 | 7.3.3.2 |
| Emergency Public Network Access Information | 2 | 7.3.3.3 |
| Network Authentication Type | 3 | 7.3.3.4 |
| Reserved | 4 – 220 | n/a |
| Vendor Specific | 221 | n/a |
| Reserved | 222 – 255 | n/a |

Editor’s Note: after approval of this amendment, Info IDs will be managed by 802.11 Assigned Numbers Authority.

      - Native Info Capabilities element

The Native Info Capabilities element provides a list of information/capabilities that has been configured on an AP.  This IE may be returned in response to a GAS Native Query Request.  The format of the Native Info Capabilities element is provided in Figure u13.

|  | Info ID | Length | Status Code | Info ID #1 | Info ID #2 (Optional) | … | Info ID #N (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 1 | 1 | … | 1 |

Figure u20—Native Info Capabilities List element format

The Info ID field is equal to the value in Table u5 corresponding to the Capabilities List element.

The Length field is 2-octet length of the Capabilities List element and is equal to the number of Info IDs present plus 2.

The Status Code is a 2-octet field whose value is drawn from Table 23.

Each Info ID included in the Capabilities List element declares that the corresponding capability is either configured or supported in the BSS.  A Native query for any Info ID in the Capabilities List will not return a response with status code indicating failure.  Each Info ID returned is one of the Info IDs in Table u5.

      - Native Info Multiple SSID Set element

The Native Info Multiple SSID Set element provides a list of non-default SSID(s) that have been configured on an AP.  This IE may be returned in response to a GAS Native Query Request.  The format of the Native Info Multiple SSID List element is provided in Figure u14.

|  | Info ID | Length | Status Code | SSIDC IE #1 (optional) | SSIDC IE #2 (optional) | … | SSIDC IE #N (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable | Variable | … | variable |

Figure u21—Native Info Multiple SSID List element format

The Info ID field is equal to the value in Table u5 corresponding  to the Multiple SSID List element.

The Length field is 2-octect the length of the multiple SSID Set element and is the determined by the number and size of the SSIDC IEs.

The Status Code is a 2-octet field whose value is drawn from Table 23.

Each SSIDC IE in the multiple SSID Set provides information for one of the non-default SSIDs defined in the BSS.

      - Native Info Emergency Public Network Access Information element

The Native Info Emergency Public Network Acccess Information element provides information related to network access for public emergency use.  This IE may be returned in response to a GAS Native Query Request that can be iniitated by non-AP STA to the AP that supports Emergency Services.  The format of the Public Emergency Access Information element is provided in 7.3.2.44.  It includes zero or more duples of Emergency Subtype code and Emergency Information data elements.

|  | Info ID | Length | Status Code | Emergency Subtype Code #1 | Emergency Information Data #1 | Emergency Subtype Code #2 | Emergency Information Data #2 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 1 | variable | 1 | variable | … |

Figure u22—Native Info Emergency Public Network Access Information element format

The Info ID field is equal to the value in Table u5 corresponding  to the Emergency Public Network Access Information element.

The Length field is the 2-octet length of the information element and is the determined by the number and size of the following IEs.

The Status Code is a 2-octet field whose value is drawn from Table 23.

The Emergency Subtype codes are as defined in Table u6.

Table u7—Emergency Public Network Access Information Subtypes

| Subtype Name | Subtype Code |
| --- | --- |
| Not used | 0 |
| Open Association – Emergency Services Only | 1 |
| Public Credentials | 2 |
| Reserved | 2 – 255 |

The Emergency Information Data for the Open Association subtype is the SSID IE of variable length that is configured to provide an open association for emergency access only.

The Emergency Information Data for the Public Credential subtype is shown in Figure u12—Emergency Information Data format using Public Credentials. It includes

- An Emergency Service Public Credential IE as shown in Figure u27

- SSID IE as defined in the base specification.

|  | Emergency Service Public Credentials IE | SSID IE |
| --- | --- | --- |
| Octets: | Variable | variable |

Figure u23—Emergency Information Data format using Public Credentials

      - Native Info Network Authentication Type Information element

The Native Info Network Authentication Type Information Element provides a list of authentication types that are used on the indicated SSID.  The format of the Native Info Network Authentication Type element is shown in Figure u17.

|  | Info ID | Length | Status Code | Network Authentication Type Unit #1 | Network Authentication Type Unit #2 | … | Network Authentication Type Unit #N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable | variable | … | Variable |

Figure u24—Native Info Network Authentication Type element format

The Info ID field is equal to the value in Table u5 corresponding  to the Network Authentication Type element.

The Length field is the length of the Network Authentication Type element and is the determined by the number and size of the Network Authentication Type Units, as described in this subclause.

The Status Code is a 2-octet field with values from Table 23.

Each Network Authentication Type Unit has the structure shown in Figure u29.

|  | Network Authentication Type Indicator Value | Network Authentication Type Unit Length | Network Authentication Type Indicator data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure u25—Network Authentication Type Unit

The Network Authentication Type indicator value has one of the values show in Table u5.

The Network Authentication Type Unit Length field is set to the number of octets in the Network Authentication Type Unit.

Each indicator value defines additional information that may be communicated.

Table u8—Network Authentication Type indicator value

| Value | Meaning |
| --- | --- |
| 0 | Acceptance of legal terms and conditions |
| 1 | On-line enrollment supported |
| 2 | UAM |

If the Network Authentication Type indicator value is 0, the network requires the user to accept terms and conditions.  No Network Authentication Type data is present.

If the Network Authentication Type indicator value is 1, the network supports on-line enrolment.  Therefore, higher-layer software on the non-AP STA may indicate to the user that accounts may be created.  No Network Authentication Type Data is present.

If the Network Authentication Type indicator value is 2, the network supports the browser-based Universal Access Method.  In this case, the Network Authentication Type data is used to indicate the version of UAM in use, as shown in Figure u30.

|  | Network Authentication Type Indicator Value (2 - UAM) | Network Authentication Type Unit Length (2) | UAM Version |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure u26—Network Authentication Type Unit for UAM

The UAM Version is a one-octet field indicating the version of UAM in use.  When the version number is equal to 1, there is no further data required.

- 11.10.1 Interworking Procedures: Generic Advertisement Services

11.10.1.1	Native GAS Protocol

        - Native GAS Procedures at the Non-AP STA

Editors note: modify the following text as indicated:

Upon receipt of a MLME-GAS.request primitive with Advertisement Protocol ID set to Native Query Protocol, the non-AP STA shall engage in a Native GAS message exchange according to the following procedure:

- The non-AP STA transmits a Native GAS query by transmitting a GAS Initial Request Action frame containing a Dialog Token, an Advertisement Protocol information element containing an Advertisement Protocol ID set to Native Query protocol and one Native Query Info ID value, drawn from Table u5, in the query request field.  If the Native Query Info ID value is equal to the Vendor Specific value, then the Vendor Specific information element (see 7.3.2.26) shall be appended to the Info ID value in the query request field.

        - Native GAS Procedures at the AP

Editors note: modify the following text as indicated:

Upon receipt of a GAS Initial Request Action frame with Advertisement Protocol ID set to Native Query Protocol, an MLME-GAS.indication primitive shall be issued to the AP’s SME.  Upon receipt of a MLME-GAS.response primitive, the AP shall transmit a GAS Initial Response Action frame to the requesting STA according to the following procedures.  If the requesting STA is in state 3 and in the power-save state, the AP shall buffer the frame for transmission according to the procedures in 11.2.1; otherwise the AP shall queue the frame for transmission.  The following fields in a GAS Initial Response Action frame are not used by Native GAS and shall be set to 0: GAS Query ID, GAS multicast address and Comeback delay.

- If the query request corresponds to information that has been configured on the AP, the AP shall transmit a directed GAS Initial Response Action frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request Action frame, a Status Code set to “success”, an Advertisement Protocol information element containing an Advertisement Protocol ID set to Native Query protocol and a query response containing the Native Info element corresponding to the query (Table u5).  If the query request Info ID value is equal to the Vendor Specific value, then the Vendor Specific information element (see 7.3.2.26) shall be returned in the query response field.