IEEE P802.11
Wireless LANs

| Normative Text Changes for TGv LB108 RM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR 97229 | 503-264-7799 | Emily.H.Qi@intel.com |
| Joe Epstein | Meru Networks | 1309 S. Mary Ave. Sunnyvale, CA 94086 | +1 408 215 5345 | jepstein@merunetworks.com |

Abstract

This document is a normative text proposal to provide comment resolutions for TGv LB108 Roaming Management Category, Especially CID #98, #753, #974, #1036, #1166, and #1359. Changes are shown with respect to TGv draft D1.02.

The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID#753:

TGv Editor: Add the following text after the title of clause 7.3.1:

7.3.1.7 Reason Code field

Insert the following new row before the entry “Reserved” and change the entry “Reserved” in Table 7-22 as follow:

| 46 | Dissociated due to BSS Transition Management. |
| --- | --- |
| 467-65535 | Reserved |

CID#974, #1166, #1036, and #1359:

Summary: “May” and “Shall” inconsistency, lack of capability bit and MIB variables.

- Extended Capabilities information element

TGv Editor: Insert the following new row to Table 35a, and change the reserved row as follows:

| <ANA>+12 | BSS Transition | The BSS Transition bit set to 1 indicates that the STA supports BSS Transition. The BSS Transition bit set to 0 indicates that the STA does not support BSS Transition. (CID#1036) |
| --- | --- | --- |
| <ANA>+13-n*8 | Reserved | All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

- BSS Transition Management for Network Load Balancing

TGv Editor: Change the following clauses as follows

11.20.7.1 BSS Transition Capability

The BSS Transition Management Query, BSS Transition Management Request, BSS Transition Management Response frames provide a means and a protocol to exchange the information needed to enable a network to manage BSS loads by influencing STA transition decisions and by initiating STA transition to selected target BSS(s).

This protocol enables the improved throughput, effective data rate and/or QoS for the aggregate of STAs in a network by shifting (via transition) individual STA traffic loads to more appropriate points of association within the ESS. The mechanisms and procedures for the transition of a STA from one BSS to another are covered elsewhere in this specification.

A STA supporting use of BSS Transition shall indicate such support by using the Extended Capabilities information element.

The provisions in this clause for BSS Transition Management and network load balancing do not apply in an IBSS.

- BSS Transition Management Query

(CID#1363) A non-AP STA supporting BSS Transition may request a BSS Transition Candidate list by sending a BSS Transition Management Query frame to its associated AP if the associated AP indicates that it supports such capability in the Extended Capabilities information element.  (CID#974)

- BSS Transition Management Request

An AP supporting BSS Transition may send an unsolicited BSS Transition Management Request frame to a non-AP STA at any time if the non-AP STA indicates that it supports such capability in the Extended Capabilities information element.

Upon receipt of a BSS Transition Management Query frame, from a non-AP STA, the AP shall respond with a BSS Transition Management Request Frame.

The Preferred Candidate List Included bit-field is set to “0” indicates that the receiving STA may ignore the Preferred Candidate List. The Preferred Candidate List Included bit-field set to “1” indicates that the sender expects the receiving STA to process this frame.

The Abridged bit-field in the Request Mode field set to “1” indicates that every BSSID under consideration by the receiving STA is treated as if it were present in the BSS Transition Candidate List with a Preference value of 0. The Abridged bit-field set to “0” indicates that the sender of this frame has no recommendation for or against any BSSID not present in the BSS Transition Candidate List Entries.

Disassociation Imminent bit-field in the Request Mode field set to “1” indicates that STA is to be disassociated from the current AP. Disassociation Imminent bit set to “0” indicates BSS Transition Management Request is a “Suggestion” for STA’s BSS transition.

The Disassociation Timer field is included in the BSS Transition Management Request frame when the Disassociation Imminent bit is set to “1”, and is not included when the Disassociation Imminent bit is set to “0”. The Disassociation Timer indicates the time after which the AP will issue a Disassociation frame to this STA. A value of 0 indicates that the serving AP has not determined when it sends a Disassociation frame to this STA.

The AP shall include the BSS Transition Candidate List Entries field in the BSS Transition Management Request frame. The BSS Transition Candidate List Entries field contains one or more Neighbor Report elements describing the preferences for target AP candidates. The candidate Preference value is a number ranging from 0 to 255, indicating a relative preference for this transition candidate and the STA. A value of Preference “0” indicates that the AP being listed is an “excluded” AP, and STA shall not attempt to associate to the listed AP, unless the STA is unable to establish effective service with any non-“excluded” APs that are compatible with the STA’s choice of SSID and, if the STA is associated with the AP that sent the Request, the STA has transmitted a BSS Transition Candidate Response frame with the reason of “reject”. The Preference values are used only to establish the relative order of entries within the given list at the given time, and for the given AP. The values between 1 and 255 provide the indication of order, with 1 indicating the most preferred AP within the given candidate list, increasing numbers representing decreasing preference relative only to entries with lower values of the Preference field, and equal numbers representing equal preference. The preference value is only valid before the Validity Interval has expired. Zero or more sub-elements may be included in the BSS Transition Candidate List Entries field. Load Sub-elements contain additional information describing the BSS load and QBSS Admission Control Capacity. The sub-element is used by a non-AP QoS STAs to select a QoS AP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC shall admit these requests.

A STA receiving a BSS Transition Management Request frame containing BSS Transition Candidate List Entries may use this list of candidates, with their individual transition preference values, to make BSS transition decisions. Upon receiving a BSS Transition Management Request, the STA shall disregard any previous BSS Transition Management Request received from the same AP. The STA shall not use the corresponding BSS Transition Candidate List Entries information after the indicated Validity Interval. The STA may send a BSS Transition Management Query frame at any time to obtain an updated BSS Transition Candidate List Entries field.

A STA receiving a BSS Transition Management Request frame containing a Disassociation Timer value shall attempt to re-associate with some other AP before the indicated disassociation time. If the receiving STA cannot find a suitable AP with which to associate, the STA shall send a BSS Transition Management Response frame containing a Status Code indicating “reject” before the indicated disassociation time.

- BSS Transition Management Response

A STA receiving a BSS Transition Management Request frame may respond with a BSS Transition Management Response frame.

A STA may include the result of its BSS transition decision in the Target BSSID field and Status Code field in the BSS Transition Management Response frame. A Status Code set to “accept” indicates the STA will transition from the current BSS. The STA receiving a BSS Transition Management Request frame may reply with a BSS Transition Management Response frame with a valid nonzero status code indicating rejection if it is unable to comply with this BSS Transition Management Request.

- Wireless Network Management extensions

TGv editor: Change the following text as below:  (CID#1166)

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionPresenceEnabled 		TruthValue,

dot11MgmtOptionFBMSImplemented		TruthValue,

dot11MgmtOptionFBMSEnabled			TruthValue,

dot11MgmtOptionEventsEnabled			TruthValue,

dot11MgmtOptionDiagnosticsEnabled		TruthValue,

dot11MgmtOptionMultiBSSIDImplemented		TruthValue,

dot11MgmtOptionMultiBSSIDEnabled		TruthValue,

dot11MgmtOptionTFSImplemented 		TruthValue,

dot11MgmtOptionTFSEnabled			TruthValue,

dot11MgmtOptionSleepModeImplemented		TruthValue, 																			 			dot11MgmtOptionSleepModeEnabled				TruthValue,

dot11MgmtOptionTIMBroadcastImplemented		TruthValue,

dot11MgmtOptionTIMBroadcastEnabled			TruthValue,

dot11MgmtOptionBSSTransitionEnabled		TruthValue }

TGv editor: Add the following text at the end of A.4.18:

dot11MgmtOptionBSSTransitionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide BSS Transition is enabled. The capability is disabled, otherwise. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 14 }

CID#98:

TGv Editor: Change 10.3.54, 10.3.55 and 10.3.56 as follows:

- BSS Transition Management Query

10.3.54.1 BSS Transition Management ProcedureError: Reference source not found109 depicts the BSS Transition Management procedurs. The figure shows an example of the BSS Transition Management protocol.

- BSS Transition Management request — accepted

      - MLME-BTMQUERY.request

10.3.54.2.1

Function

This primitive requests transmission of a BSS Transition Management Query frame to the AP with which the STA is associated.

10.3.54.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTMQUERY.request	(
Peer MAC Address,
DialogToken,
BSSTransitionQueryReason
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the BSS Transition Management Query frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify this BSS Transition Management transaction. |
| BSSTransitionQuery Reason | Integer | 0 – 255 | As defined in 7.4.11.9. |

10.3.54.2.3 When Generated

This primitive is generated by the SME to request that a BSS Transition Management Query frame be sent to the AP with which the STA is associated to initiate a BSS Transition Management procedure.

10.3.54.2.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a BSS Transition Management Query management frame of action type. The STA then attempts to transmit the frame to the AP with which it is associated.

|  |  | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE |  |
| --- | --- | --- | --- |

10.3.54.3

MLME-BTMQUERY.indication

10.3.54.3.1 Function

This primitive indicates that a BSS Transition Management Query frame was received from a non-AP STA.

10.3.54.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTMQUERY.indication	(
Peer MAC Address,
DialogToken,
BSSTransitionQueryReason
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a BSS Transition Management Query frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token identifying this BSS Transition Management transaction as received in the BSS Transition Management Query frame. |
| BSSTransitionQuery Reason | Integer | 0 – 255 | The BSS Transition Query Reason Code in the BSS Transition Management Query frame that was received. |

10.3.54.3.3 When Generated

This primitive is generated by the MLME when a valid BSS Transition Management Query frame is received.

10.3.54.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.20.7.

10.3.54.4

MLME-BTM.request

10.3.54.4.1 Function

This primitive requests transmission of a BSS Transition Management Request frame to a non-AP STA.

10.3.54.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTM.request	(
Peer MAC Address
DialogToken,
RequestMode,
DisassociationTimer,
BSSTransitionCandidateList
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the BSS Transition Management Request frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the BSS Transition Management transaction. Set to 0 for an autonomous BSS Transition Management Request frame. |
| RequestMode | Integer | As specified in 7.4.11.10 | Contains the Disassociation Imminent and Abridged bits for the BSS Transition Management Request. |
| Disassociation Timer | Integer | 0 – 255 | Specifies the number of TBTTs until the AP shall disassociate the non-AP STA. A value of 0 indicates time of disassociation has not yet been determined and a value of 1 indicates disassociation shall occur before the next TBTT. |
| BSSTransition CandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.37 | Contains the description of candidate BSS transition APs and their capabilities as described in section 7.3.2.37. |

10.3.54.4.3 When Generated

This primitive is generated by the SME to request that a BSS Transition Management Request frame be sent to an associated non-AP STA. This request is sent either following the reception of a MLME-BTMQUERY.indication or may be sent autonomously.

10.3.54.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a BSS Transition Management Request management frame of action type. The STA then attempts to transmit this frame to the indicated non-AP STA.

10.3.54.5

MLME-BTM.indication

10.3.54.5.1 Function

This primitive indicates that a BSS Transition Management Request frame was received from the AP with which the STA is associated. This primitive is also generated when the MLME-BTMQUERY.request contains invalid parameters and when a timeout or failure occurs.

10.3.54.5.2

Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTM.indication	(
ResultCode,

Peer MAC Address,
DialogToken,
RequestMode,
DisassociationTimer,
BSSTransitionCandidateList
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-BTMQUERY.request. The ResultCode field is set to SUCCESS if the received BSS Transition Request frame is an unsolicited frame. |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the MAC entity from which a BSS Transition Management Request frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identity this BSS Transition Management transaction as received in the BSS Transition Management Request frame. |
| RequestMode | Integer | As specified in 7.4.11.10 | Contains the Disassociation Imminent and Abridged bits for the BSS Transition Management Request. |
| Disassociation Timer | Integer | 0 – 255 | Specifies the number of TBTTs until the AP shall disassociate the non-AP STA. A value of 0 indicates time of disassociation has not been determined yet and a value of 1 indicates disassociation shall occur before the next TBTT. |
| BSSTransition CandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.37 | Contains the description of candidate BSS transition APs and their capabilities as described in 7.3.2.37. |

10.3.54.5.3 When Generated

This primitive is generated by the MLME when a valid BSS Transition Management Request frame is received.

10.3.54.5.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.20.7.

10.3.54.6 MLME-BTM.response

10.3.54.6.1 Function

This primitive requests transmission of a BSS Transition Management Response frame to the AP with which the STA is associated.

10.3.54.6.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTM.response	(
Peer MAC Address,
DialogToken,
StatusCode,
TargetBSSID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the BSS Transition Management Query frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the BSS Transition Management transaction. |
| StatusCode | Integer | 0 – 255 | As defined in 7.4.11.11. |
| TargetBSSID | MACAddress | Any valid individual MAC Address | The BSSID of the BSS that the STA decided to roam to. Field shall be null if STA decided not to roam. |

10.3.54.6.3 When Generated

This primitive is generated by the SME to request that a BSS Transition Management Response frame be sent to the AP with which the STA is associated.

10.3.54.6.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a BSS Transition Management Response management frame of action type. The non-AP STA then attempts to transmit this to the AP with which it is associated.

|  |  | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE |  |
| --- | --- | --- | --- |

10.3.54.7 MLME-BTM.confirm

10.3.54.7.1  Function

This primitive indicates that a BSS Transition Management Response frame was received from a non-AP STA. This primitive is also generated when the MLME-BTM.request contains invalid parameters and when a timeout or failure occurs.

10.3.54.7.2  Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTM. confirm	(
ResultCode,

Peer MAC Address,
DialogToken,
BSSTransitionQueryReason,
StatusCode,
TargetBSSID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-BTM.request. |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a BSS Transition Management Response frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify this BSS Transition Management transaction as received in the BSS Transition Management Response frame. |
| StatusCode | Integer | 0 – 255 | As defined in 7.4.11.11. |
| TargetBSSID | MACAddress | Any valid individual MAC Address | The BSSID of the BSS that the STA indicated to roam to as received in the BSS Transition Management Response frame. |

10.3.54.7.3  When Generated

This primitive is generated by the MLME when a valid BSS Transition Management Response frame is received.

10.3.54.7.4  Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.20.7.