IEEE P802.11
Wireless LANs

| Normative Text Changes for LB108 Sleep Mode and TFS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR 97229 | 503-264-7799 | Emily.H.Qi@intel.com |

Abstract

This document is a normative text proposal to provide comment resolutions for TGv LB108 Roaming Management Category, Especially CID #546, #544, #545, #869, and #1029. Changes are shown with respect to TGv draft D1.02.

The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID#869:

TGv Editor: Change the following text as follows:

- Reassociation Request frame format

| - Reassociation Request frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | TFS Request | The TFS Request element can be present if dot11WirelessManagementImplemented is true and the TFS bit in the Extended Capabilities element is set to 1. One or more TFS Request information elements can appear in this frame. |

- Reassociation Response frame format

| - Reassociation Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | TFS Response | The TFS Response element can be present if dot11WirelessManagementImplemented is true and the TFS bit in Extended Capabilities element is set to 1 and the TFS Request element was present in the corresponding Reassociation Request frame. One or more TFS Response information elements can appear in this frame. |

- TFS Request frame format

The TFS Request frame is sent by a non-AP STA to the AP to request the specified traffic filtering. The format of TFS Request frame is defined in TFS Request frame format.

|  | Category | Action | Dialog Token | TFS Request Elements | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |
| - TFS Request frame format |  |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in Error: Reference source not found.

The Action field is set to the value indicating TFS Request frame, as specified in Error: Reference source not found in Error: Reference source not found.

The Dialog Token field is set to a value chosen by the STA sending the TFS Request frame to identify the request/response transaction.

The TFS Request Elements field contains zero or more TFS Request elements to specify the traffic filters that are requested by the non-AP STA, as defined in Error: Reference source not found.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

- TFS Response frame format

The TFS Response frame is sent by an AP in response to a TFS Request frame. The format of the TFS Response frame is defined in TFS Response frame format.

|  | Category | Action | Dialog Token | TFS Response Elements | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |
| - TFS Response frame format |  |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in Error: Reference source not found.

The Action field is set to the value indicating TFS Response frame, as specified in Error: Reference source not found in Error: Reference source not found.

The Dialog Token field is set to the value in the corresponding TFS Request frame.

The TFS Response Elements field contains zero or more TFS Response elements to indicate the traffic filters that the AP is configured to support, as defined in Error: Reference source not found.

The Vendor Specific Elements field contains zero or more Vendor Specific element, as defined in 7.3.2.26.

- Sleep Mode Request frame format

The Sleep Mode Request frame is sent by a non-AP STA to the AP to enter the Sleep Mode. The format of the Sleep Mode Request frame is defined in Sleep Mode Request frame format.

|  | Category | Action | Dialog Token | Sleep Mode Element | TFS Request Elements | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | variable |
| - Sleep Mode Request frame format |  |  |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in Error: Reference source not found.

The Action field is set to the value indicating Sleep Mode Request frame, as specified in Error: Reference source not found in Error: Reference source not found.

The Dialog Token field is set to a value chosen by the non-AP STA sending the Sleep Mode Request frame to identify the request/response transaction.

The Sleep Mode Element field contains a Sleep Mode element that is requested by a non-AP STA, as described in Error: Reference source not found.

The TFS Request Elements field contains zero or more TFS Request elements to specify the traffic filters that are requested by a non-AP STA, as defined in Error: Reference source not found.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

- Sleep Mode Response frame format

The Sleep Mode Response frame is sent by an AP in response to a Sleep Mode Request frame. The format of the Sleep Mode Response frame is defined in Sleep Mode Response frame format..

|  | Category | Action | Dialog Token | Sleep Mode Element | TFS Response Elements | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | variable |
| - Sleep Mode Response frame format. |  |  |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in Error: Reference source not found.

The Action field is set to the value indicating Sleep Mode Response frame, as specified in Error: Reference source not found in Error: Reference source not found.

The Dialog Token field is set to the value in the corresponding Sleep Mode Request frame.

The Sleep Mode Element field contains a Sleep Mode element, as described in Error: Reference source not found.

The TFS Response Elements field contains zero or more TFS Response elements to specify the traffic filters, as defined in Error: Reference source not found.

The Vendor Specific Elements field contains zero or more Vendor Specific element, as defined in 7.3.2.26.

- Reassociate

- MLME-REASSOCIATE.request

- Semantics of the service primitive

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSRequest | A set of TFS Request elements | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. One or more TFS Request elements can be present only if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Extended Capabilities is set to 1. |

- MLME-REASSOCIATE.confirm

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies service parameters for the TFS. One or more TFS Response elements can be present only if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Extended Capabilities is set to 1 and the TFS Request element is present in corresponding Reassociation Request frame. |

- MLME-REASSOCIATE.indication

- Semantics of the service primitive

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSRequest | A set of TFS Request elements | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. One or more TFS Request elements can be present only if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Extended Capabilities is set to 1. |

- MLME-REASSOCIATE.response

- Semantics of the service primitive

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies service parameters for the TFS. One or more TFS Response elements can be present only if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Extended Capabilities is set to 1 and the TFS Request element is present in the corresponding Reassociation Request frame. |

- TFS Setup

The following MLME primitives support the signaling of TFS Setup. The signaling is shown in TFS request and response exchange.

TGv Editor: Replace Figure v112 with the following figure.

- TFS request and response exchange

- MLME-TFS.request

- Function

This primitive requests that a TFS Request frame be sent to the AP with which the STA is associated.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.request	(
PeerSTAAddress,
DialogToken,
TFSRequest,
VendorSpecific)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity for TFS. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the TFS Request and Response transaction. |
| TFSRequest | A set of TFS Request elements | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. One or more TFS Request elements. |
| VendorSpecific | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When Generated

This primitive is generated by the SME to request that a TFS Request frame be sent to the AP with which the STA is associated.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a TFS Request action management frame. The STA then attempts to transmit this to the AP with which it is associated.

CID#544:

- MLME-TFS.confirm

- Function

This primitive reports the result of a TFS procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.confirm	(

ResultCode,
TFSResponse,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-TFS.request. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the TFS Response frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the TFS Request and Response transaction. |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies service parameters for the TFS. One or more TFS Response elements. |
| VendorSpecific | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When Generated

This primitive is generated by the MLME as a result of the receipt of a TFS Response action management frame. It indicates the results of the request.

This primitive is also generated when the MLME-TFS.request contains invalid parameters and when a timeout or failure occurs.

- Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

- MLME-TFS.indication

- Function

This primitive indicates that a TFS Request frame was received from a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.indication	(
PeerSTAAddress,
DialogToken,
TFSRequest,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TFS Request frame was received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the TFS Request and Response transaction. |
| TFSRequest | A set of TFS Request elements | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. One or more TFS Request elements |
| VendorSpecific | One or more information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When Generated

This primitive is generated by the MLME when a valid TFS Request action management frame is received.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 9.2.7.1.1.

- MLME-TFS.response

- Function

This primitive is generated in response to an MLME-TFS.indication requesting a TFS Response frame be sent to a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.response	(
PeerSTAAddress,
DialogToken,
TFSResponse,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TFS Request frame was received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the TFS Request and Response transaction. |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies service parameters for the TFS. One or more TFS Response elements. |
| VendorSpecific | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When Generated

This primitive is generated by the SME in response to an MLME-TFS.indication requesting a TFS Response be sent to a non-AP STA.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a TFS Response action management frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

- Sleep Mode request

This set of primitives supports the exchange of sleep mode parameter information between peer SMEs. Sleep Mode Request and Response Exchange depicts the sleep mode request and response process. Sleep Mode Request and Response Exchange illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses.

TGv Editor: Replace Figure v113 with the following figure.

- Sleep Mode Request and Response Exchange

- MLME-SLEEPMODE.request

- Function

This primitive requests that a Sleep Mode Request frame be sent to the AP with which the STA is associated.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.request	(
PeerSTAAddress,
DialogToken,
SleepMode,
TFSRequest,
VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Sleep Mode Request frame is to be sent. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Request frame. |
| TFSRequest | A set of TFS Request elements | As defined in the TFS Request element | Specifies the proposed TFS service parameters for the Sleep Mode Request frame. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the SME to request that a Sleep Mode Request frame be sent to the AP with which the STA is associated.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a Sleep Mode Request action frame. The STA then attempts to transmit this to the AP with which it is associated.

- 10.3.61.2 MLME-SLEEPMODE.indication

- Function

This primitive indicates that a Sleep Mode Request frame was received from a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.indication	(
PeerSTAAddress,
DialogToken,
SleepMode,
TFSRequest,
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Sleep Mode request frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Request frame. |
| TFSRequest | A set of TFS Request elements | As defined in the TFS Request element | Specifies the proposed TFS service parameters for the Sleep Mode Request frame. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the MLME when a valid Sleep Mode Request frame is received.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.12.

- MLME-SLEEPMODE.response

10.3.61.3.1 Function

This primitive requests the transmission of Sleep Mode information to a peer entity, in response to a received Sleep Mode Request frame.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.response	(
PeerSTAAddress,
DialogToken,
SleepMode,
TFSResponse,
VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Sleep Mode Response frame is to be sent. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Response frame. |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies the proposed TFS service parameters for the Sleep Mode Response frame. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the SME to request that a Sleep Mode Response frame be sent to a peer entity to convey Sleep Mode information.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a Sleep Mode Response frame containing the Sleep Mode elements specified. This frame is then scheduled for transmission.

CID#546:

- MLME-SLEEPMODE.confirm

- Function

This primitive reports the result of a request to send a Sleep Mode Response action frame.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.confirm	(
ResultCode,

PeerSTAAddress,
DialogToken,
SleepMode,
TFSResponse,
VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-SleepMode.request. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the Sleep Mode Response frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Response frame. |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies the proposed TFS service parameters for the Sleep Mode Response frame. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the MLME when a Sleep Mode Response frame is received. It indicates the results of the request.

This primitive is also generated when the MLME- SLEEPMPDE.request contains invalid parameters and when a timeout or failure occurs.

- Effect of Receipt

On receipt of this primitive, the SME evaluates the result code.

CID#1029:

TGv editor: Insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry::=

SEQUENCE {

dot11WirelessManagementImplemented TruthValue,

dot11MaxIdlePeriod INTEGER }

TGv editor: Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11MaxIdlePeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute indicates that the number of 1000 TUs that pass before an AP disassociates an inactive non-AP STA.”

::= { dot11StationConfigEntry 60}