IEEE P802.11
Wireless LANs

| Normative Text for High Throughput Mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho SEOK | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Alexander SAFONOV | IITP RAS | 19 Bolshoy Karetny per, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |

Abstract

This document is a normative text proposal to provide comment resolution for TGs LB 93 comment 235, 1935, 3745, 4206 and 4228. Changes are shown with respect to TGs draft D1.07.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment Summary

- Don't forget in any of the primitives that exchange STA capability information, to include the HT Capability element.

- Is it possible to send an A-MSDUs in the body of a Mesh Data frame?

- Given that an MSDU size can be up to 2304 bytes, it appears to me that the size of the Body is greater than 2312. Also, Can A-MSDUs be sent in the body of a Mesh Data frame?

- How does TGs interact with TGn features?

- Add the following paragraph at the end of 7.2.2.2

- Aggregate MSDU format (A-MSDU)

(Ed: CID 3727)

When the Mesh Data frames are aggregated, the Subframe Header contains four fields: Destination MP Address (Mesh DA), Source MP Address (Mesh SA), Mesh Header and Length.

| Octets: 6 |  | 6 |  | 5 or 17 |  | 2 |  | 0-2304 |  | 0-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mesh DA | Mesh SA |  | Mesh Header |  | Length |  | MSDU |  | Padding |  |
| Subframe Header |  |  |  |  |  |  |  |  |  |  |
| - A-MSDU Subframe structure for Mesh Data |  |  |  |  |  |  |  |  |  |  |

An A-MSDU shall only contain MSDUs whose Mesh DA, Mesh SA and Mesh Header parameter values map to the same RA and TA values, i.e., all the MSDUs are intended to be received by a single receiver, and necessarily they are all transmitted by the same transmitter.

NOTE 4—It is possible to have different Mesh DA, Mesh SA and Mesh Header in Subframe Headers of the same A-MSDU as long as they all map to the same Address 1 and Address 2 values.

(Ed: CID 25)

- Change 7.4.9.1, 7.4.9.2 and 7.4.9.3  as shown

- Peer Link Open frame format

The Peer Link Open frame is used to open a peer link using the procedures defined in 11A.2. The frame body of a Peer Link Open frame contains the information shown in Peer Link Open frame body.

| - Peer Link Open frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action Value |  |
|  | Capability |  |
|  | Supported rates |  |
|  | Extended Supported Rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. |
|  | Power Capability | The Power Capability element shall be present if dot11SpectrumManagementRequired is true. |
|  | Supported Channels | The Supported Channels element shall be present if dot11SpectrumManagementRequired is true. |
|  | RSN | The RSN information element is only present within Peer Link Open frames generated by MPs that have dot11RSNAEnabled set to TRUE. |
|  | QoS Capability | The QoS Capability element is present when dot11QoS-OptionImplemented is true. |
|  | Mesh ID | The Mesh ID information element is present when dot11MeshEnabled is true. |
|  | Mesh Configuration | The Mesh Configuration information element is present when dot11MeshEnabled is true. |
|  | Peer Link Management | The Peer Link Management information element is present only when dot11MeshEnabled is true. The subtype of the Peer Link Management Element is set to 0. |
|  | MSCIE | The MSCIE element is present when dot11MeshEnabled is true. |
|  | MSAIE | The MSAIE element is present when dot11MeshEnabled is true. |
|  | MIC | This field is present when dot11MeshEnabled is true and the abbreviated handshake is enabled |
| 16 | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented attribute is true. |
| Last | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Peer Link Management.

The Action field is set to the value in Error: Reference source not found for this action frame type.

- Peer Link Confirm frame format

The Peer Link Confirm frame is used to confirm a peer link using the procedures defined in 11A.2. The frame body of a Peer Link Open  Confirm frame contains the information shown in Peer Link Confirm frame body.

| - Peer Link Confirm frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action Value |  |
|  | Capability |  |
|  | Status code |  |
|  | AID |  |
|  | Supported rates |  |
|  | Extended Supported Rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. |
|  | RSN | The RSN information element is only present when dot11RSNAEnabled is set to TRUE. |
|  | EDCA Parameter Set |  |
|  | Mesh ID | The Mesh ID information element is present when dot11MeshEnabled is true. |
|  | Mesh Configuration | The Mesh Configuration information element is present when dot11MeshEnabled is true. |
|  | Peer Link Management | The Peer Link Management information element is present only when dot11MeshEnabled is true. The subtype of the Peer Link Management Element is set to 1. |
|  | MSCIE | The MSCIE element is present when dot11MeshEnabled is true. |
|  | MSAIE | The MSAIE element is present when dot11MeshEnabled is true. |
|  | MIC | This field is present when dot11MeshEnabled is true and the abbreviated handshake is enabled |
| 16 | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented attribute is true. |
| 17 | HT Information | The HT Information element is included by an AP when dot11HighThroughputOptionImplemented attribute is true. |
| Last | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Peer Link Management.

The Action field is set to the value in Error: Reference source not found for this action frame type.

- Peer Link Close frame format

The Peer Link Close frame is used to close a peer link using the procedures defined in 11A.2. The frame body of a Peer Link Open Close frame contains the information shown in Error: Reference source not found.