IEEE P802.11
Wireless LANs

| Normative Text Proposal for Jamming Co-Located Interference Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel |  | +1 503 264 7799 | emily.h.qi@intel.com |
| Jing Zhu | Intel | 2111, NE 25th Ave., Hillsboro, OR 97124 | +1 (503) 264 7073 | jing.z.zhu@intel.com |

Abstract

This submission contains the normative text for Co-Located Interference Enhancements for Jamming Co-located Interference Report. It is intended to resolve the comment with CID 96.

The text is aligned with P802.11v-D1.0.1.

7.4.11.15 Co-located Interference Response

- Change the 7th paragraph as follows:

The Interference Level field is a signed integer indicating the level of the expected self- co-located interference in units of dBm averaged over a 4 μs period during an interference period. If no co-located interference is present the field is set to 0. The interference level is referenced to the connector of the currently in-use receiving antenna. The value of 127 indicates that the reporting STA is not able to receive any frame due to co-located jamming interference or other reasons during the reported interference periods, regardless of the actual interference level and frequency. After the absence period ends, the reporting STA in order to transmit shall perform CCA until a frame sequence is detected by which it can correctly set its NAV, or until a period of time equal to the ProbeDelay has transpired.