IEEE P802.11
Wireless LANs

| Normative Text Proposal for Jamming Co-Located Interference Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel |  | +1 503 264 7799 | emily.h.qi@intel.com |
| Jing Zhu | Intel | 2111, NE 25th Ave., Hillsboro, OR 97124 | +1 (503) 264 7073 | jing.z.zhu@intel.com |

Abstract

This submission contains the normative text for Co-Located Interference Enhancements for Jamming Co-located Interference Report. It is intended to partially address the comment with CID 96.

The text is aligned with P802.11v-D1.0.1.

7.4.11.15 Co-located Interference Response

- Change the 7th paragraph as follows:

The Interference Level field is a signed integer indicating the level of the expected self-interference co-located interference in units of dBm averaged over a 4 μs period during an interference period. If no co-located interference is present the field is set to 0. The interference level is referenced to the connector of the currently in-use receiving antenna. The value of 127 indicates that the reporting STA is not able to receive any frame due to co-located jamming interference or other reasons during the reported interference periods, regardless of the actual interference level and frequency.