IEEE P802.11
Wireless LANs

| LB115 20-40 Coex State Maintenance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 USA | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document proposes resolutions for the various comments from the coex 20-40 tab of the TGn LB115 COEX adhoc comment resolution group spreadsheet.

The most significant changes are:

The 20/40 coexistence mechanism contains one minor oversight. The 20 MHz BSS Width Request field of the 20/40 BSS Coexistence Management action frame is set by a STA when the STA detects a trigger condition a). But trigger condition a) is not limiting with respect to the “affected” channels – it covers the entire 2.4 GHz band, while the 20 MHz BSS Width Request field is forcing. This document proposes a solution that allows the AP to determine whether it is required to switch to 20 or not.

Also, the 20/40 coexistence mechanism has some implications for the complexity of AP and STA implementations that can be simplified at the cost of generating a bit of additional OBSS reporting traffic. The rationale is that the extra traffic is minimal given OBSS scanning parameter values. Also, the current text causes some double accounting of detected trigger event information decay.

A third important change is to eliminate a promiscuous receive requirement for STAs.

REVISION NOTES:

R12:

Removed CID 5077 and 5827.

R11:

One more change also in GRAY HIGHLIGHTING.

In the changes for CID 5474 for 11.15.2, the rules on which channel can be used for which type of frame transmission by which type of STA vs AP were slightly ambiguous. These changes clarify the rules.

Updated document references to r11.

R10:

One more change also in GRAY HIGHLIGHTING.

In one of the changes to 11.15.10, the indicated change is incorrect.

Specifically, with respect to the AP setting the secondary channel offset to a value indicating 20/40 mhz BSS operation, the set of restrictions included a restriction that was predicated on the AP maintaining information regarding the last received values for the 20 MHz BSS Width Request from each associated STA. This requirement was necessary with the old behaviour (STAs reporting only when changes occur), but made obsolete with the new description (STAs always reporting at OBSS Scan intervals). By mentioning trigger condition “c” in the previous statement, the deleted statement is effectively covered.

The following changes are the correct changes for the new mechanism, and these now appear later in the document as part of the editing instructions:

no BSS channel width trigger events b) or c) are detected.

R9:

A few more changes also in GRAY HIGHLIGHTING.

See CID 5032 vs CDI 5011 – resolving wording conflicts between these two CIDs – and fixing occurrences of wrong element reference, but no substantive changes from what has been intended previously.

See CID 5080 which simply adds an editor instruction but does not change the technical intent of the proposed resolution or proposed text changes – just making the editor’s job easier.

See CID 5082 which simply adds an editor instruction but does not change the technical intent of the proposed resolution or proposed text changes – just making the editor’s job easier.

See CID 5894 which simply adds an editor instruction but does not change the technical intent of the proposed resolution or proposed text changes – just making the editor’s job easier.

See CID 5112 which simply adds an editor instruction but does not change the technical intent of the proposed resolution or proposed text changes – just making the editor’s job easier.

See CID 5792 which simply adds an editor instruction but does not change the technical intent of the proposed resolution or proposed text changes – just making the editor’s job easier.

See CID 5133 which slightly modifies the resolution because other CIDs resolutions are accounting for what is being requested.

Updated references to the document that occur within resolutions.

R8:

In response to feedback, changed wording slightly in two edit instructions – see changes for CID 5869 and 5717. These changes appear in GRAY HIGHLIGHTING.

Updated references to the document that occur within resolutions.

R7:

Updated references to the document that occur within resolutions.

Modified some of the edits that were introduced in REV 6 due to group review.

Changed remaining yellow comments to green after review by group.

R6:

Reviewed items during January 2008 interim session.

Added text to cover scanning exemption request/grant.

R5:

Reviewed items during during additional TGn adhoc January 2008 Taipei session hours.

Now also using BLUE highlighting for agreed, but need editing instructions.

R4:

Reviewed items during TGn adhoc January 2008 in Taipei.

R3:

Brought in 8 more comments from COEX REORG comment group.

Adding resolutions and editing for almost all CIDs.

R2:

Changes for revision 2 show the results of review during the TGn December 05, 2007 conference call.

Reviewed items are either marked as GREEN, indicating that the participants of the call were in general agreement with the proposed resolution of the comment or marked as RED, indicating that the issue remains open.

Unreviewed items are either UNHIGHLIGHTED or are YELLOW highlighted. UNHIGHLIGHTED means that the author of the document has not yet prepared a proposed response and YELLOW highlighting means that a proposed resolution has been prepared but has not yet been discussed outside of the author’s head.

Note that a GREEN-highlighted comment in this document does NOT denote that the COEX adhoc has accepted the proposed resolution officially – the green simply means that consensus has been reached, but the official position of the COEX adhoc has not yet been polled.

R1:

Adding resolutions and editing for more CIDs, but not yet all.

Changes from R0 shown in yellow highlighting with some controversial comments shown in pink highlighting.

| 5117 | Chan, Douglas |  | General |  | It is essential that there is fair coexistence between HT devices of 20/40 MHz operations and legacy/20 MHz HT operations, for both 2.4 and 5 GHz bands. | Analyze and address any weaknesses that may remain. | Counter – 20/40 coexistence text and procedures modified in several places, mostly with respect to reporting mechanism. Fundamentals of allowance/disallowance of 40 MHz operation are unchanged, since the group consensus is that interoperation of 20 MHz and 20/40 MHz BSSs will allow for a sufficiently adequate user experience for legacy devices and 20 MHz HT devices in the cases of interest and there is always the nuclear option (i.e. the ability of a neighbor to set the 40 MHz Intolerant bit) which prevents 20/40 MHz BSS operation. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5510 | Scarpa, Vincenzo | 69.50 | 7.3.2.52.2 |  | If a station is 40MHz intolerant, it is intolerant to 40MHz transmissions at all, including the ones occurring within the BSS it is associated to. | Use Forty MHz Intolerant subfield to prohibit 40MHz transmissions also in the current BSS the station is associated to. | Reject – no change to the draft is necessary, since the suggested behavior already exists in subclause 11.15.10. If the commenter is suggesting the deletion of the 20 MHz BSS Width request bit, then that suggestion is rejected because of the need to avoid a propagation effect when the STA is communicating a discovered 40 mhz intolerance to its associated AP. |
| 5045 | Cam-Winget, Nancy | 78.01 | 7.3.2.53 |  | The HT IE contains a: * Secondary Channel Offset to note its presence and a * STA Channel Width, which defines whether the STA is capable of receiving in 20 or 20/40MHz But the STA Channel Width field can also be detected by the secondary channel offset as: * Secondary Channel Offset = 0 => STA Channel Width = 0 * Secondary Channel Offset = 1 or 3 => STA Channel Width = 1 | Delete STA Channel Width as it can be deduced from the Secondary Channel offset | Reject – the existence of the second bit allows the option of having a 40 mhz bandwidth for the BSS, while the transmitter of the HT IE prefers operation at 20 mhz. See 11.15.3 for additional information. Some interesting cases include 5 GHz IBSS and 40 Mhz mask PPDU transmissions between DLS participants in a BSS in which the AP is using only 20 mhz. |
| 5889 | Myles, Andrew | 78.01 | 7.3.2.53 |  | The HT IE contains a: * Secondary Channel Offset, which defines the secondary channel as being above the primary channel, below the primary channel or not existing * STA Channel Width, which defines whether the STA is capable of receiving in 20 or 20/40MHz (see pp 210, line 36-40) However, the STA Channel Width field can be deleted by noting that: * Secondary Channel Offset = 0 => STA Channel Width = 0 * Secondary Channel Offset = 1 or 3 => STA Channel Width = 1 | Delete STA Channel Width field from entire document I note that there is a hint that STA Channel Width may only be advisory (see 11.15.3, pp 210 , line 60). In this case, the STA Channel Width might make some sense. However, I would ask, why is it only advisory? | Reject – see CID 5045. |
| 5094 | Chan, Douglas | 82.36 | 7.3.2.54 |  | How are these values, in particular -- the Regulartory Class field, encoded in binary? | Specify if not already. | Accept – editor shall execute changes shown in document 11-07/2742r11 under the heading that includes CID 5094. |
| 5594 | Stephens, Adrian | 82.60 | 7.3.2.54 |  | "A 20/40 BSS Intolerant Channel Report shall only report channels for a single regulatory class. Multiple 20/40 BSS Intolerant Channel Report elements may be used to report channels in more than one regulatory class." This is normative language. | Rephrase to take out normative verbs or move to clause 9. | Counter – accept in principle, changing text to simple declarative statements. Editor to make changes shown in 11-07-2742r11 under the heading that includes CID 5594. |
| 5595 | Stephens, Adrian | 83.03 | 7.3.2.54 |  | "A 20/40 BSS Intolerant Channel Report element shall only include channels that are valid for the regulatory domain in which the STA transmitting the element is operating and that are consistent with the Country element transmitted by the AP of the BSS of which it is a member." This is normative language. | Rephrase a non-normative or move to clause 9. | Counter – accept in principle, changing text to simple declarative statements. Editor to make changes shown in 11-07-2742r11 under the heading that includes CID 5595. |
| 5095 | Chan, Douglas | 83.03 | 7.3.2.55 |  | How are these values encoded in binary? | Specify if not already. | Accept – editor shall execute changes shown in document 11-07/2742r11 under the heading that includes CID 5095. |
| 5485 | Petranovich, James | 85.05 | 7.4 |  | There should be an action frmae to allow a device to change its operating capability from 20 MHz only to 20-40 capable. This will allow a virtually inactive device to operate as a 20 Mhz only device in all respects, but if it becomes active act as a 20-40 device. This coudl result in power savings by eliminating extra OBSS scan requirements as well as reducing prcoessign requirements. This is different from the notify channel width frame, as I propsoe a chaneg in capapbiltiy.. | Create a new action frmae to signal this switch. Create rules in the MAC section to allow devices to change capability without re-associating. If there is fear of abuse, require that these devices implement a scan (if requeired) of OBSSs and submit a report to the AP before switching, or even require a delay of several seconds from the time the capability swtich from 20 to 20-40 is made until the notify channel width message is sent. (Until then the device woudl operate as a 20-40 MHz device but it would only accept 20 MHz BW frames.) | Reject - While the idea of allowing a STA to painlessly switch to 20MHz capability and drop the requirement of scanning is initially attractive, deeper analysis shows it to be flawed. The main issue is that, if the AP is required to switch its BSS to 20MHz operation, there is nothing to stop all its STAs indicating they are now 20MHz-only capable. This brings a the slight benefit of reduced power consumption and increased performance. But in this state, the AP is now no longer able to observe reports from any STA, and has no valid basis ever to leave 20MHz BSS operation. If it wrongly infers it is safe to leave 20MHz operation when it is not in fact safe to do, it will re-enable 20/40 operation, its STAs will respond by re-enabling their 40MHz capability. They will then scan and probably discover the original cause of the switch, which is reported to the AP. The AP will then switch to 20MHz operation. The result is potential thrashing between 20 and 20/40 operation, resulting in additional management frame overhead, and increased interference to the BSS that was the cause of the original switch. |

| 5424 | Marshall, Bill | 86.55 | 7.4.7a.1 |  | Value zero is a very bad one to use for a legitimate message. Often software bugs cause junk messages with unfilled fields, which more often than not contain zeroes. | Leave value 0 as reserved (as is in TGy); allocate an action field value at the end of the list. | Reject – A very large number of existing fields in many different frames use zero value codings for valid messages. Fields which are a single bit in width natively require that meaning is applied to the zero value. No interesting problems have been identified related to this question over the 15+ year history of 802.11. And while it may be the experience of the commenter that software bugs more often than not produce zeros when they go haywire, neither such an unpredictable (i.e. probable, but not certain) behavior nor in general, any anecdotal experience provides sufficient justification for making such a change. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5711 | Stephens, Adrian | 205.12 | 11.9.8.1 |  | "A BSS in which supported channel width of the AP is 20 MHz (Channel Width field is set to 0) and the Secondary Channel Offset field is set to 0." So what kind of BSS is it in which the supported channel width field is 1 and the secondary channel offset is 0? Answer, we don't know because this case, while valid, is excluded from both definitions. | Change to "A BSS in which the Secondary Channel Offset field is set to 0." for 20MHz BSS. | Accept – editor to make changes shown in 11-07-2742r11 under the heading that includes CID 5711. |
| 5029 | Adachi, Tomoko | 205.16 | 11.9.8.1 |  | "20/40 MHz BSS: A BSS in which … the Secondary Channel Offset field is set to a non-zero value." If the Secondary Channel Offset is 2, 4 or larger, it is not the 20/40 MHz BSS that we are thinking of any more… | Change it to "20/40 MHz BSS: A BSS in which … the Secondary Channel Offset field is set to 1 or 3." | Accept – editor to make changes shown in 11-07-2742r11 under the heading that includes CID 5029. |
| 5849 | Yamaura, Tomoya | 205.35 | 11.9.8.2 |  | "An AP or IDO STA operating a 20/40MHz BSS, on detecting an overlapping BSS whose primary channel is the AP’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz BSS operation." Does IDO STA operate BSS, even though the definition of IDO STA is "the DFS owner of an IBSS" ? And may IDO STA switch to 20MHz BSS operation ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with "An AP operating a 20/40MHz BSS, on detecting an overlapping BSS whose primary channel is the AP’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz BSS operation. An IDO STA operating a 20/40MHz IBSS ,on detecting an overlapping BSS whose primary channel is the IDO STA’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz IBSS operation.". And if you think the definitions of 20/40MHz IBSS and 20MHz IBSS are required, add definition in 11.9.8.1. | Counter – while the commenter misses the point that an IBSS is a BSS by definition, it is true that other parts of the cited text could be improved, and therefore, the suggested replacement is accepted in principle. Editor shall make the changes shown under the heading that includes CID 5849 within 11-07-2742r11. Note that definitions specific to IBSS are not needed. |
| 5850 | Yamaura, Tomoya | 205.35 | 11.9.8.2 |  | "An AP or IDO STA operating a 20/40MHz BSS, on detecting an overlapping BSS whose primary channel is the AP’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz BSS operation." Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace "overlapping BSS"s in the quoted sentence with "overlapping BSS (or IBSS)". | Reject –it really is not necessary, since the definition of BSS includes IBSS. |
| 5447 | Miller, Robert | 205.39 | 11.9.8.3 |  | For 2.4GHz operations, there is no rule for starting a 20/40 BSS if there is any 22MHz (Clause 18) BSS detected (either via ED or CCK carrier). | Add rules for 20/40 BSS to avoid any 22MHz BSS if possible. If cannot be avoided, start only 20MHz BSS with center freq aligned with the center freq of the 22MHz BSS so ED CCA can do its job. | Reject – this condition is already covered – see equation 11-4 within 11.9.8.3 that covers a broad range of frequencies that is used to determine whether an overlap exists. Note that the condition of identifying an overlapping BSS is based on detection of beacons, among other methods, which does not depend on the type of beacon. |
| 5375 | Ji, Lusheng | 205.39 | 11.9.8.3 |  | For 2.4GHz operations, there is no rule for starting a 20/40 BSS if there is any 22MHz (Clause 18) BSS detected (either via ED or CCK carrier). | Add rules for 20/40 BSS to avoid any 22MHz BSS if possible. If cannot be avoided, start only 20MHz BSS with center freq aligned with the center freq of the 22MHz BSS so ED CCA can do its job. | Reject – see CID 5447. |

| 5851 | Yamaura, Tomoya | 205.42 | 11.9.8.3 |  | "Before an AP or IDO STA starts a 20/40 MHz BSS it shall perform a minimum of dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS Scans (see 11.15.4 (Scanning requirements for 40 MHz capable STA)) to search for existing BSSs." Does IDO STA start a BSS, even though the definition of IDO STA is "the DFS owner of an IBSS" ? | Replace quoted sentence with "Before an AP or IDO STA starts a 20/40 MHz BSS or IBSS it shall perform a minimum of Dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS Scans (see 11.15.4 (Scanning requirements for 40 MHz capable STA)) to search for existing BSSs." | Reject - The commenter has raised an interesting point, but not addressed it with the suggested change. Commenter should note that an IBSS is a BSS. With respect to the DFS owner portion of the comment, 11.9.7.2 specifically requires that in a domain that requires DFS owner rules, any STA that starts an IBSS shall be a DFS owner. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5852 | Yamaura, Tomoya | 205.42 | 11.9.8.3 |  | "Before an AP or IDO STA starts a 20/40 MHz BSS it shall perform a minimum of dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS Scans (see 11.15.4 (Scanning requirements for 40 MHz capable STA)) to search for existing BSSs." Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace "Overlapping BSS scan" with Overlapping BSS and/or IBSS scan", and replace "existing BSSs" with "existing BSSs and/or IBSSs." | Reject - Commenter should note that an IBSS is a BSS. |
| 5854 | Yamaura, Tomoya | 205.47 | 11.9.8.3 |  | "If the AP or IDO STA chooses to start a 20/40 MHz BSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS, then the AP or IDO STA shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSS, unless the AP discovers that on these two channels are existing 20/40 MHz BSSs with different primary and secondary channels." Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace four "existing 20/40 MHz BSS"s with "existing 20/40 MHz BSS and/or IBSS"s. | Reject - Commenter should note that an IBSS is a BSS. |
| 5180 | Ecclesine, Peter | 205.47 | 11.9.8.3 |  | Description should be rewritten to be 'not in 2.4 GHz ' and 'in 2.4 GHz' to allow application to other non-2.4 GHz bands. | In 11.9.8.3 change all occurrances of 'BSS in 5 GHz' to 'BSS not in 2.4 GHz'. | Reject – the group notes that there are no 40 mhz channels defined for TGy and that when any other band gets 40 mhz channels, the TG that makes that move will bear the responsibility for making all of the changes necessary to allow 20/40 coex procedures to operate in those new channels – note that the group came to this conclusion after realizing that the necessary changes to the text were more involved than just replacing 5GHz with “not 2.4 GHz” in 11.9.8.3. (E.g. STA 17 definition needs to change and uses of that term would be changed as well – it gets complicated quickly.) |
| 5853 | Yamaura, Tomoya | 205.47 | 11.9.8.3 |  | "If the AP or IDO STA chooses to start a 20/40 MHz BSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS, then the AP or IDO STA shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSS, unless the AP discovers that on these two channels are existing 20/40 MHz BSSs with different primary and secondary channels." Does IDO STA start a BSS, even though the definition of IDO STA is "the DFS owner of an IBSS" ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with ""If the AP or IDO STA chooses to start a 20/40 MHz BSS or IBSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS, then the AP or IDO STA shall ensure that the primary channel of the new BSS or IBSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS or IBSS is identical to the secondary channel of the existing 20/40 MHz BSS, unless the AP discovers that on these two channels are existing 20/40 MHz BSSs with different primary and secondary channels." | Reject – see CID 5851. |
| 5855 | Yamaura, Tomoya | 205.59 | 11.9.8.3 |  | "If an AP or IDO STA chooses to start a 20/40 MHz BSS in 5 GHz on a pair of channels, the selected secondary channel should correspond to a channel on which no beacons are detected during the dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS scan time performed by the AP or IDO STA , unless there are beacons detected on both channels." Does IDO STA start a BSS, even though the definition of IDO STA is "the DFS owner of an IBSS" ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with "If an AP or IDO STA chooses to start a 20/40 MHz BSS or IBSS in 5 GHz on a pair of channels, the selected secondary channel should correspond to a channel on which no beacons are detected during the dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS scan time performed by the AP or IDO STA , unless there are beacons detected on both channels." | Reject – see CID 5851. |
| 5856 | Yamaura, Tomoya | 205.62 | 11.9.8.3 |  | Here is "Overlapping BSS". Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace "Overlapping BSS" with Overlapping BSS and/or IBSS". | Reject – BSS definition includes IBSS |
| 5858 | Yamaura, Tomoya | 206.01 | 11.9.8.3 |  | "An HT AP or IDO STA should not start a 20 MHz BSS in 5 GHz that is a secondary channel of any 20/40 MHz BSS." Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace "any 20/40 MHz BSS" in the quoted sentence with "any 20/40 MHZ BSS and/or IBSS". | Reject – BSS definition includes IBSS |
| 5857 | Yamaura, Tomoya | 206.01 | 11.9.8.3 |  | "An HT AP or IDO STA should not start a 20 MHz BSS in 5 GHz that is a secondary channel of any 20/40 MHz BSS." Does IDO STA start a BSS, even though the definition of IDO STA is "the DFS owner of an IBSS" ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with "An HT AP or IDO STA should not start a 20 MHz BSS or IBSS in 5 GHz that is a secondary channel of any 20/40 MHz BSS." | Counter – editor shall make changes shown under any heading that includes reference to CID 5857 within submission 11-07-2742r11, |
| 5859 | Yamaura, Tomoya | 206.05 | 11.9.8.3 |  | "If the AP chooses to start a 20/40 MHz BSS in 2.4 GHz, then the AP shall ensure that the primary channel of the new BSS is identical to the primary channel of any existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of any existing 20/40 MHz BSS." Is there any rule to selecting Primary/secondary in 20/40MHz IBSS at 2.4GHz ? | Add similar rule to choose Primary/secondary at 2.4GHz 20/40MHz IBSS, such as 5GHz. Or, explicitly prohibit to use 40MHz at 2.4GHz for IBSS. | Reject – no change needed because 20/40 MHz IBSS in 2.4 GHz is already expressly prohibited. See D3.0 11.15.3 “An HT STA 19 that is not a member of an infrastructure BSS shall not transmit a 40 MHz Mask PPDU.” And see D3.0 11.15.1 “An HT STA 19 that is a member of an IBSS shall not set the Secondary Channel Offset field of the 20/40 BSS Coexistence element to a non-zero value in transmitted frames.” |
| 5030 | Adachi, Tomoko | 206.05 | 11.9.8.3 |  | How about when there are multiple 20/40 MHz BSSs on the two channels with different primary and secondary channels? | Change "any" appearing twice in this paragarph to "the". Also add a description what to do when there are multiple 20/40 MHz BSSs discovered on the two channels with different primary and secondary channels. | Counter – editor shall make changes shown under any heading that contains CID 5030 in 11-07-2742r11 – noting that within these changes the sense of the phrases that include “existing 20/40 MHz BSS” for the uncited 5 GHz language change from singular to plural in the second paragraph of subclause 11.9.8.3. Also, the information expressed in the sentence being deleted from the cited language covering 2.4GHz operation and the information requested regarding the presence of multiple BSSs is found in the correctly described use of the operation permitted variable appearing here and in 11.15. |
| 5860 | Yamaura, Tomoya | 206.12 | 11.9.8.3 |  | "The AP or IDO STA may continue to periodically scan after the BSS has been started." Does IDO STA start a BSS, even though the definition of IDO STA is "the DFS owner of an IBSS" ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with "The AP or IDO STA may continue to periodically scan after the BSS or IBSS has been started." | Reject – see CID 5851. |
| 5861 | Yamaura, Tomoya | 206.12 | 11.9.8.3 |  | "The AP or IDO STA may continue to periodically scan after the BSS has been started." I assume this rule is for 5GHz, because it refers IDO STA, and DFS is defined only at 5GHz in 11h (though, it is not referred in this amendment). | If my interpretation is correct, move quoted sentence to line-3 of page-206. If my understanding is not correct and IDO (DFS) is used in 2.4GHz, please add definition of DFS at different way from 11h, and rewrite the definition of IDO STA at page-205. | Reject – the definition of IDO STA includes a reference to 5 GHz – note that this is fine because 20/40 MHz BSS is not allowed for IBSS in 2.4 GHz – see CID 5851. Not sure why the commenter wants the text moved – it seems to flow with the development of the behavior as described in the surrounding paragraphs regarding the establishment of a BSS.. |
| 5862 | Yamaura, Tomoya | 206.14 | 11.9.8.3 |  | "overlapping BSS" Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | replace "overlapping BSS" with "overlapping BSS and/or IBSS". | Reject – BSS definition includes IBSS |
| 5031 | Adachi, Tomoko | 206.15 | 11.9.8.3 |  | It says, after BSS startup but before making transition from a 20 MHz BSS to a 20/40 MHz BSS, the AP shall perform overlapping BSS Scans. It is not clear what is required after the AP gets the scan result. If nothing is required, then we don't need this paragraph. It could be read that "the AP" applies to both APs operating in 2.4 GHz and 5 GHz, but is it correct? | Clarify. | Counter - clarification made. Editor shall make changes shown in 11-07-2742r11 under the heading that includes CID 5031. Action required on the part of the AP after scanning is implicit in the existing rules describing behavior in response to received BSS width trigger events. If scanning yields such events, then appropriate behavior must follow. 2.4 GHz vs 5 GHz issue resolved with changes made to text. |
| 5808 | Trainin, Solomon | 206.15 | 11.9.8.3 |  | "After BSS startup, the AP shall perform a minimum of dot11BSSWidthChannelTransitionDelayFactor overlapping BSS scans either itself or through its associated STAs, of the set of channels that are allowed operating channels within the current operational regulatory domain and whose center frequency falls within the 40 MHz affected channel range given by Equation (11-4) before making a transition from a 20 MHz BSS to a 20/40 MHz BSS. If no secondary channel has been selected, then all channels in the frequency band shall be scanned." Inherently this definition assumes transition from 20 MHz BSS to a 20/40 MHz BSS w/o disassociation. This tran sition changes supported channel width capability and there is no convinient way how to change capabilty in the establshed BSS. So this case is realted to creating new BSS after disassiciation of the older one. | Make it clear that this transition is not about capability changing by using any different terminology instead of 20 MHz BSS to a 20/40 MHz BSS or remove this rule | Counter – see CID 5711, which has made changes to the definition of a 20 MHz BSS and also editor shall make the changes shown under any heading that includes CID 5808 which include adding a note in 11.9.8.3 which describes the specific case where it is possible to make the transition while maintaining associations by changing the secondary channel offset without changing the capability. |
| 5078 | Chan, Douglas | 206.30 | 11.9.8.3 |  | The logical expression is illdefined. The meaning of the "=" in the expression is intended to be a question asking whether they're equal, not to assign the value of the right side to the left side of "="; in other words the "==" operator is intended. | Change "=" to "==" in the brackets and define the meaning of the operator "==", that it means to ask whether the two sides are equal, and if so, assign the value "TRUE", else "FALSE" to the expression. | Counter – accept in principle. Editor to make changes shown in 11-07-2742r11 under any heading that includes CID 5078. |
| 5079 | Chan, Douglas | 206.30 | 11.9.8.3 |  | The logical expression is illdefined. Each expression for for OP_i, OT_i and OS_i works only if each respective i >= 1; otherwise, there's no need to evaluate it. | Rewrite the expressions so that they're evaluated only if i >= 1 for OP_i, OT_i and OS_i, resepectively. | Counter – accept in principle. Editor to make changes shown in 11-07-2742r11 under any heading that includes CID 5079. |
| 5863 | Yamaura, Tomoya | 206.40 | 11.9.8.3 |  | "20/40 MHz BSS" Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | replace "20/40 MHz BSS" with "20/40 MHz BSS or IBSS". | Reject – BSS definition includes IBSS |
| 5864 | Yamaura, Tomoya | 206.45 | 11.9.8.3 |  | "20/40 MHz BSS" Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | replace "20/40 MHz BSS" with "20/40 MHz BSS or IBSS". | Reject – BSS definition includes IBSS |
| 5865 | Yamaura, Tomoya | 206.50 | 11.9.8.3 |  | "20 MHz BSS" Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | replace "20 MHz BSS" with "20 MHz BSS or IBSS". | Reject – BSS definition includes IBSS |
| 5080 | Chan, Douglas | 206.62 | 11.9.8.3 |  | We are subtracting 25 MHz not 25 Hz. | Add the units of MHz after "25"; or change "25" to "25e6". | Counter – editor shall add MHz after the cited occurrences of 25. Editor to make changes shown in 11-07-2742r11 under any heading that includes CID 5080. |
| 5032 | Adachi, Tomoko | 206.65 | 11.9.8.3 |  | "Secondary Channel Offset field set to a non-zero value". Do you want to include the cases when the value is 2,4 or larger? | Change it to "Secondary Channel Offset field set to 1 or 3". Search for other places and make the same change. | Counter – accept in principle. Editor to replace the non-zero instance cited as desired by the commenter, except employing mnemonics for the values 1 and 3, with the mnemonics being chosen by the editor and make a global replacement of all explicit values for this field where such values are used with reference to the Secondary Channel offset field throughout the TGn draft and define the mnemonics for all values of this field within the definition of the Secondary Channel offset field in subclause 7 and add a definition for each of the mnemonics within clause 4. Note also that the equivalent statement is left in the edited locations by inverting the verb and using the opposite values for the fields. (i.e. 0 vs 1 and 3 before the mnemonic substitutions) |
| 5319 | Fischer, Matthew | 207.04 | 11.9.8.3 |  | The paragraph that contains the text "Report information from the most recently received 20/40 BSS Coexistence Management frames from each STA" needs to be more explicit about how this information is used. | Indicate that the information from the channel list fields from the 20/40 BSS Coexistence Report elements is used as "20 MHz BSS operational channels" within the OT set calculation. | Counter – accept in principle. Editor to make changes shown in 11-07-2742r11 under any heading that includes CID 5319. |
| 5716 | Stephens, Adrian | 207.09 | 11.9.8.3 |  | "If, after initial establishment of the 20/40 MHz BSS, the value of 20/40 Operation Permitted is FALSE, then the FC HT AP 19 shall immediately revert to 20 MHz BSS operation or the FC HT AP 19 shall immediately move the BSS channel(s) to a channel (or pair of channels) where 20/40 Operation Permitted evaluates to TRUE." What does "immediately" mean? This needs unpacking as this is a normative requirement. | Rewrite to say that either: 1. The AP shall not accpect any associations until this transition has completed, or 2. The AP shall not send any beacons and shall not response to probe requests until this transition has completed. | Counter – the only required action is to change to 20 mhz BSS operation as described by the changes to the text of 11.9.8.3 and 11.15.1. Editor to make changes shown in 11-07-2742r11 under any heading that includes CID 5716. |
| 5809 | Trainin, Solomon | 207.10 | 11.9.8.3 |  | "If, after initial establishment of the 20/40 MHz BSS, the value of 20/40 Operation Permitted is FALSE, then the FC HT AP 19 shall immediately revert to 20 MHz BSS operation or the FC HT AP 19 shall immediately move the BSS channel(s) to a channel (or pair of channels) where 20/40 Operation Permitted evaluates to TRUE." It is not completely clear what is immediately about. It is not the same if the decision is made at start of BSS or during active BSS. In the first case the move has been yet to start beaconing and in the second as defined in 11.15 | Make the definition clear for both cases OR remove this paragraph at all as not realted to the Scanning requirements | Counter – see CID 5716. |
| 5081 | Chan, Douglas | 207.12 | 11.9.8.3 |  | If the FC HT AP 19 is moving its 40 MHz channels, then how can it be just one channel? | Remove "()" from (s) and "(or a pair of channels)" and remove "a channel". | Counter – accept in principle. Editor to make changes shown in 11-07-2742r11 under any heading that includes CID 5081. |
| 5866 | Yamaura, Tomoya | 207.18 | 11.9.8.4 |  | "While operating a 20/40 MHz BSS, a STA that is the DFS owner in an IBSS or an AP may decide to move its BSS and/or switch the BSS to 20 MHz operation if," Does a STA that is the DFS owner of IBSS operate 20/40MHz BSS ? Also, we already defined IDO STA, and is there any reason not to use it ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted part with ""An AP while operating a 20/40MHz BSS or a STA while operating 20/40MHz IBSS may decide to move its BSS or IBSS and/or switch the BSS or IBSS to 20 MHz operation if," | Reject – STA is not appropriate, because it is too broad – only DFS Owners and APs may perform the functions described here. Also, BSS includes IBSS. |
| 5867 | Yamaura, Tomoya | 207.22 | 11.9.8.4 |  | "Specifically, the AP or STA may move its BSS to a different pair of channels, or change from a 20/40 MHz BSS to a 20 MHz BSS using either the primary channel of the previous channel pair or any other available 20 MHz channel." Why doesn't it mention IBSS ? And STA should not move the BSS channel. I guess this "STA" is IDO STA. D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with "Specifically, the AP or STA in an IBSS may move its BSS or IBSS to a different pair of channels, or change from a 20/40 MHz BSS or IBSS to a 20 MHz BSS or IBSS using either the primary channel of the previous channel pair or any other available 20 MHz channel." | Reject – BSS definition includes IBSS. Within a paragraph, the qualifiers for the first instance of AP or STA are not needed when “the” is the article in the subsequent references, since “the” definitively refers to the previously mentioned AP or STA, qualifiers included. |
| 5868 | Yamaura, Tomoya | 207.24 | 11.9.8.4 |  | "While operating a 20 MHz BSS, a STA that is the DFS owner in an IBSS or an AP may decide to move its BSS and/or switch the BSS to a 20/40 MHz BSS." Does a STA that is the DFS owner of IBSS operate 20/40MHz BSS ? Also, we already defined IDO STA, and is there any reason not to use it ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted sentence with ""An AP while operating a 20 MHz BSS or a STA while operating in 20 MHz IBSS may decide to move its BSS or IBSS and/or switch the BSS or IBSS to a 20/40 MHz BSS or IBSS." | Counter - see changes introduced by the resolution of CID 5033 which changes the phrase to IDO STA and note that yes, an IBSS can be a 20/40 MHz BSS as per the rules found in D3.0 and note that the BSS definition includes IBSS. |
| 5869 | Yamaura, Tomoya | 207.29 | 11.9.8.4 |  | "An AP or IDO STA switches between 20/40 MHz BSS and 20 MHz BSS" Does IDO STA switches 20/40 MHz BSS and 20 MHz BSS ? D2.0 doesn't mention IDO STA and this new comment related to the text appeared in D3.0. | Replace quoted part with "An AP or in an IBSS switches between 20/40 MHz BSS and 20 MHz BSS or between 20/40 MHz IBSS and 20MHz IBSS". | Counter – it is the IDO STA that performs the operation for an IBSS – any STA in the IBSS is not sufficient, but the group has decided that IBSS shall be forbidden from making channel width changes. editor to make changes shown under any heading including CID 5869 found within 11-07-2742r11 |
| 5870 | Yamaura, Tomoya | 207.35 | 11.9.8.4 |  | This paragraph only mentioned the behavior in a BSS. What is the rule for IBSS ? | Add similar rule for a STA operating in an IBSS. | Accept – modified language to include the DFS owner for an IBSS – editor to make changes shown under any heading including CID 5870 found within 11-07-2742r11. |
| 5871 | Yamaura, Tomoya | 207.48 | 11.9.8.4 |  | "the AP or STA moving the BSS or changing its channel width" Does a STA can move BSS ? | Replace quoted part with "the AP moving or changing its channel width or the STA changing its channel width ". | Reject – a STA in an IBSS acting as DFS owner can move the BSS. |
| 5320 | Fischer, Matthew | 207.58 | 11.9.8.4 |  | Secondary Channel Offset element is missing from the list. | Add Secondary Channel Offset element to the list and indicate how the fields are filled in, using the same description as that found already in the subclause describing the filling of the Secondary Channel Offset field of the HT Information element. | Accept – editor to make specific changes under the headings that include CID 5320 in doc 11-07-2742r11. |
| 5717 | Stephens, Adrian | 207.60 | 11.9.8.4 |  | "The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to 1 if the Behavior Limit parameter of the selected row contains the value 13. The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to 3 if the Behavior Limit parameter of the selected row contains the value 14." This implies that any change of Secondary Channel Offset (i.e. switching BSS channel width) must be accompanied by a change of regulatory class. I believe this is unnecessary, and the time taken to notify STA of a change of the regulatory class is not consistent with the need to change channel widths "immediately" when something inconsistent with 40MHz operation is detected. | Replace cited text with: "The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to either 0 or 1 if the Behavior Limit parameter of the selected row contains the value 13. The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to either 0 or 3 if the Behavior Limit parameter of the selected row contains the value 14." | Counter – editor to make specific changes under the headings that include CID 5717 in doc 11-07-2742r11, also see CID 5810, 5721 which allow 20 MHz BSS operation within a reg class that allows 20/40 MHz BSS operation, but only for APs that are FC. |
| 5810 | Trainin, Solomon | 207.62 | 11.9.8.4 |  | "The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to 3 if the Behavior Limit parameter of the selected row contains the value 14."It is true only when the decision is to establish 40MHz channel width otherwise it is not true | Fix the sentence to refelct that the 20MHz channel width may be established in the channel with regulatory class 13 and 14 as well | Counter – see CID 5717 and 5721. |
| 5082 | Chan, Douglas | 208.31 | 11.9.8.4 |  | Why are we distinguishing only FC HT AP 19? This also applies to FC HT AP 17 too. | Remove "19" from all "FC HT AP 19" in this paragraph. | Counter - delete the last paragraph of 11.9.8.4 of TGn draft D3.02. The only use for this information as determined by the group is to determine if a 40 MHz PPDU transmissionis allowed, and this information is covered in the set of rules found in 11.15.x 40 MHz PPDU transmission restrictions. Editor shall make changes shown under any heading that includes CID 5082 in document 11-07-2742r11 |

| 5083 | Chan, Douglas | 208.40 | 11.9.8.4 |  | Whose STA Channel Width record? Are we keeping a record for each of the associated STAs? | Clarify. | Counter – see CID 5082, which deletes the paragraph. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5011 | Adachi, Tomoko | 208.50 | 11.15.1 |  | For the Supported Channel Width Set field, isn't it 1 when it is a non-zero value? Or is there any other value that it can take? | Change "an HT AP that included a non-zero value in the Supported Channel Width Set field …" to "an HT AP that set the Supported Channel Width Set field to 1 …" Make similar change to the next definition for FC HT STA. | Counter – slightly different wording chosen. Editor shall make changes shown under any heading that includes CID 5011 in document 11-07-2742r11. |
| 5182 | Ecclesine, Peter | 209.01 | 11.15.1 |  | 11k adds Japan channel 14, which has a 2.414 GHz Channel Starting Frequency, and is part of 11n baseline. | Change text to include Japan channel 14 Channel Starting Frequency. | Accept – Editor shall make changes shown under any heading that includes CID 5182 in document 11-07-2742r11 |
| 5084 | Chan, Douglas | 209.17 | 11.15.1 |  | Shouldn't the 20/40 BSS Coexistence support field be set according to the corresponding MIB variable? | Check and correct. | Reject – there is no corresponding MIB variable. By definition, an FC HT STA is a STA that has set the Supported Channel Width field to 1 in HT Capabilities Elements that it transmits and such STA are required in the cited text, to also signal support for 20/40 coex mgmt action frames. |
| 5085 | Chan, Douglas | 209.21 | 11.15.1 |  | There are no Secondary Channel Offset field in the 20/40 BSS Coexistence element, do you mean the Forty MHz Intolerent or 20 MHz BSS Width Request field? | Check and correct. | Counter – the correct reference is to the HT Information element and Secondary Channel Offset element – Editor shall make changes shown under any heading that includes CID 5085 in document 11-07-2742r11 |
| 5718 | Stephens, Adrian | 209.48 | 11.15.2 |  | "An FC HT STA, non-FC HT STA or non-HT STA may be associated in a 20/40 MHz BSS." It is unclear whether this is a normative statement for the non-AP STA (i.e. may attempt an association) or for an AP (i.e., may accept an association), or whether this is intended to be informative. | Turn into an informative NOTE, perhaps as follows: "NOTE--A 20/40 MHz BSS can include any mixture of FC HT STAs, non-FC HT STAs and non-HT STAs." | Accept – editor shall make changes shown in 11-07-2742r11 under any heading that includes reference to CID 5718. |
| 5719 | Stephens, Adrian | 209.51 | 11.15.2 |  | "A non-AP HT STA may switch between FC HT STA and non-FC STA operation by disassociation and association or reassociation." There's nothing to stop it doing already - i.e. this sentence is redundant. | Either remove or turn into an informative NOTE | Accept – editor shall make changes shown in 11-07-2742r11 under any heading that includes reference to CID 5719. |
| 5474 | Perahia, Eldad | 209.55 | 11.15.2 |  | Does "An FC HT STA shall use the 20 MHz primary channel to transmit and receive 20 MHz HT PPDUs ." conflict with "Independently of the values of the five fields, a non-AP STA that is a member of a 20/40 MHz BSS shall not transmit non-Control type 20 MHz PPDUs in the secondary channel." in 11.15.3? | resolve conflict. I think this sentence needs to be modified to non-Control type like in 11.15.3 | Counter – accept in principle, but note that there is also a duplicate statement regarding 40 mhz mask ppdu transmission in subclause 11.15.2 that repeats information found in 11.15.3, so in addition to the requested change, the redundant sentence is deleted. Editor shall make changes shown under all headings that include reference to CID 5474 within 11-07-2742r11. |
| 5086 | Chan, Douglas | 209.62 | 11.15.2 |  | Why is this paragraph on protection requirements here? It's out of place. | Move to appropriate clause if it doesn’t exist there already. | Counter – see CID 5720. |
| 5720 | Stephens, Adrian | 209.62 | 11.15.2 |  | "The HT AP indicates protection requirements in the BSS through the HT Protection field and Non-greenfield HT STAs Present field of the HT Information element as well as the Use_Protection field of the ERP Information element. Protection requirements for different settings of these fields are defined in 9.13" While this is true, it adds no value and is out of place in this subclause. | Remove cited text. | Counter – The cited text is not wholly out of context – since the subclause is discussing the possible mixed nature of a 20/40 MHz BSS. Have put one referential sentence in the paragraph that discusses the idea that a mixed network is possible. Editor shall make changes shown under all headings that include reference to CID 5720 within 11-07-2742r11 |
| 5087 | Chan, Douglas | 210.01 | 11.15.3 |  | It appears that the Notify Action Frame is rendered to be an optional item in its function for determining the status of tx and rx of 40 MHz PPDUs. If so, why not make it mandatory or completely remove it? | Make the function of Notify Action Frame in determining the status of tx and rx of 40 MHz PPDUs to mandatory, or completely remove it. | Reject – see CID 5890 |
| 5890 | Myles, Andrew | 210.01 | 11.15.3 |  | The text on (pp210, line 60 to pp 2111, line 2) in this clause seems to imply that STA Channel Width is only advisory, in that a STA is not required to respect it based on the use of "should" on line 60. However, pp 210, lines 36-37 clearly states that STA Channel Width is a capability, and it seems very odd that one would ignore a capability. ie why would one send a 40MHz packet when the receiver was not capable of receiving it? | Resolve the contradiction | Reject. The STA Channel Width field is not a capability, but a dynamic notification carried in the NotifyChannelWidth (for a non-AP STA) and HT Information Element (for an AP). The purpose of the “should” is to simply resolve any race between receiving the Notify Channel Width (which is generally handled in the upper reaches of the MAC) and adjusting transmit parameters for a destination. (I.e. non-empty TX queue may have old TX parameters.) |
| 5893 | Myles, Andrew | 210.01 | 11.15.3 |  | The text in this clause is far too complex with ill defined and contradictory (see other comments) inputs | I suggest that the conditional clauses be rewritten as a table. I can't provide the table at this time because the intent of those writing the original clause is not clear. | Reject – no other commenter has suggested that the text is so confusing as to require a rewrite and the commenter has not provided specific instances of issues to be resolved, other than CID 5890, to which a response has been written to provide insight as to the perceived, but not actual contradiction. |
| 5721 | Stephens, Adrian | 210.31 | 11.15.3 |  | "The Secondary Channel Offset field and/or the Extended Channel Switch Announcement element are used to indicate whether or not the BSS is occupying a 40 MHz wide pair of channels," See my related earlier comment. I would like to be able to see a BSS perform a width transition without requiring that it changes regulatory class - as the expectation of notification for a change of regulatory class is more heavyweight than a change of width (i.e. you'd have to delay for at least a DTIM). I'm wondering if we can allow a 20MHz BSS to exist in a 40Mhz regulatory class when the AP is 20/40 capable. This is implied by 214.10, "An FC HT AP 19 that detects either of the BSS width trigger events b) or c) shall set the Secondary Channel Offset field to 0 and shall set the STA Channel Width field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted to indicate that no secondary channel is present (i.e., that the BSS operating width is 20 MHz)." which should otherwise also require transmission of an Extended Channel Switch announcement to carry an updated regulatory class. | Change to: "The Secondary Channel Offset field is used to indicate whether or not the BSS is currently occupying a 40 MHz wide pair of channels, and when a secondary channel exists, whether it is above or below the primary channel in frequency. The Extended Channel Switch element can be used to indicate a transition from 40MHz to 20MHz operation, and when a secondary channel exists, whether it is above or below the primary channel in frequency." | Counter – accept in principle – the edited text is modified somewhat from the text proposed by the commenter. Editor shall make changes shown under all headings that include reference to CID 5721 within 11-07-2742r11 See also CID 5717, 5810 |
| 5892 | Myles, Andrew | 210.36 | 11.15.3 |  | The text in Table 7-43m states that the STA Channel Width field "defines the channel widths that may be used to transmit to the STA". Lines 60-65 imply that this "may" is only advisory. However, lines 36-37 clearly state that this STA Channel Width is a capability that is more than advisory | Resolve the contradiction | Accept – use of word able was confusing – reworded to eliminate this. Editor shall make changes shown under all headings that include reference to CID 5892 within 11-07-2742r11 |

| 5722 | Stephens, Adrian | 210.54 | 11.15.3 |  | "c) The most recently received Extended Channel Switch Announcement element transmitted by the AP did not contain a value of 0 in the Channel Switch Count field while also containing a value in the New Regulatory Class field that indicates a regulatory class that does not correspond to a Channel Spacing value of 40 MHz." "The most recently received" doesn't define what to do if no such frame has been received. Ditto comment at 211.12 | Add, "or no such frame has been received since association with the AP". And add 211.15 add: "or no such frame has been transmitted since assocation of that STA." | Counter – Because of the difficulties in getting an expression that accounts for all possible signaling sequences, a shorter version of the intent is used. Editor shall make changes shown under all headings that include reference to CID 5722 within 11-07-2742r11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5088 | Chan, Douglas | 210.54 | 11.15.3 |  | The ECSA may not broadcasted continuously after the switch count is zero. In case the last one with the Channel Switch Count = 0 is missed, then the conditions here would not function as intended. So we should phrase this according to a count on the remaining TBTTs to occur before the channel switch is to occur, based on the most recently received ECSA. | Phrase this condition (c) according to a count on the remaining TBTTs to occur before the channel switch is to occur, based on the most recently received ECSA. | Counter – Because of the difficulties in getting an expression that accounts for all possible signaling sequences, a shorter version of the intent is used. Editor shall make changes shown under all headings that include reference to CID 5088 within 11-07-2742r11 |

| 5089 | Chan, Douglas | 211.12 | 11.15.3 |  | The ECSA may not broadcasted continuously after the switch count is zero. In case the last one with the Channel Switch Count = 0 is missed, then the conditions here would not function as intended. So we should phrase this according to a count on the remaining TBTTs to occur before the channel switch is to occur, based on the most recently received ECSA. | Phrase this condition (c) according to a count on the remaining TBTTs to occur before the channel switch is to occur, based on the most recently received ECSA. | Counter – see CID 5088 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5723 | Stephens, Adrian | 211.21 | 11.15.3 |  | "was transmitted by the STA (if any), is non-zero" - the exclusion is unnecessary because of "either, the AP has not received a Notify Channel Width action frame that was transmitted by the STA, or" | Delete the "(if any)" | Accept - Editor shall make changes shown under all headings that include reference to CID 5723 within 11-07-2742r11 |
| 5724 | Stephens, Adrian | 211.42 | 11.15.3 |  | "Independently of the values of the five fields, a non-AP STA that is a member of a 20/40 MHz BSS shall not transmit non-Control type 20 MHz PPDUs in the secondary channel." The implication is that: 1. An AP may transmit 20MHz data (and any other frames) on the secondary channel 2. A non-AP STA may transmit 20MHz control frames on the secondary channel. Neither of these are consistent with: 209.55: "An FC HT STA shall use the 20 MHz primary channel to transmit and receive 20 MHz HT PPDUs ." which bans both implied behaviours. | Delete the first cited text. | Counter – the rationale for the change is different – since the other cited text is modified to be consistent with the text that is being deleted here - but still , the text is redundant. Editor shall make changes shown under all headings that include reference to CID 5724 within 11-07-2742r11 |
| 5872 | Yamaura, Tomoya | 211.46 | 11.15.4 |  | Why should we only consider the overlapping BSS, but not consider overlapping IBSS ? | Add overlapping IBSS to be scanned. Or, explicitly prohibit to use 40MHz for IBSS. | Counter – see CID 5869. the proposed change seems to refer to two separate issues. The first being that STA should be scanning for both BSS and IBSS – this part of the comment is rejected, since an IBSS is by definition, a BSS. The second proposed change asks for a prohibition of 40 mhz operation in an IBSS. The current draft includes allowance for 40 mhz IBSS operation within 5 GHz only, and does this by using the DFS owner mechanism which is already in the baseline. But the DFS owner mechanism is deemed buggy, so there will be an introduced prohibition against making CHANGES to the BSS width of an IBSS. See CID 5869. See also CIDs 5132, 5473, 5710. |
| 5097 | Chan, Douglas | 211.46 | 11.15.4 |  | We see in the last paragraph of this clause that an associated FC HT STA 19 has a min requirement for performing scanning, yet this requirement is not suffice to fulfill a 2.4 GHz 20/40 BSS's the OBSS Scan operation's min requirements. It is unclear then who's job it is to make sure these min requirments are fulfilled. | Specify whose responsibilies is it to ensure these OBSS Scan requiremtns are met, i.e., the AP or its associated STAs or both. | Counter – Editor shall make changes as shown under any heading within 11-07-2742r11 that includes CID 5097. An FC HT STA 19 has a requirement to scan as determined by its MIB variables. These MIB variables start with defaults and are set by any received Overlapping BSS Scan Parameters elements. It is the STA’s responsibility to honour the settings of its OBSS-scan-related MIB variables and to update these according to any received Overlapping BSS Scan Parameters elements. It is the AP’s choice what value these parameters are set to outside of leaving them at the default values, constrained by the minimum and maximum value for each parameter as specified in the MIB. These roles are identified in the draft (11.15.4). Some changes to the text include the allowance of STA to request an exemption from scanning that is dependent on the amount of TX and RX activity perfomed by the STA. |
| 5091 | Chan, Douglas | 211.56 | 11.15.4 |  | What is the meaning of "total scan time"? During this time, is the respective passive or active scan supposed to happen continuously? | Clarify the meaning of "total scan time". | Accept - Editor shall make changes shown under all headings that include reference to CID 5091 within 11-07-2742r11. See also CID 5092, 5093. |
| 5090 | Chan, Douglas | 211.56 | 11.15.4 |  | This sentence which describes the min overlapping BSS scan opeartion requirements is written very unclearly and may be proned to misinterpretations. In particular, the dot11OBSSScanPassiveTotalPerChannel and dot11OBSSScanActiveTotalPerChannel time units by themselves can already mandate the min no. of times each channel is scanned, and so no need to state that per dot11BSSWidthTriggerScanInterval each channel is scanned at least twice. (I understand the following NOTE tries to clarify this, but still....) | See if we can remove the requirement to scan at least twice and make this depend on the dot11OBSSScanPassiveTotalPerChannel and dot11OBSSScanActiveTotalPerChannel times, if in fact that during the total scan time the respective scans are supposed to happen continuously. | Accept –- Editor shall make changes shown under all headings that include reference to CID 5090 within 11-07-2742r11 which remove the requirement for two scans minimum per interval |
| 5096 | Chan, Douglas | 211.65 | 11.15.4 |  | Why are we transmitting the "limits" of these MIB variables? We should be transmitting the values of these MIB vars. | Change word "limits" to "values". | Counter - Editor shall make changes shown under all headings that include reference to CID 5096 within 11-07-2742r11 |

| 5098 | Chan, Douglas | 212.05 | 11.15.4 |  | The "relationships b/w the frame fields and MIB vars" mentioned here is undefined. Moreover, there're in fact no relationships b/w them, are there? | Clarify meaning of this "relationship". | Counter - replace “relationship” with “mapping”. - Editor shall make changes shown under all headings that include reference to CID 5098 within 11-07-2742r11 In reply to the commenter, this mapping is defined unambiguously in 7.3.2.55. for which reference is already included in the cited sentence. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5725 | Stephens, Adrian | 212.20 | 11.15.4 |  | It has not been proven to me that any non-zero value of dot11OBSSScanActivityThreshold is reasonable. I think the presence of this parameter is unnecessary, as there is nothing to stop a STA that wants to avoid scanning from re-associating with altered capabilities. On establishment of a call, a mobile power-saving STA can quickly re-associate - the time taken to do this is irrelevant (~10ms) compared to the duration of a call or the call set-up time. | Remove this parameter and relevant associated text. | Reject. Group feels that the current description satisfies the largest group of voters that it is possible to satisfy over other possible alternatives. Straw poll deletion of parameter: 3-7-10 Straw poll changing threshold to allowance to petition for exemption, without requiring the exemption: |
| 5475 | Perahia, Eldad | 212.20 | 11.15.4 |  | Remove the OBSS scan exception and dot11OBSSScanActivityThreshold. | as in comment | Reject - See CID 5725 |
| 5894 | Myles, Andrew | 213.44 | 11.15.10 |  | If one makes an analogy between this clause and a software procedure definition then one could assert that the clause is unacceptable and unreviewable in its current form: * The input are not defined at the start of the procedure * The outputs are not defined at the start of the procedure * There is no documentation as to what the procedure is supposed to do. * It is spaghetti code * It contains ambiguities, eg is "trigger event a)" on pp 214 line 44 the same as trigger event a) in line 44 or trigger event a) on pp 213 line 48? | Completely rewrote this clause so that it is clear: * What the inputs are * What the outputs are * What it is trying to achieve. * How it is trying to achieve it. I would suggest a state diagram might be the clearest way of doing this | Counter. While the clarity of this subclause has been incrementally improved by resolution to other more specific comments, it is not possible to meaningfully respond to the commenter’s main point “rewrite it to make it clearer” without further input from the commenter. Regarding the specific point raised. Editor shall label the definitions of the trigger events “TE-A”, “TE-B”, … and then replace references to these events with these labels. This change removes any possible ambiguity about references to a) when there are multiple “a)”s in the subclause. Editor shall make changes shown under all headings that include reference to CID 5894 within 11-07-2742r11 |
| 5279 | Fischer, Matthew | 213.44 | 11.15.10 |  | Implementation complexity problems at both AP and STA. The requirement of the STA to perform hystersis is redundant to the Aps requirement and causes extra work for the STA implementation. The report at change requirement at the STA causes extra complexity at the AP because it requires the AP to maintain last known state information per STA per channel and then perform periodic summation of said state information. | Eliminate the STA hysteresis requirement and force the STAs to send a report after every OBSS Scan unless nothing was detected. This might increase traffic a bit, but with the current OBSS Scan parameter values, it is not that much traffic, unless of course, you have dozens of STAs... | Accept - Editor shall make changes shown under all headings that include reference to CID 5279 within 11-07-2742r11 |
| 5106 | Chan, Douglas | 213.51 | 11.15.10 |  | Missing "Management frame" after "20/40 BSS Coexistence". | Add "Management frame" after "20/40 BSS Coexistence". | Counter: It is not necessary to include “management” as this is not part of the name of the frame. However, the list structure can be clarified so that the final “frame” more obviously attaches: Editor shall make changes shown under all headings that include reference to CID 5106 within 11-07-2742r11 |

| 5277 | Fischer, Matthew | 213.51 | 11.15.10 |  | Trigger condition b) address check - this trigger condition, as currently defined, requires a promsicuous MAC receive behavior, and I am surprised that no one objected earlier. Currently, you would have to check Address1, Type, Subtype, Category and Action before deciding on whether to keep this frame or not. | Eliminate the promiscuous receive requirement found in trigger condition b) by stating that it is only a trigger condition if the frame matches an Address1 condition - i.e. unicast Address1 value that matches this STA's MAC address, or any MCAST or BCAST address1 value with no regard to the Address3 value for either case. | Accept - Editor shall make changes shown under all headings that include reference to CID 5277 within 11-07-2742r11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5731 | Stephens, Adrian | 214.16 | 11.15.10 |  | I believe it makes no sense for an AP to permit 40Mhz operation in its BSS if it itself is unable or unwilling to receive 40MHz packets. If we accept this we can simplify statements such as: "An FC HT AP 19 shall not set the STA Channel Width field to 1 in transmitted 20/40 BSS Coexistence elements and shall not set the Secondary Channel Offset field to a non-zero value in transmitted 20/40 BSS Coexistence elements unless the following three conditions have been met:" | Add a statement that the AP's STA Channel Width shall be 1 when the secondary channel offset is non-zero and 0 otherwise. Then we can simplify the above to: "An FC HT AP 19 shall not set the Secondary Channel Offset field to a non-zero value in transmitted 20/40 BSS Coexistence elements unless the following three conditions have been met:" | Accept - Editor shall make changes shown under all headings that include reference to CID 5731 within 11-07-2742r11 |
| 5107 | Chan, Douglas | 214.16 | 11.15.10 |  | The fields mentioned here are not in the 20/40 BSS Coexistence element, but in the HT IE. | Change the two references to "20/40 BSS Coexistence" in this paragraph to "HT Information" | Counter – accept in principle – but the reference to STA Channel width has been deleted. See CID 5731. |

| 5108 | Chan, Douglas | 214.32 | 11.15.10 |  | It's not 11.15.4, but 11.9.8.3. | Fix. | Counter – phrase containing the reference is deleted due to other changes. See 11-07-2742r11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5811 | Trainin, Solomon | 214.33 | 11.15.10 |  | "An FC HT AP 19 shall not set the STA Channel Width field to 1 in transmitted 20/40 BSS Coexistence elements and shall not set the Secondary Channel Offset field to a non-zero value in transmitted 20/40 BSS Coexistence elements unless the following three conditions have been met:" The conditions how to manipulate the STA Channel Width field and the Secondary Channel Offset field includes the same trigger events as the conditions used to reevaluate the local variable 20/40 Operation Permitted therefore is not clear what is the need to include the 20/40 Operation Permitted as the argument to make decision (condition c) in the line 33 p 214) | Remove the local variable 20/40 Operation Permitted in the subclause 11.15.10 | Counter – The first subcondition of item a) is redundant to the item c). With new changes, item a) covers AP detection of fortymhz intolerance, b) covers associated STA signaling of their detections of 40mhz intolerance bits and c) (20/40 op permit var) covers AP and STA detection of OBSSs . Editor shall make changes shown under all headings that include reference to CID 5811 within 11-07-2742r11 |
| 5732 | Stephens, Adrian | 215.34 | 11.15.10 |  | "The STA shall set to 1, the Forty MHz Intolerant field of the 20/40 BSS Coexistence element for inclusion in the candidate frame if and only if the dot11FortyMHzIntolerant MIB attribute is true." is a redundant subset of: 213.37: "A STA 19 shall set the Forty MHz Intolerant field to 1 in transmitted 20/40 BSS Coexistence fields if and only if the value of the MIB attribute dot11FortyMHzIntolerant is true." | remove the redundancy - i.e., by deleting the first cited text | Counter – Editor to make changes shown under any heading that includes 5732 in doc 11-07-2742r11. the cited text is part of a set of instructions for building a frame, and if it is deleted, it might appear that this bit is never to be set when following the instructions, so the proposed change is to instead, remove the shall in the cited text and instead, insert a reference to the other location in subclause 11.15.9 |
| 5278 | Fischer, Matthew | 215.41 | 11.15.10 |  | Overkill on use of 20 MHz BSS Width Request field - this field is set in the STA outgoing frame when any trigger condition a) (legacy BSS detected within the 2.4 GHz band) is active - but the BSS is not required to switch to 20 MHz operation unless there is a detected legacy BSS within some subset of the 2.4 GHz band. | Change the setting of the 20 MHz BSS Width Request field so that it is only set for the first condition listed - that is, a trigger b) condition = Forty_MHZ_Intolerant=1 detected. Let the channel report information indicate legacy BSS placement per channel, and let the receving AP collect and collate that information and make its own decision as to 20 MHZ BSS operation based on the calculation of 20/40 MHz BSS Operation Permitted variable. | Accept - Editor shall make changes shown under all headings that include reference to CID 5278 within 11-07-2742r11 |
| 5110 | Chan, Douglas | 219.16 | 11.17 |  | This paragraph says that a STA supporting tx/rx of 20/40 BSS Coex Mgmt frame may transmit to any other STA that supports this ability as well. Should we add some restrain to when this is transmitted? | Add some restrain to when this is transmitted. | Reject – there are many instances in the baseline standard where, for example, an MLME request generates an action frame for transmission. There are, in general, no restrictions on the frequency of generation of such requests. |
| 5747 | Stephens, Adrian | 219.32 | 11.17 |  | "A STA may transmit to any STA a 20/40 BSS Coexistence Management frame with a broadcast/multicast address in the address1 field" The "to any STA" is probably unnecessary and certainly incorrect. For example, there is undoubedtly a STA on a different channel located in a screened room somewhere in outer Mongolia that would probably not receive such a frame unless its receiver sensitivity is a little better than average. | Replace with: "A STA may transmit a 20/40 BSS Coexistence Management frame with a broadcast/multicast address in the address1 field." | Counter – accept in principle with some wording changes to accommodate the current fashion with respect to some of the terminology. Editor shall make changes shown in submission 11-07-2742r11 under any heading that includes a reference to CID 5747. |
| 5111 | Chan, Douglas | 219.37 | 11.17 |  | Whom is this transmission success directed to? | Specify. | Counter – delete the cited sentence, since it seems to have little useful meaning. Editor shall make changes shown under all headings that include reference to CID 5111 within 11-07-2742r11. |
| 5748 | Stephens, Adrian | 219.50 | 11.17 |  | "A STA that receives a 20/40 BSS Coexistence Management frame that contains a value of 1 for the Request Information field shall transmit a 20/40 BSS Coexistence Management frame that has the value of its RA field set to the address of the STA from which it received the request." Without a timing constraint this "shall" is meaningless. i.e. - after 1000 years is still a compliant implementation - but it is unlikely that the requesting STA is still hanging around for the response, or the operator of the STA is still around to care either way. | Either define a time constraint, or turn into a "should". | Counter – Make the language specify “immediately queue for transmission”. Editor shall make changes shown under all headings that include reference to CID 5748 within 11-07-2742r11 |
| 5807 | Trainin, Solomon | 219.50 | 11.17 |  | "A STA that receives a 20/40 BSS Coexistence Management frame that contains a value of 1 for the Request Information field shall transmit a 20/40 BSS Coexistence Management frame that has the value of its RA field set to the address of the STA from which it received the request." "A STA that receives a 20/40 BSS Coexistence element with the Information Request field set to 1 shall transmit a 20/40 BSS Coexistence Management frame with the transmitting STA as the recipient." The second sentence covers also the case of the first one. The first sentence seems to be excessive | leave one sentence | Counter – delete the first cited sentence as specified in the editor instructions found in 11-07-2742r11 under any heading containing a reference to CID 5807. |
| 5749 | Stephens, Adrian | 219.56 | 11.17 |  | "A STA that receives a 20/40 BSS Coexistence element with the Information Request field set to 1 shall transmit a 20/40 BSS Coexistence Management frame with the transmitting STA as the recipient." This is redundant given 219.50: "A STA that receives a 20/40 BSS Coexistence Management frame that contains a value of 1 for the Request Information field shall transmit a 20/40 BSS Coexistence Management frame that has the value of its RA field set to the address of the STA from which it received the request." | Delete the first cited sentence. | Accept – see also CID 5807. |
| 5750 | Stephens, Adrian | 219.57 | 11.17 |  | "When the Information Request field is set to 0, the recipient of the 20/40 BSS Coexistence element may transmit a 20/40 BSS Coexistence Management frame with the transmitting STA as the recipient, but should not." What is this trying to tell me? I cannot understand "may ... but should not" Also, there is no timing constraint. This tells me that at any time after (even 10 years) receiving the frame with the request field set to 0, I should not transmit the coexistence management frame. | Delete the cited sentence (my preference) or replace it with: "A STA that receives a 20/40 BSS Coexistence element with the Information Request field is set to 0 should not transmit a frame containing a 20/40 BSS Coexistence element directed to the transmitter of the received frame for a period defined by dot11HTWaitAroundForABit TU." And add the relevant MIB definition. | Counter – delete as first suggested. Editor shall make changes shown under all headings that include reference to CID 5750 within 11-07-2742r11 |
| 5112 | Chan, Douglas | 515.39 | S.4.2 |  | The MIB variables listed do not pertain to just "the particulars of channel dwell timing during overlapping BSS scanning", but the particulars of the overlapping BSS scanning. | Remove "channel dwell timing during" from "the particulars of channel dwell timing during overlapping BSS scanning". | Accept. Editor shall make changes shown under all headings that include reference to CID 5112 within 11-07-2742r11 |
| 5792 | Stephens, Adrian | 517.11 | S.4.3 |  | "HT STAs that are not associated with an AP whose BSS is operating on a channel in the 2.4 GHz band" This is misleading. it is "not 2.4" that is the essence, not "not associated" | Reword thus: "HT STAs that are associated with an AP whose BSS is operating on a channel that is not in the 2.4 GHz band" | Accept. Editor shall make changes shown under all headings that include reference to CID 5792 within 11-07-2742r11 |

COMMENTS FROM COEX REORG GROUP

| 5132 | Chan, Douglas | 11.9 | 204.40 |  | In the clause of 11.9 has an introduction that briefly describes the purposes and scope of its subclauses. We should provide such an introduction for those new 11.9 subclauses introduced by 11n. | Add such an intro. | Counter – see 5473. |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 5473 | Perahia, Eldad | 11.9.8 | 205.06 |  | Channel selection for 20/40 MHz operation has nothing to do with DFS and should not be part of the DFS subclause | Move all of 11.9.8 Channel selection methods for 20/40 MHz Operation out of 11.9 and into 11.15 | Accept - editor shall make changes shown under all headings that include reference to CID 5473 within 11-07-2742r11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5710 | Stephens, Adrian | 11.9.8.1 | 205.11 |  | "The following definitions apply in 11.9.8.2 (Introduction) to 11.16.1 (General description of PCO):" There's a lot of stuff between these two references that doesn't care about these definitions. | Reference those subclauses where these definitions apply. | Counter – set of subclauses modified due to the movement of 11.9.8 to 11.15.3a or thereabouts for CID 5473. Editor shall make changes shown under all headings that include reference to CID 5710 within 11-07-2742r11 |
| 5133 | Chan, Douglas | 11.9.8.1 | 205.11 |  | These definitons also apply to clauses outside of 11.9.8 only, we should move them to the next highest subclause. | Move these definitions the next highest subclause, like 11.9. | Counter – see CID 5473 – which instructs the editor to move the definitions to subclause 11.15.x as part of the general move of subclause 11.9.8 to within 11.15 due to resolution of CID 5473. |
| 5181 | Ecclesine, Peter | 11.9.8.3 | 206.22 |  | Add reference to 11.15.1, where FC HT AP 19 is defined. | Change to 'FC HT AP 19 (see 11.15.1)' | Counter – 11.9.8.3 is moved to appear after the definitions per resolution to CID 5473. |
| 5714 | Stephens, Adrian | 11.9.8.3 | 206.23 |  | The term FC HT AP 19 is used in 11.9.8.3 and defined in 11.15.1 to apply through 11.15. | Add reference in 11.9.8.3 to definition in 11.15.1 | Counter – 11.9.8.3 is moved to appear after the definitions in 11.15.1 per resolution to CID 5473. |
| 5137 | Chan, Douglas | 11.9.8.3 | 206.65 |  | The term FC HT AP 19 is introduced in a later subclause. | Should we move the definition up? | Counter – 11.9.8.3 is moved to appear after the definitions in 11.15.1 per resolution to CID 5473. |
| 5012 | Adachi, Tomoko | 11.15 | 208.50 |  | FC HT AP is used in 11.9.8.3 and 11.9.8.4, too. | Move the definition to 11.9.8.1. | Counter – 11.9.8.3 is moved to appear after the definitions in 11.15.1 per resolution to CID 5473. |

DISCUSSION

20 MHz BSS Width Request field

The 20/40 coexistence mechanism contains one minor oversight. The 20 MHz BSS Width Request field of the 20/40 BSS Coexistence Management action frame is set by a STA when the STA detects a trigger condition a). But trigger condition a) is not limiting with respect to the “affected” channels – it covers the entire 2.4 GHz band, while the 20 MHz BSS Width Request field is forcing. This document proposes a solution.

The solution to fix this overkill is to change the use of the field. Since the 20/40 Operation Permitted variable already checks the appropriate range of channels, the AP only needs to consider the Channel List information to determine whether or not to switch to 20 MHz operation. It is already required to use the information in the report elements when checking the value of Operation Permitted. This means that the 20 field should only be set when the reporting STA detects a Forty_MHZ_Intolerant bit = 1.

An additional proposed change is to the use of the 20MHZ_BSS_WIDTH_REQUEST field and the channel report information in the 20/40 BSS Coexistince management frame. Looking carefully at the text, one notes that the detection of a Forty_MHZ_Intolerant bit in ANY channel in the 2.4 GHz band is cause for reversion to 20 MHz BSS operation. Therefore, specific channel information regarding the source of the Forty_MHZ_Intolerant field is not useful to the AP, and so, the channel information in the 20/40 BSS Coexistince management frame can be made more useful if it reflects only the trigger events a) detected by STA (i.e. only information regarding legacy BSS detection).

State-change reporting vs per-scan reporting

This proposal suggests removing the requirement to report OBSS scan information only at state change (which exists to reduce generated report traffic), thereby eliminating the requirement for scanning STA to preserve local state, and thereby eliminating any possible duplicate accounting of the hysteresis to revert to 20/40 MHz BSS operation. This change allows the simplification of the STA implementation and it simplifies the AP implementation as well. The STA is no longer required to maintain any state or timer information if this change is adopted. For the AP, the change allows the AP to eliminate the requirement to maintain per-STA records of received channel information. With per-scan period reporting, the AP can simply logically OR received information into a single global state variable as that information arrives. It would keep a history listing that is dot11BSSWidthChannelTransitionDelayFactor records deep.

This change also eliminates some double accounting. The original text requires BOTH STA and AP to maintain old state for a period of dot11BSSWidthChannelTransitionDelayFactor times dot11BSSWidthTriggerScanInterval seconds. This was suggested in the current draft in order to minimize the transmission of overlap report frames by scanning STAs. In the current draft, reports are only sent as a result of state change at the STA. In order to determine state change, the STA must preserve past state. However, given the parameter values for scanning operations, and assuming that a report is transmitted for every scan, the resulting frequency of transmission of report frames would not be expected to be high.

Furthermore, by allowing the STA to NOT send a report if there is nothing to report, traffic is reduced in those cases.

Another possibility is to have the STA only send on state change by including only the channels for which state information has changed. Since each report can have a variable number of channels, and the reports are unicast with reliable acknowledgements, this can work well. However, the 20 mhz BSS Width request bit, which signals the detection of a 40 mhz intolerant bit, this is not directly possible with the existing frame format. So this alternative proposal would require modifying the existing format to create another signaling indication, which validates the 20 mhz BSS width request bit. But given the overhead of a set of reports sent at a fairly low frequency (default value for dot11BSWidthTriggerScanInterval is 300 seconds), and the requirement that self-dissociating STA and STA dissociated due to timeout would require the AP to rebuild the current state with a query of all remaining STA, the difference in overhead could be either positive or negative, and is likely to be small. The mechanism that provides for a simple instruction to scan and then send a report with no memory requirement on the part of the STA and also allowing the AP to simply SUM the reports is simple, and this simplicity is probably the most solid footing on which to make a decision among the alternatives.

Promiscuous reception requirement

The last change proposed here is to eliminate the requirement to promiscuously detect the presence of the Forty_MHZ_Intolerant field. This is done by modifying the trigger condition b) such that a unicast address must match the receiver’s MAC address in order to be detected as a trigger. The other MCAST and BCAST conditions remain, and there is still no change to the requirements for the BSSID (i.e. address3) value in such a frame. (I.e. the STA is still required to be promiscuous with respect to the received BSSID value.)

CID 5097

TGn Editor: make the following changes to Draft 3.01, inserting two new bits to the 20/40 BSS Coexistence element to replace to existing reserved bits as shown:

7.3.2.56 20/40 BSS Coexistence element

B0		B1	B2		B3	   B4	      B5           B7

| Element ID | Length | Information Request | Forty MHz Intolerant | 20 MHz BSS Width Request | OBSS Scanning Exemption Request | OBSS Scanning Exemption Grant | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |

Octets : 	1	   1

1

Figure 7-95am—20/40 BSS Coexistence element format

…………………………………….. described in 11.15.10 (Switching between 40 MHz and 20 MHz (#75)).

The Scanning Exemption Request field it set to 1 to indicate that the transmitting non-AP STA is requesting the BSS to allow the STA to be exempt from OBSS scanning.

The Scanning Exemption Grant field is set to 1 to indicate that the receiving STA is exempted from performing OBSS Scanning when transmitted by the AP.

The OBSS Scanning Exemption Request field is reserved when transmitted by an AP. The OBSS Scanning Exemption Grant field is reserved when transmitted by a non-AP STA. Scanning Exemption Request and Scanning Exemption Grant field are reserved when 20/40 BSS Coexistence element is included in Group Addressed Frame.

CID 5277, 5278, 5279, 5032, 5319, 5716, 5106, 5107, 5108, 5808, 5811, 5732, 5731

TGn Editor: Remove the Secondary Channel Offset element from the Beacon and Probe Response frames in clause 7 TGn Draft D3.02 and remove the references to the inclusion of this element in those frames that appears within subclause “7.3.2.20a Secondary Channel Offset element”.

TGn Editor: Change the text of subclause “11.15.10 Switching between 40 MHz and 20MHz” beginning on page 213 at about line 44 of TGn Draft D3.0, as follows:

11.15.10 Switching between 40 MHz and 20 MHz

The following events are defined to be BSS channel width trigger events:

a) on any of the channels of the channel set defined in Clause 19, reception of a Beacon frame that does not contain an HT Capabilities element

b) on any of the channels of the channel set defined in Clause 19, reception of a 20/40 BSS Coexistence, Beacon, Probe Request or Probe Response frame that contains a value of 1 in a Forty MHz Intolerant field and that has the Address 1 field set to the receiving STA’s address, or to a group addressvalue, with no further addressing qualifications

c) reception of a 20/40 BSS Coexistence frame with the 20 MHz BSS Width Request field set to 1 and with a value for the Address 1 field that matches the receiving STA using either unicast or MCAST/BCAST addressing and with a value for the TA field that corresponds to the MAC address of a STA with which the receiver is associated

d) reception of a 20/40 BSS Coexistence frame containing at least one 20/40 BSS Intolerant Channel Report element with a non-zero length and with a value for the Address1 field that matches the receiving STA using either unicast or MCAST/BCAST addressing, but with no qualification of the Address 3 value.

An FC HT AP 19 shall re-evaluate the value of the local variable 20/40 Operation Permitted (see 11.9.8.3) when either of the following conditions occurs:

a) a BSS channel width trigger event a) is detected, or

b) a BSS channel width trigger event d) is detected.

An FC HT AP 19 may re-evaluate the value of the local variable 20/40 Operation Permitted (see 11.9.8.3) when either of the following conditions occurs:

a) no BSS channel width trigger events a) are detected for a period of time equal to dot11BSSWidthChannelTransitionDelayFactor * dot11BSSWidthTriggerScanInterval seconds

b) no BSS channel width trigger events d) are detected for a period of time with a duration of dot11BSSWidthChannelTransitionDelayFactor * dot11BSSWidthTriggerScanInterval seconds

An FC HT AP 19 that detects either of the BSS channel width trigger events b) or c) or that determines that the value of its variable 20/40 Operation Permitted has changed from TRUE to FALSE shall set the Secondary Channel Offset field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted to indicate that no secondary channel is present (i.e., that the BSS operating width is 20 MHz).

An FC HT AP 19 shall not set the Secondary Channel Offset field to a value of 1 or 3 in transmitted HT Information elements unless the following two conditions have been met:

a) A period of dot11BSSWidthChannelTransitionDelayFactor * dot11BSSWidthTriggerScanInterval seconds have elapsed during which

no BSS channel width trigger events b) or c) are detected.

c) The value of the local variable 20/40 Operation Permitted (see 11.15.4 (Scanning requirements for 40 MHz capable STA)) is TRUE.

To request an update of the status of the 20 MHz BSS Width Request field, an FC HT AP 19 can transmit a 20/40 BSS Coexistence Management frame with a value of 1 in the Information Request field as described in 11.17 (20/40 BSS Coexistence Management frame usage).

An FC-HT–STA 19 that is associated with an FC HT AP 19 shall maintain a record of detected BSS channel width trigger events as follows:

a) For each detected BSS channel width trigger event a):

1) If a DS Parameter Set field is present in the received Beacon, then the channel of the BSS channel width trigger event is the value of the Current Channel field of the DS Parameter Set field, otherwise, the channel of the BSS channel width trigger event is the channel on which the detecting STA received the Beacon.

2) If a Supported Regulatory Classes element is present in the received Beacon, then the regulatory class of the BSS channel width trigger event is the value of the Current Regulatory Class field of the Supported Regulatory Classes element of the received Beacon, otherwise, the regulatory class of the BSS channel width trigger event is “unknown”.

b) For each detected BSS channel width trigger event a) of a unique combination of regulatory class and channel, the FC HT STA 19 shall maintain a record containing two variables:

1) the regulatory class of the BSS channel width trigger event,

2) the channel of the BSS channel width trigger event

If a BSS channel width trigger event a) is detected for a regulatory class and channel combination for which no record exists, then the STA shall create such a record.

If a BSS channel width trigger event a) is detected for a regulatory class and channel combination for which a record already exists, then the information in that record is updated with the information determined from the new trigger event.

For all BSS channel width trigger events b), the FC HT STA 19 shall maintain a single record containing an indication of whether one or more trigger events b) have been detected.

At the completion of an overlapping scan operation (i.e. at the end of the period of time equal to dot11BSSWidthTriggerScanInterval) or when it receives a 20/40 BSS Coexistence Management frame from its associated AP and containing a value of 1 in the Information Request field, an FC-HT–STA 19 that is associated with an FC HT AP 19 shall create a 20/40 BSS Coexistence Management frame by including a value of zero for all fields of a 20/40 BSS Coexistence Management frame and then transferring information from the BSS channel width trigger event a) and b) records to the frame according to the following four steps:

a) For each unique regulatory class that is stored in the set of BSS channel width trigger event a) records, the STA shall create a 20/40 BSS Intolerant Channel Report element for inclusion in the frame and include all of the unique channels associated with the regulatory class in the channel list of that element.

b) The STA sets the Forty MHz Intolerant field of the 20/40 BSS Coexistence element based on the value of the dot11FortyMHzIntolerant MIB attribute (see 11.15.9 Signaling 40 MHz intolerance).

c) The STA shall set to 1, the 20 MHz BSS Width Request field of the 20/40 BSS Coexistence element for inclusion in the frame if

a record for BSS channel width trigger event b) exists and indicates that at least one trigger event b) has been detected.

d) The STA may set to 1, the Information Request field

Upon completion of these four steps, the FC HT STA 19 shall delete all records for trigger events a) and b). Subsequently detected trigger events cause the creation of new records as necessary to be used in subsequently generated 20/40 BSS Coexistence Management action frames. Following the record deletion, the FC HT STA 19 shall transmit to its associated FC HT AP 19 the 20/40 BSS Coexistence Management action frame if any of the following five conditions is true:

1) at least one 20/40 BSS Intolerant Channel Report element(s) with the length field set to a non-zero value is included

2) the Forty MHz Intolerant field is set to 1

3) the 20 MHz BSS Width Request field is set to 1

4) the Information Request field is set to 1

5) if the frame was created in response to the reception of an Information Request field that was set to 1

11.9.8.3 Scanning requirements for a 20/40 MHz BSS

TGn Editor: Change the text of what might be the tenth paragraph of subclause “11.9.8.3 Scanning requirements for a 20/40 MHz BSS” on page 206 at about line 48 of TGn Draft D3.0, as follows:

OTi = member i of the set of channels that comprises all channels that are members of the channel set C that were listed at least once in the Channel List fields of received 20/40 BSS Intolerant Channel Report elements during the previous dot11BSSWidthChannelTransitionDelayFactor * dot11BSSWidthTriggerScanInterval seconds and all channels that are members of the channel set C and that are the primary operating channel of at least one 20 MHz BSS that were detected within the AP's BSA during the previous dot11BSSWidthChannelTransitionDelayFactor * dot11BSSWidthTriggerScanInterval seconds

TGn Editor: Change the text of what might be the fourteenth paragraph of subclause “11.9.8.3 Scanning requirements for a 20/40 MHz BSS” on page 207 at about line 4 of TGn Draft D3.0, as follows:

In addition to information obtained by the FC HT AP 19 through its own scanning, an FC HT AP 19 shall use 20/40 BSS Intolerant Channel Report information from  received 20/40 BSS Coexistence Management frames with a value for the Address 1 field that matches the FC HT AP 19 using either unicast or MCAST/BCAST addressing, but with no qualification of the Address 3 value, when determining if 20/40 Operation Permitted is TRUE or FALSE. The information from the Channel List fields of received 20/40 BSS Intolerant Channel Report elements is used in generating the OT set.

CID 5894

TGn Editor: Within subclause “11.14.10 Switching between 40 MHz and 20 MHz” beginning on page 232 at about line 31 of TGn Draft D3.03, label the definitions of the trigger events as “TE-A”, “TE-B”, ... and then replace references to these events with these labels.”

CID 5094, 5594, 5595

TGn Editor: Change the fourth paragraph of subclause “7.3.2.54 20/40 BSS Intolerant Channel Report element” on page 82 at about line 59 of TGn Draft D3.0, as follows:

Regulatory Class contains an enumerated value from Annex J, encoded as an unsigned integer, specifying the regulatory class in which the Channel List is valid. A 20/40 BSS Intolerant Channel Report only reports channels for a single regulatory class. Multiple 20/40 BSS Intolerant Channel Report elements are used to report channels in more than one regulatory class.

TGn Editor: Change the last paragraph of subclause “7.3.2.54 20/40 BSS Intolerant Channel Report element” on page 83 at about line 6 of TGn Draft D3.0, as follows:

A 20/40 BSS Intolerant Channel Report element includes only channels that are valid for the regulatory domain in which the STA transmitting the element is operating and that are consistent with the Country element transmitted by the AP of the BSS of which it is a member.

CID 5095

TGn Editor: Insert the phrase “encoded as an unsigned integer” following each phrase that is of the sort “contains a value in units” where units is a placeholder for some specific measure, within subclause “7.3.2.55 Overlapping BSS Scan Parameters element” on page 83 at about line 8 of TGn Draft D3.0.

CID 5869

TGn Editor: Insert the following editor instructions and text to appear immediately preceeding subclause “11.9.8 Channel selection methods for 20/40 MHz operation” and the editor instruction that appears just before that header, on page 209 at about line 31 of TGn Draft D3.02.

Editor: change the first paragraph of 11.9.7.2 as shown:

11.9.7.2 Selecting and advertising a new channel in an IBSS

DFS in an IBSS is complicated by the following:

— There is no central AP function for collating measurements or coordinating a channel switch. If STAs make independent decisions to switch channel in the presence of radar, there is a danger that all STAs will announce a switch to differing channels if several of them detect the radar.

— There is no association protocol that can be used to

— Exchange supported channel information and

— Determine membership of the IBSS at a given instant for requesting measurements.

- Beaconing is a shared process; therefore, it cannot be guaranteed that a STA that has something to send (e.g., a channel switch message) will be the next STA to Beacon frame.

- A 20/40 MHz IBSS cannot be changed to a 20 MHz IBSS and a 20 MHz IBSS cannot be changed to a 20/40 MHz IBSS.

CID 5711, 5029, 5849

TGn Editor: Change the definitions found in subclause “11.9.8.1 General” on page 205 beginning at about line 9 of TGn Draft D3.0 as follows:

11.9.8.1 General

The following definitions apply in 11.9.8.2 (Introduction) to 11.16.1 (General description of PCO):

— 20 MHz BSS: A BSS in which the Secondary Channel Offset field is set to 0.

— 20/40 MHz BSS: A BSS in which supported channel width of the AP or IDO STA is 20 MHz and 40 MHz (Channel Width field is set to 1) and the Secondary Channel Offset field is set to a value of 1 or 3.

CID 5849

TGn Editor: Change the third paragraph of subclause “11.9.8.2 Introduction” on page 205 at about line 35 of TGn Draft D3.0 as follows:

An AP operating a 20/40 MHz BSS, on detecting an overlapping BSS whose primary channel is the AP’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz BSS operation. An IDO STA operating a 20/40MHz IBSS, on detecting an overlapping BSS whose primary channel is the IDO STA’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz IBSS operation.

CID 5857, 5030, 5860, 5031, 5808, 5078, 5079

TGn Editor: Change the first ten(?) paragraphs of subclause “11.9.8.3 Scanning requirements for a 20/40 MHz BSS” beginning on page 205 at about line 42 of TGn Draft D3.0, as follows:

Before an AP or IDO STA starts a 20/40 MHz BSS it shall perform a minimum of dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS Scans (see 11.15.4 (Scanning requirements for 40 MHz capable STA)) to search for existing BSSs.

If the AP or IDO STA chooses to start a 20/40 MHz BSS in 5 GHz and that occupies the same two channels as any existing 20/40 MHz BSSs, then the AP or IDO STA shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSSs and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSSs, unless the AP discovers that on these two channels are existing 20/40 MHz BSSs with different primary and secondary channels.

If an AP or IDO STA chooses to start a 20/40 MHz BSS in 5 GHz, then the selected secondary channel should correspond to a channel on which no beacons are detected during the  dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS scan time performed by the AP or IDO STA , unless there are beacons detected on both the selected primary and secondary channels.

NOTE—The 20/40 MHz channel sets and their corresponding behavior limits (i.e., choice of primary and secondary channels) permissible in each regulatory class are respectively defined in Annex J and Annex I.

An HT AP or an IDO STA that is also an HT STA should not start a 20 MHz BSS in 5 GHz on a channel that is the secondary channel of a 20/40 MHz BSS.

An AP may only start a 20/40 MHz BSS in 2.4 GHz if the value of the local variable 20/40 Operation Permitted is TRUE (see Equation (11-3)).

The AP or IDO STA may continue to periodically scan after the BSS has been started.

After starting a 20 MHz BSS, an FC HT AP 19 shall perform a minimum of dot11BSSWidthChannelTransitionDelayFactor overlapping BSS scans either itself or through its associated STAs, of the set of channels that are allowed operating channels within the current operational regulatory domain and whose center frequency falls within the 40 MHz affected channel range given by Equation (11-4) before making a transition from a 20 MHz BSS to a 20/40 MHz BSS. If no secondary channel has been selected, then all channels in the frequency band shall be scanned.

Note – an FC HT AP can change from operating a 20 MHz BSS to a 20/40 MHz BSS while maintaining associations by making a change to the transmitted value of the Secondary Channel Offset field.

Note to TGn editor and readers of the document – equation 11-4 is not very reproducible and so, does not appear in this document, but should remain unchanged by this document.

An FC HT AP 19 shall maintain a local Boolean variable 20/40 Operation Permitted that can have either the value TRUE or FALSE. The initial value of 20/40 Operation Permitted shall be FALSE. The value of 20/40 Operation Permitted is recomputed according to Equation (11-3) whenever a BSS channel width trigger event is detected or whenever a period of time has elapsed with no BSS channel width triggers being detected and no overlap being reported, as defined in 11.15.10 (Switching between 40 MHz and 20 MHz).

20/40 Operation Permitted = (P == OPi for all values of i) AND

(P == OTi for all values of i) AND

(S == OSi for all values of i)

When the value of OPi is the empty set, then the expression (P == OPi for all values of i) is defined to be TRUE.

When the value of OTi is the empty set, then the expression (P == OTi for all values of i) is defined to be TRUE.

When the value of OSi is the empty set, then the expression (S == OSi for all values of i) is defined to be TRUE.

Note – the use of “==” in the above expressions means that the values on either side of the “==” are to be tested for equality with a resulting Boolean value.

CID 5032, 5085

TGn Editor: Change what might be the 12th paragraph, depending on how you count, of subclause “11.9.8.3 Scanning requirements for a 20/40 MHz BSS” beginning on page 206 at about line 65 of TGn Draft D3.0, as follows:

An FC HT AP 19 may transmit a frame containing  a Secondary Channel Offset field set to a value of 1 or 3 only if 20/40 Operation Permitted is TRUE.

CID 5716, 5081

TGn Editor: Change the last paragraph of subclause “11.9.8.3 Scanning requirements for a 20/40 MHz BSS” beginning on page 207 at about line 9 of TGn Draft D3.0, as follows:

If, after initial establishment of the 20/40 MHz BSS, the value of 20/40 Operation Permitted is FALSE, then the FC HT AP 19 reverts to 20 MHz BSS operation (see 11.15.10 Switching between 40 MHz and 20 MHz). The FC HT AP 19 can subsequently move the BSS to a pair of channels where 20/40 Operation Permitted evaluates to TRUE.

CID 5080

TGn Editor: Insert the text “MHz” following the several occurrences of “25” found in the equation for “40 MHz affected channel range” that is found in subclause “11.9.8.3 Scanning requirements for a 20/40 MHz BSS” beginning on page 224 at about line 50 of TGn Draft D3.03:

CID 5032, 5085

TGn Editor: Change the last paragraph of subclause “11.15.1 Terminology and rules for operation in 20/40 MHz BSS” beginning on page 209 at about line 21 of TGn Draft D3.0, as follows:

An HT STA 19 that is a member of an IBSS shall set the Secondary Channel Offset field of the HT Information element and the Secondary Channel Offset element to a zero in transmitted frames.

CID 5870

TGn Editor: Change the 3rd paragraph of subclause “11.9.8.4 Channel management at the AP and in an IBSS” beginning on page 212 at about line 1 of TGn Draft D3.02, as follows:

In order to maintain existing associations and/or minimize disruption to communications with other STA while making a channel width change or while performing a channel pair relocation, an AP or DFS owner may inform HT STAs within the BSS that it is making the change by including an Extended Channel Switch Announcement element in Beacon, Probe Response, and Extended Channel Switch Announcement frame transmissions until the intended channel switch time. The New Channel Number of the Extended Channel Switch Announcement element represents the new channel in the case that the BSS after relocation/width change will be a 20 MHz BSS, or the primary channel of the new pair of channels in the case that the BSS after relocation/width change will be a 20/40 MHz BSS. When changing to a new pair of channels, the New Regulatory Class field specifies the position of the secondary channel relative to the new primary channel, i.e., either above or below.

CID 5320, 5717

TGn Editor: Add the following item to the bulleted list of items found between the fourth and fifth paragraphs of subclause “11.9.8.4 Channel management at the AP and in an IBSS” on page 212 at about line 27 of TGn Draft D3.02, while also changing the reference in the paragraph immediately preceding the bulleted list from “three elements” to “four elements”:

- Secondary Channel Offset element

TGn Editor: Change paragraph five of subclause “11.9.8.4 Channel management at the AP and in an IBSS” beginning on page 212 at about line 29 of TGn Draft D3.02, as follows:

The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements and transmitted Secondary Channel Offset elements to 1 if the Behavior Limit parameter of the selected row contains the value 13. The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements and transmitted Secondary Channel Offset elements to 3 if the Behavior Limit parameter of the selected row contains the value 14. The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements and transmitted Secondary Channel Offset elements to 0 if the Behavior Limit parameter of the selected row contains neither the value 13 nor the value 14. (#2583)

CID 5082

TGn Editor: Delete the last paragraph of subclause “11.9.8.4 Channel management at the AP and in an IBSS” on page 227 at about line 7 of TGn Draft D3.03, which begins with “An FC HT AP 19 shall maintain a record (known as the STA Channel Width record) of the STA Channel Width field value most recently received from each of its associated STA.”

CID 5011, 5182, 5085

TGn Editor: Change the contents of subclause “11.15.1 Terminology and rules for operation in 20/40 MHz BSS” beginning on page 213 at about line 23 of TGn Draft D3.02 as shown:

In 11.15 (20/40 MHz BSS Operation), the following terms are used:

— FC HT AP: an HT AP that included a value of 1 in the Supported Channel Width Set field (indicating its capability to operate on a 40 MHz channel) of its most recent transmission of a frame containing an HT Capabilities Information element.

— FC HT STA: an HT STA that included a value of 1 in the Supported Channel Width Set field (indicating its capability to operate on a 40 MHz channel) of its most recent transmission of a frame containing an HT Capabilities Information element.

— STA 17: a STA that is operating on a channel that belongs to any regulatory class that has a value of “20” or “40” for the entry in the column labeled “Channel Spacing (MHz)” and that has a value of “5” for the entry in the column labeled “Channel Starting Frequency (GHz)” of any of the tables found in Annex J.

— STA 19: a STA that is operating on a channel that belongs to any regulatory class that has a value of “25” or “40” for the entry in the column labeled “Channel Spacing (MHz)” and that has a value of “2.407” or “2.414” for the entry in the column labeled “Channel Starting Frequency (GHz)” of any of the tables found in Annex J.

- HT STA 17: an HT STA that is also a STA 17.

- HT STA 19: an HT STA that is also a STA 19.

- non-FC HT STA: a STA that is not an FC HT STA (#5138)

- FC HT AP 17: an HT AP 17 that is also an FC HT AP.

- FC HT STA 17: an HT STA 17 that is also an FC HT STA.

- FC HT AP 19: an HT AP 19 that is also an FC HT AP.

- FC HT STA 19: an HT STA 19 that is also an FC HT STA.

An FC HT STA shall set the 20/40 BSS Coexistence support field to 1 in transmitted Extended Capabilities elements.

An HT STA 19 that is a member of an IBSS shall set the Secondary Channel Offset field of the HT Information element and the Secondary Channel Offset element to zero in transmitted frames.

CID 5474, 5724, 5731, 5718, 5719, 5720, 5731, 5869, 5132, 5473, 5710

TGn Editor: Change the contents of subclause “11.15.2 Basic 20/40 MHz BSS functionality” beginning on page 213 at about line 62 of TGn Draft D3.02 as shown:

The following terms are used in this subclause to describe transmitted PPDU formats:

— 40 MHz HT is a Clause 20 transmission using FORMAT=HT_MF or HT_GF and

CH_BANDWIDTH= HT_CBW40

— 20 MHz HT is a Clause 20 transmission using FORMAT=HT_MF or HT_GF and

CH_BANDWIDTH=HT_CBW20

- DSSS/CCK is a Clause 15 or Clause 18 transmission

An HT AP declares its channel width capability (20 MHz only or 20/40 MHz) in the Supported Channel Width Set field of the HT Capabilities element. (#3331)

An HT AP shall set the STA Channel Width field to 1 in frames in which it has set the Secondary Channel Offset field to 1 or 3. An HT AP shall set the STA Channel Width field to 0 in frames in which it has set the Secondary Channel Offset field to 0.

A non-AP HT STA declares its channel width capability (non-FC HT STA or FC HT STA(#715, 5138)) in the Supported Channel Width Set in the HT Capabilities element.

Note – A 20/40 MHz BSS can include any mixture of FC HT STAs, non-FC(#715) HT STAs and non-HT STAs (#263). Protection requirements for mixed networks are defined in 9.13.

Note - A non-AP HT STA can switch between FC HT STA and non-FC STA(#715) operation by disassociation and association or reassociation.

An HT STA shall not transmit a 20 MHz PPDU containing one or more data MPDUs using the secondary channel of a 20/40 MHz BSS.. The Notify Channel Width Action (#1096) frame may be used by a non-AP STA to notify another STA that the STA wishes to receive frames in the indicated channel width.

(#2592)

An HT STA that is a member of an IBSS adopts the value of the Secondary Channel Offset field in received frames according to the rules in 11.1.4 and shall not transmit a value for the Secondary Channel Offset field that differs from the most recently adopted value.

An HT STA that is a member of an IBSS and that has a value of true for dot11FortyMHzOptionImplemented shall use the new channel selection and advertising procedures defined in subclause 11.9.7.

(#265)

CID 5474, 5892, 5722, 5723, 5724, 5088, 5721

TGn Editor: Change the contents of subclause “11.15.3 40 MHz PPDU transmission restrictions” beginning on page 214 at about line 46 of TGn Draft D3.02 as shown:

Several fields from various frames are used to convey information between STAs regarding the support for 40 MHz PPDU transmission and reception and regarding any current prohibition against the  transmission and reception of 40 MHz PPDUs.

The following rules describe the behavior that accompanies those fields.

The fields that are used to determine the status of the transmission and reception of 40 MHz  PPDUs are as follows:

— The Supported Channel Width Set field of the HT Capabilities element

— The Secondary Channel Offset field of the HT Information element

— The STA Channel Width field of the HT Information element

— The STA Channel Width field of the Notify Channel Width action frame

— The Extended Channel Switch Announcement element

The Supported Channel Width Set field is used to indicate whether or not the transmitting STA is capable of transmitting and receiving 40 MHz PPDUs.

NOTE—The Supported Channel Width Set field transmitted by an AP is constant for the lifetime of its BSS as it is a parameter of the MLME-START.request primitive.

A STA that is associated with an AP shall not transmit a value for the Supported Channel Width Set field that differs from a previously transmitted value unless the STA first disassociates or is disassociated (#5139) from its BSS.

The Secondary Channel Offset field is used to indicate whether or not the BSS is occupying a 40 MHz wide pair of channels, and when a secondary channel exists, whether it is above or below the primary channel in frequency. The Extended Channel Switch Announcement action frame and the Extended Channel Switch Announcement element can each be used to indicate a transition from 20/40 MHz BSS operation to 20 MHz BSS operation and vice versa, and to indicate whether a secondary channel, when it exists, is above or below the primary channel in frequency.

An FC HT STA shall maintain a local Boolean variable 40MHzRegulatoryClass as described here. The initial value of 40MHzRegulatoryClass shall be FALSE. The value of 40MHzRegulatoryClass is recomputed according to the rules in this subclause at every TBTT and following the reception of a frame transmitted by the AP associated with the STA when that frame contains either of the following fields:

Current Regulatory Class field

New Regulatory Class field

The local Boolean variable 40MHzRegulatoryClass becomes TRUE upon reception of a frame transmitted by the associated AP if the frame contained a Current Regulatory Class field with a value that corresponds to a regulatory class that corresponds to a Channel Spacing value of 40 MHz, as specified in Annex J.

The local Boolean variable 40MHzRegulatoryClass becomes FALSE upon reception of a frame transmitted by the associated AP if the frame contained a Current Regulatory Class field with a value that corresponds to a regulatory class that does not correspond to a Channel Spacing value of 40 MHz.

The local Boolean variable 40MHzRegulatoryClass becomes TRUE at the nth TBTT upon reception of a frame transmitted by the associated AP that contains an Extended Channel Switch Announcement element with a value of n in the Channel Switch Count field and a value in the New Regulatory Class field that corresponds to a regulatory class that corresponds to a Channel Spacing value of 40 MHz.

The local Boolean variable 40MHzRegulatoryClass becomes FALSE at the nth TBTT upon reception of a frame transmitted by the associated AP that contains an Extended Channel Switch Announcement element with a value of n in the Channel Switch Count field and a value in the New Regulatory Class field that corresponds to a regulatory class that does not correspond to a Channel Spacing value of 40 MHz.

A STA can choose to dynamically constrain its operating channel width to 20MHz while being a member of a 20/40 MHz BSS. Transitions to and from this constrained state are indicated using the transmission of a frame that carries theSTA Channel Width field. A transmitted value of zero in the STA Channel Width field indicates that the transmitting STA is not currently able to receive 40 MHz PPDUs, beginning at the end of the transmission of the frame that contained the STA Channel Width field.

A STA shall not transmit a frame containing a STA Channel Width field with a non-zero value if the value of its most recently transmitted Supported Channel Width Set field is zero.

A STA that is associated with an infrastructure BSS (STA1) shall not transmit a 40 MHz PPDU to a STA (STA2) unless the following three conditions are true:

a) The Supported Channel Width Set field of the HT Capabilities element of both STAs is set to a nonzero value

b) The Secondary Channel Offset field of the most recently received HT Information element sent by the AP of the BSS has a value of 1 or 3

c) The local Boolean variable 40MHzRegulatoryClass is TRUE.

If the above three conditions are met, STA1 should not transmit a 40 MHz PPDU to STA2 unless the following two conditions are true:

a) Either, STA1 has not received a Notify Channel Width action frame that was transmitted by STA2, or the STA Channel Width field of the most recently received Notify Channel Width action frame at STA1 that was transmitted by STA2 is non-zero

b) The STA Channel Width field of the most recently received HT Information element that was transmitted by STA2 is non-zero

An AP shall not transmit a 40 MHz PPDU to another STA unless the following three conditions are true:

a) The Supported Channel Width Set field of the HT Capabilities element of the AP and the STA are set to a non-zero value

b) The Secondary Channel Offset field of the AP's most recently transmitted HT Information element sent has a value of 1 or 3

c) The local Boolean variable 40MHzRegulatoryClass is TRUE.

If the above three conditions are met, the AP should not transmit a 40 MHz PPDU to another STA unless either, the AP has not received a Notify Channel Width action frame that was transmitted by the STA, or the STA Channel Width field of the most recently received Notify Channel Width action frame at the AP that was transmitted by the STA, is non-zero

An AP shall not transmit a 40 MHz PPDU with a group Address 1 value unless the following three conditions are true:

a) The Supported Channel Width Set field of the HT Capabilities element of the AP is set to a non-zero value

b) The Secondary Channel Offset field of the AP's most recently transmitted HT Information element sent has a value of 1 or 3

c) The local Boolean variable 40MHzRegulatoryClass is TRUE.

If the above three conditions are met, the AP should not transmit a 40 MHz PPDU with a group Address 1 value if the most recently received Notify Channel Width action frame for any of the STA associated with the AP is non-zero.

An HT STA 19 that is not a member of an infrastructure BSS shall not transmit a 40 MHz Mask PPDU.

An HT STA 17 that is not associated with an infrastructure BSS (STA1) shall not transmit a 40 MHz PPDU to a STA (STA2) unless the following three conditions are true:

a) The Supported Channel Width Set field of the HT Capabilities element of both STAs is set to a nonzero value

b) The Secondary Channel Offset field of the most recently received HT Information element sent by STA2 has a value of 1 or 3

c) The Secondary Channel Offset field of the most recently transmitted HT Information element sent by STA1 has a value of 1 or 3

If the above three conditions are met, STA1 should not transmit a 40 MHz PPDU to a STA2 unless the STA1 has not received a STA Channel Width field that was transmitted by STA2, or the value of the most recently received STA Channel Width field at STA1 that was transmitted by STA2, is non-zero.

An HT STA 17 that is not associated with an infrastructure BSS (STA1) shall not transmit a 40 MHz PPDU with a group Address 1 value unless the following two conditions are true:

a) The Supported Channel Width Set field of the HT Capabilities element most recently transmitted by the transmitting STA was set to a nonzero value

b) The Secondary Channel Offset field of the most recently transmitted HT Information element sent by STA1 had a value of 1 or 3

If the above two conditions are met, STA1 should not transmit a 40 MHz PPDU with a group Address 1 value if the most recently received STA Channel Width field for each other known member of the BSS of which STA1 is a member is non-zero.

CID 5091, 5096, 5098, 5090, 5097

TGn Editor: Change the contents of subclause “11.15.4 Scanning requirements for 40 MHz capable STA” beginning on page 216 at about line 29 of TGn Draft D3.02 as shown:

An overlapping BSS scan operation is a passive or active scan of a set of channels that are potentially affected by 20/40 MHz BSS operation. Each channel in the set may be scanned more than once during a single Overlapping BSS scan operation. Overlapping BSS scans are performed by FC HT STA 19. FC HT STA 17 are not required to perform overlapping BSS scan operations.

During an individual scan within an overlapping BSS scan operation, the per-channel scan duration is a minimum of dot11OBSSScanPassiveDwell TU when scanning passively and a minimum of dot11OBSSScanActiveDwell TU when scanning actively. During an overlapping BSS scan operation, each channel in the set is scanned at least once per dot11BSSWidthTriggerScanInterval seconds and the minimum total scan time (i.e., the sum of the scan durations) per channel within a single overlapping BSS scan operation is dot11BSSWidthTriggerScanInterval seconds is dot11OBSSScanPassiveTotalPerChannel TU for a passive scan and dot11OBSSScanActiveTotalPerChannel TU for an active scan. (#5126)

NOTE—The values provided in the previous paragraph indicate the minimum requirements. In some cases, not all minimum values can be achieved simultaneously.

When an AP transmits an Overlapping BSS Scan Parameters element, the values of each of the fields of the element shall be no less than the associated MIB variable minimum values and the field values shall be no greater than the associated MIB variable maximum values. Upon transmission of a frame containing an Overlapping BSS Scan Parameters element, an AP shall update the values of its MIB variables that are used during overlapping BSS scanning operations and 20/40 MHz BSS switching operations according to the mapping between the frame fields and MIB variables as defined in 7.3.2.55 (Overlapping BSS Scan Parameters element).

Upon receipt of a frame containing an Overlapping BSS Scan Parameters element from the AP with which an FC HT STA 19 has an active association, the receiving FC HT STA 19 MLME shall update each of the values of the MIB variables used during overlapping BSS scanning operations according to the mapping between the frame fields and MIB variables as defined in 7.3.2.55 (Overlapping BSS Scan Parameters element).

(#5127)

An FC HT AP 19 may transmit frames containing an Overlapping BSS Scan Parameters element to any or all associated STAs in order to provide overlapping BSS scan parameter values that are different from the default values.

An FC HT STA 19 that is associated with an FC HT AP 19 shall perform at least one OBSS scan every dot11BSSWidthTriggerScanInterval seconds, unless the FC HT STA 19 satisfies the conditions described in 11.15.4a.

CID 5097

TGn Editor: Insert a new subclause 11.15.4a to follow “11.15.4 Scanning requirements for 40 MHz capable STA”, where the end of 11.15.4 is found on page 217 at about line 6 of TGn Draft D3.02:

11.15.4a Exemption from OBSS Scanning

An FC HT STA 19 shall maintain a local variable ActivityFraction. The value of ActivityFraction is equal to the total duration of transmitted MSDUs and received individually addressed MSDUs during the previous dot11BSSWidthChannelTransitionDelayFactor * dot11BSSWidthTriggerScanInterval seconds.

An FC HT STA 19 may transmit a 20/40 BSS Coexistence Management Action frame with the Scanning Exemption Request field in 20/40 Coexistence Element set to 1 to its associated AP.

If the last 20/40 BSS Coexistence Management Action frame received by an FC HT STA 19 in an individually addressed frame from its associated AP has the Scanning Exemption Grant field set to 1, the STA is exempted from scanning whenever the value of its local variable ActivityFraction is less than dot11OBSSScanActivityThreshold/10000.

An FC HT AP 19 shall not transmit a 20/40 BSS Coexistence Management Action frame with the Scanning Exemption Grant field set to 1 addressed to an FC HT STA if the following condition is true:

- The FC HT STA has transmitted one or more channel report elements and is the only STA in the BSS that has indicated one or more channels on which a STA has found conditions that disallow the use of a 20/40 MHz BSS.

If there is more than one FC HT STA in the BSS that has indicated conditions that disallows the use of 20/40 MHz BSS on a specific channel then:

- If all the FC HT STAs that have indicated unavailability of a channel have also requested to be exempt from scaning,  the AP shall disallow atleast one of the FC HT  STA to be exempt from scanning

- If there is atleast one FC HT STA from the group of FC HT STAs that have indicated unavailability of a channel and that has not requested to be exempt from scanning, the AP may allow all the STAs that have requested to be exempt from scanning to be exempted from scanning

TGn Editor: Modify the text of the bullet item within subclause “S.4.3 Monitoring channels for other BSS operation” on page 530 at about line 31 of TGn Draft D3.02 as shown:

S.4.3 Monitoring channels for other BSS operation

— HT STAs that are exempt from scanning are specified in 11.15.4a

CID 5747

TGn Editor: Change the contents of the fourth paragraph of subclause “11.17 BSS Coexistence Management frame usage” beginning on page 224 at about line 31 of TGn Draft D3.02 as shown:

A STA shall not transmit to another STA a 20/40 BSS Coexistence Management frame with an individual (#5255) address in the Address 1 (#5746) field if the most recently received Extended Capabilities element from the recipient STA contained a value of 0 in the 20/40 BSS Coexistence support field. A STA that transmits a 20/40 BSS Coexistence Management frame may set the Address 1 field to a group (#5225) address  (#5746).(#75)

CID 5111

TGn Editor: Delete the contents of the fifth paragraph of subclause “11.17 BSS Coexistence Management frame usage” beginning on page 224 at about line 38 of TGn Draft D3.02 as shown:

CID 5748, 5807, 5749, 5750

TGn Editor: Change the contents of the seventh and eighth paragraphs of subclause “11.17 BSS Coexistence Management frame usage” beginning on page 224 at about line 60 of TGn Draft D3.02 as shown:

A STA may transmit a 20/40 BSS Coexistence Management frame that contains a value of 1 for the Request Information field to another STA that supports the transmission of and reception of the 20/40 BSS Coexistence Management frame, except when the frame is a response to a 20/40 BSS Coexistence Management frame that contains a value of 1 for the Request Information field.

(#2131, moved from 7.4.9.10) A STA that receives a 20/40 BSS Coexistence element (#75) with the Information Request field set to 1 shall immediately queue for transmission, a 20/40 BSS Coexistence Management (#75) (#1096, 2617) frame with the transmitting STA as the recipient.

CID 5132, 5473, 5710

TGn Editor: Within “Table 7-8 Beacon frame body” remove the row that contains “Secondary Channel Offset” in the Information column, in TGn draft D3.02 on page 36 at about line 63.

TGn Editor: Within “Table 7-15 Probe response frame body” remove the row that contains “Secondary Channel Offset” in the Information column, in TGn draft D3.02 on page 40 at about line 20.

TGn Editor: Move the entirety of subclause “11.9.8 Channel selection methods for 20/40 MHz BSS operation” and all of its subclauses which begin on page 209 of TGn draft 3.02, to a location that is just ahead of subclause “11.15.3 40 MHz PPDU transmission restrictions” at about line 44 of page 214 of TGn Draft D3.02 and correct any editing instructions associated with the relevant subclauses and adjust clause numbering and references as appropriate and modify the first sentence of the current 11.9.8.1 so that the reference in that sentence now lists subclause range 11.15 to 11.16 instead of the current 11.9.8.1 to 11.16.1.

TGn Editor: Insert the following text to appear just after subclause “11.9.6 Requesting and reporting of measurements” on page 209 at about line 31 of TGn draft D3.02:

11.9.7 Selecting and advertising a new channel

11.9.7.2 Selecting and advertising a new channel in an IBSS

Change paragraph four of 11.9.7.2 as follows:

A STA at which an IBSS is started shall be a DFS owner in that IBSS. That STA shall include its MAC address in the DFS Owner field of the IBSS DFS element and DFS Recovery Interval field from the MLME.START.request parameter. The purpose of the DFS owner is to coordinate a channel switch when required. All STAs within a spectrum-managed IBSS shall have the ability to become DFS owner. All HT STAs with dot11FortyMHzOptionImplemented set to true within an IBSS shall have the ability to become DFS owner.

CID 5717

TGn Editor: Change the contents of the “Behavior limits set” column for the first two rows of table “Table I.3 Behavior limits sets” found in TGn draft D3.02 on page 485 at about line 13 as shown:

13 Reserved 20/40 MHz BSS Primary channel with Secondary channel above the Primary channeland also or 20 MHz BSS primary channel operated by an FC HT AP and also 20 MHz operational channel for a non- AP STA when the non-AP STA is associated with an FC HT AP.

14 Reserved 20/40 MHz BSS Primary channel with Secondary channel below the Primary channel or 20 MHz BSS primary channel operated by an FC HT AP and also 20 MHz operational channel for a non-AP STA when the non-AP STA is associated with an FC HT AP.

TGn Editor: Change the note at the bottom of the table “Table J.1 7-8 Regulatory classes in the USA” found in TGn draft D3.02 on page 487 at about line 47 as shown:

NOTE—The Channel Spacing for Regulatory Classes 22-33 is for the supported bandwidth rather than the operating bandwidth. In these regulatory classes, the AP operates either a 20/40 MHz BSS(#715) or a 20 MHz BSS, and the operating bandwidth for a non-AP STA is either 20 MHz or 40 MHz.

CID 5112

TGn Editor: Change the last sentence of the first paragraph of subclause “T.3.2 establishing a 20/40 MHz BSS” found in TGn draft D3.03 on page 549 at about line 15 as shown:

The particulars of overlapping BSS scanning are controlled by the following MIB attributes:

CID 5792

TGn Editor: Change the fourth bulleted item in the list of bulleted items that immediately follows the fifth paragraph of subclause “T.3.3 Monitoring channels for other BSS operation” found in TGn draft D3.03 on page 550 at about line 51, as shown:

  - HT STAs that are associated with an AP whose BSS is operating on a channel that is not in the 2.4 GHz band

References: