IEEE P802.11
Wireless LANs

| Normative text for cipher suites value usage, SSPN interactions, etc. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hong Cheng | Panasonic | BLK1022 TaiSeng Ave. #06-3530 Singapore 534415 | +65-65505477 | hong.cheng@sg.panasonic.com |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Colin Blanchard | BT Group | BT Design MLB1 PP8 Adastral Park Ipswich IP3 5RE | +44 01473 605353 | Colin.blanchard@bt.com |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

This submission resolves comment 780, 790, 835, 1681, 1867, 1999, 2002 from 802.11 LB#107 related to the use of SSPN interface for authentication, authorization, and service control.

The text of this submission is based on P802.11u-D1.0, P802.11u-D1.02 and IEEE 802.11-2007.

# Introduction

LB#107 CID 780 requests for a normative description of the use of Cipher Suites information in the non-AP STA connection control. And, CID 1681 requests for a description of the interface between the AP and SSPN, and how the AP reflects that for the non-AP STA. The following text provides the necessary details for such requests.  CID 1867 requests for a description of how the reason code is communicated to the AP. CIDs 790&835 request to clarify that the cipher suites in the MIB is only for unicast frames. CIDs 1999 & 2002 request to support the command mode over the SSPN interface to enforce SSPN decisions.

# Editing Instructions

6.2.1 MAC data services

6.2.1.2 MA-UNITDATA.indication

Editor: Amend the text in the following clause as shown:

6.2.1.2.4 Effect of receipt

The effect of receipt of this primitive by the LLC sublayer is dependent on the content of the MSDU.

At an AP, if the source MAC address corresponds to a non-AP STA for which the AP has an dot11InterworkingEntry and the priority is an integer in the range of 0 to 7 inclusive, then the access category shall be derived from the priority using the mapping in Table 9-1 and the AP shall update the following MIB variables in the non-AP STa’s dot11InterworkingEntry:

- If the access category is AC_VO, then dot11NonApStaVoiceFrameCount shall be incremented by 1 and dot11NonApStaVoiceOctetCount shall be incremented by the number of octets in the data.

- If the access category is AC_VI, then dot11NonApStaVideoFrameCount shall be incremented by 1 and dot11NonApStaVideoOctetCount shall be incremented by the number of octets in the data

- If the access category is AC_BE, then dot11NonApStaBestEffortFrameCount shall be incremented by 1 and dot11NonApStaBestEffortOctetCount shall be incremented by the number of octets in the data

- If the access category is AC_BK, then dot11NonApStaBackgroundFrameCount shall be incremented by 1 and dot11NonApStaBackgroundOctetCount shall be incremented by the number of octets in the data

At an AP, if the source MAC address corresponds to a non-AP STA for which the AP has an dot11InterworkingEntry and the priority is an integer in the range of 8 to 15 inclusive, then the AP shall update the following MIB variables in the non-AP STa’s dot11InterworkingEntry:

- dot11NonApStaHCCAFrameCount shall be incremented by 1, and

- dot11NonApStaHCCAOctetCount shall be incremented by the number of octets in the data.

An AP is responsible for enforcing authorized access limits from an SSPN.  These limits are stored in the dot11InterworkingTable based on information received over the SSPN interface.

6.2.1.3 MA-UNITDATA.confirm

Editor: Amend the text in the following clause as shown:

6.2.1.3.4 Effect of receipt

The effect of receipt of this primitive by the LLC sublayer is dependent upon the type of operation employed

by the LLC sublayer entity.

At an AP, if the source MAC address corresponds to a non-AP STA for which the AP has an dot11InterworkingEntry and the provided priority is an integer in the range of 0 to 7 inclusive, then the access category shall be derived from the provided priority using the mapping in Table 9-1 and the AP shall update the following MIB variables in the non-AP STa’s dot11InterworkingEntry:

- If the access category is AC_VO, then dot11NonApStaVoiceFrameCount shall be incremented by 1 and dot11NonApStaVoiceOctetCount shall be incremented by the number of octets in the data.

- If the access category is AC_VI, then dot11NonApStaVideoFrameCount shall be incremented by 1 and dot11NonApStaVideoOctetCount shall be incremented by the number of octets in the data

- If the access category is AC_BE, then dot11NonApStaBestEffortFrameCount shall be incremented by 1 and dot11NonApStaBestEffortOctetCount shall be incremented by the number of octets in the data

- If the access category is AC_BK, then dot11NonApStaBackgroundFrameCount shall be incremented by 1 and dot11NonApStaBackgroundOctetCount shall be incremented by the number of octets in the data

At an AP, if the source MAC address corresponds to a non-AP STA for which the AP has an dot11InterworkingEntry and the provided priority is an integer in the range of 8 to 15 inclusive, then the AP shall update the following MIB variables in the non-AP STa’s dot11InterworkingEntry:

- dot11NonApStaHCCAFrameCount shall be incremented by 1, and

- dot11NonApStaHCCAOctetCount shall be incremented by the number of octets in the data.

An AP is responsible for enforcing authorized access limits from an SSPN.  These limits are stored in the dot11InterworkingTable based on information received over the SSPN interface.

Editor: Amend the text in the following clause as shown:

9.2.7 Broadcast and multicast MPDU transfer procedure

In the absence of a PCF, when broadcast or multicast MPDUs are transferred from a STA with the To DS field clear, only the basic access procedure shall be used. Regardless of the length of the frame, no RTS/CTS exchange shall be used. In addition, no ACK shall be transmitted by any of the recipients of the frame. Any broadcast or multicast MPDUs transferred from a STA with a To DS field set shall, in addition to conforming to the basic access procedure of CSMA/CA, obey the rules for RTS/CTS exchange and the ACK procedure because the MPDU is directed to the AP.

Subsequent to an AP receiving a broadcast/multicast frame and before forwarding it, the AP shall determine if the frame was received from a STA for which the AP has an dot11InterworkingEntry .  If so, it shall only forward the frame if the dot11NonApStaAuthSourceMulticast in the STA’s dot11InterworkingEntry is TRUE, otherwise the frame shall be dropped..  If dot11NonApStaAuthSourceMulticast is TRUE, then the AP shall measure the aggregate datarate of all multicast streams sourced from that non-AP interworking-capable STA and drop any frames causing the dot11NonApStaMaxAuthSourceMulticastRate to be exceeded in any dot11EDCAAveragingPeriod if dot11QosOptionImplemented is TRUE or 1 second if dot11QosOptionImplemented is FALSE.

Subsequent to the above policy verification, the broadcast/multicast message shall be distributed into the BSS. The STA originating the message shall receive the message as a broadcast/multicast message. Therefore, all STAs shall filter out broadcast/multicast messages that contain their address as the source address. Broadcast and multicast MSDUs shall be propagated throughout the ESS.

10 MLME

10.3 MLME SAP interface

10.3.24 TS management interface

10.3.24.2 MLME-ADDTS.confirm

Editor: Amend the text in the following clause as shown; the changes in this clause are based upon P802.11u-d1.02:

10.3.24.2.2 Semantics of the service primitive

For values of ResultCode other than SUCCESS, no new TS has been created. In the case of REJECTED_WITH_SUGGESTED_CHANGES, the TSPEC represents an alternative proposal by the HCbased on information about the current status of the MAC entity. In the case of REJECTED_HOME_WITH_SUGGESTED_CHANGES, the TSPEC represents an alternative proposal by the HC based on information received across the SSPN interface. A TS is not created with this definition. If the suggested changes are acceptable to the non-AP STA, it is the responsibility of the non-AP STA to set up the TS with the suggested changes.

11.4.4 TS setup

11.4 TS Operation

Editor: Amend the text in the following clause as shown; the changes in this clause are based upon P802.11u-d1.02:

11.4.4 TS setup

The SME in the HC decides whether to admit the TSPEC as specified, refuse the TSPEC, or not admit but suggest an alternative TSPEC. If the TSPEC is received from a non-AP STA for which the AP has SSPN policy in the STA’s dot11InterworkingEntry, the HC shall use the STA’s policy in the decision to admit or deny the request as described in the following paragraph.  The SME then generates an MLME-ADDTS.response primitive containing the TSPEC and a ResultCode value. The contents of the TSPEC and Status fields contain values specified in 10.3.24.4.2.

When the AP’s HC receives a TSPEC, the AP shall inspect it to determine the requested access policy, user priority and maximum datarate.

- For a TS to be admitted when the requested access policy is set to EDCA, both of the following shall be true:

  - The access category shall be determined from the user priority according to Table 9-1.  The bit corresponding to this access category in dot11NonApStaAuthAccessCategories from the non-AP STA’s dot11InterworkingEntry is set to 1.

  - The sum of the datarate of all active TSs in this access category plus the maximum datarate in the TSPEC shall be less than or equal to the non-AP STA’s dot11InterworkingEntry for dot11NonApStaAuthMaxVoiceRate, dot11NonApStaAuthMaxVideoRate, dot11NonApStaAuthMaxBestEffortRate, or dot11NonApStaAuthMaxBackgroundRate depending on whether the derived access category is voice, video, best effort of background respectively.

- For a TS to be admitted when the requested access policy is set to HCCA, all of the following shall be true:

  - The dot11NonApStaAuthHCCA value shall be set to TRUE.

  - The sum of the datarate of all active TSs having access policy set to HCCA plus the maximum datarate in the TSPEC shall be less than or equal to dot11NonApStaAuthMaxHCCARate in the non-AP STA’s dot11InterworkingEntry.

  - The delay bound which will be provided by the HC in the TSPEC response shall be less than or equal to dot11NonApStaAuthHCCADelay in the non-AP STA’s dot11InterworkingEntry..

The HC MAC transmits an ADDTS Response frame containing this TSPEC and status. The encoding of the

ResultCode values to Status Code field values is defined in Table 11-2.  If the requesting non-AP STA is an interworking capable STA for which the AP has a dot11InterworkingEntry, then the HC shall set the dot11nonApStaAddtsResultCode from this entry to a value equal to the ResultCode.

If dot11InterworkingServiceImplemented is true, the decision to admit the TSPEC or refuse it is based on both the available capacity as well as authorization information from the SSPN interface in the dot11InterworkingTable. The HC shall refuse to admit a TSPEC requesting service at a higher priority than authorized the non-AP STA’s entry in the dot11InterworkingTable. When a STA requests service at a higher priority than authorized, the HC may optionally provide a suggested TSPEC with a data rate and lower priority that would be authorized. Usage of the TSPEC in an Interworking environment is described in Annex K (Admission Control Operation).

Add the following subclause to 11.10:

### 11.10.4 Interworking Procedures: Interactions with SSPN

To provide interworking services, the IEEE802.11 AN interacts with the SSPN associated  with the user) of the non-AP STA either directly or via a roaming relationship.  As part of setting up the layer two security association (e.g., RSN), user policies are communicated to and stored in the AP.  They are used for controlling the service provision to the non-AP STA. The information from the SSPN affects the higher layer (i.e. above the MAC layer) authentication, authorization, and admission control decision at the AP. In addition, the AP collects statistic information about the non-AP STA and reports to the SSPN when requested. The SSPN may also send service provision instructions to the AP, e.g. to terminate the connection to a non-AP STA.

It is assumed that the AP and the server in SSPN have:

1) A trustworthy channel that can be used to exchange information, without exposure to or influence by any intermediate parties.

2) Suport for Access Control Lists (ACL’s) at each end of the channel that can be used to allow/disallow use of specific commands at the SSPN interface.

The establishment of this secure connection between the IEEE802.11AN and the SSPN is out of scope of this standard

11.10.4.1 Authentication and cipher suites selection with SSPN

When accessing the Interworking Service with a SSPN, the STA shall follow the procedure defined in subclause 8.1.3 to establish the RSNA in an ESS. The non-AP STA obtains information about the AP through normal beacon or Probe Response frames, or makes a more sophiscated query and selection using the GAS mechanism defined in subclause 11.10.1.

The non-AP STA invokes open system authentication, and starts the association process as described in clauses 8.4.2 and 8.4.3. The non-AP STA indicates all the cipher suites it supports, which are announced by the AP in the Beacon or Probe Response frames, in the RSN information element of the MLME-ASSOCIATE.request or MLME-REASSOCIATE.request. The AP obtains the cipher suites information from the corresponding MLME-ASSOCIATE.indication or MLME-REASSOCIATE.indication primitive.

Upon successful association, the non-AP STA initiates IEEE 802.1X authentication, as defined in clause 8.4.6. In the Interworking case, the EAP messages are forwarded to the SSPN based on the home realm information provided by the non-AP STA. Note that the identity of the non-AP STA may be obfuscated during the process to protect location privacy. If the IEEE 802.11 AN is unable to forward the EAP message, the AP shall disassociate the non-AP STA with Reason Code 47.

In addition to the EAP messages, the IEEE 802.11 AN also provides extra information regarding the non-AP STA to the SSPN as defined in Annex P.3.1, e.g. the Cipher Suite supported by non-AP STA, non-AP STA location information, etc. Such information is necessary for the SSPN to make authentication and service provisioning decisions.

The authentication result is carried over the SSPN interface to the AP. Upon success, keying materials are also delivered to the AP for the establishment of PMKSA with the non-AP STA. Clause 8.4.8 provides the RSNA key management procedures, which includes the 4-way Handshake process.

In the Interworking Service, the SSPN uses more information than is carried over EAP to decide on the authentication result. The SSPN may reject a connection if the cipher suites supported by non-AP STA does not meet its security requirements. The SME of the AP shall invoke a disassociation procedure as defined in 11.3.2.7 by issuing the MLME-DISASSOCIATE.request primitive. The AP dissassociates the corresponding non-AP STA with Reason Code 48.

The SSPN may reject the association request based on the location of the non-AP STA, e.g. if the non-AP STA is located in a forbidden zone. The SME of the AP shall invoke a disassociation procedure as defined in 11.3.2.7 by issuing the MLME-DISASSOCIATE.request primitive. The AP dissassociates the corresponding non-AP STA with Reason Code 49.

Replace the following section with the indicated text:

Replace text in the following subclause:

11.10.4.2 Reporting and Session Control with SSPN

If the SSPN has supplied resource utilization limits, the AP SME shall enforce those limits by comparing the values of octet counters to authorized access limits:

- dot11NonApStaVoiceOctetCount is compared to dot11NonApStaAuthMaxVoiceOctetCount.  When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

- dot11NonApStaVideoOctetCount is compared to dot11NonApStaAuthMaxVideoOctetCount.  When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

- dot11NonApStaBestEffortOctetCount is compared to dot11NonApStaAuthMaxBestEffortOctetCount.  When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

- dot11NonApStaBackgroundOctetCount is compared to dot11NonApStaAuthMaxBackgroundOctetCount.  When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

- dot11NonApStaHCCAOctetCount is compared to dot11NonApStaAuthMaxHCCAOctetCount.  When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

- The sum of dot11NonApStaVoiceOctetCount, dot11NonApStaVideoOctetCount, dot11NonApStaBestEffortOctetCount, dot11NonApStaAuthMaxBackgroundOctetCount, and dot11NonApStaHCCAOctetCount is compared to dot11NonApStaAuthMaxTotalOctetCount. When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

When a non-AP STA has exceeded the resource limits listed above, the AP terminates the connection. The AP SME invokes the disassociation procedure defined in 11.3.2.7 by issuing a MLME-DISASSOCIATE.request with the reason code of Authorized Access Limit Reached.

Change the title number of the following subclause:

# Annex K

# (Informative)

# Admission Control

## K.2.1 Use of ACM (admission control mandatory) subfield

### K.3.2 Guidelines for deriving service schedule parameters

Change the subcaluse as indicated below:

P.3.1.4 Link Layer Encryption Method

This element indicates the link layer encryption method selected during the RSNA establishment process for protecting the unicast communication between the non-AP STA and the AP. The cipher suite format of this element is drawn from the RSN information element defined in clause 7.3.2.25. AP obtains this information about the STA via the MLME SAP.

In the Interworking Service, the SSPN also participates in the selection of the cipher suite selection, as described in clause 11.10.4.1. Therefore, the link layer encryption method selected must meet or exceed the security requirement of the SSPN.

Note: In interworking, the SSPN may require visibility and configurability of the STA access.

With this information available to the SSPN, the operator would be able to have better control, e.g. barring access to IEEE 802.11 networks if NULL encryption is used. This also related to the operator network’s configuration , e.g. if pre-Authentication should be supported.

The AP stores the information in the corresponding dot11NonApStaCipherSuite element of its MIB.

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

Editing instruction: amend the following text as shown:

Dot11InterworkingEntry ::=

SEQUENCE {

dot11NonApStaMacAddress                    MacAddress,

dot11NonApStaUserIdentity                  SnmpAdminString,

dot11NonApStaInterworkingCapability        BITS,

dot11NonApStaAssociatedSSID	        OCTET STRING,

dot11NonApStaCipherSuite                   OCTET STRING,

dot11NonApStaAuthAccessCategories          BITS,

dot11NonApStaAuthMaxVoiceRate	        Unsigned32,

dot11NonApStaAuthMaxVideoRate              Unsigned32,

dot11NonApStaAuthMaxBestEffortRate         Unsigned32,

dot11NonApStaAuthMaxBackgroundRate         Unsigned32,

dot11NonApStaAuthMaxVoiceOctetCount        Unsigned32,

dot11NonApStaAuthMaxVideoOctetCount        Unsigned32,

dot11NonApStaAuthMaxBestEffortOctetCount   Unsigned32,

dot11NonApStaAuthMaxBackgroundOctetCount   Unsigned32,

dot11NonApStaAuthMaxHCCAOctetCount        Unsigned32,

dot11NonApStaAuthMaxTotalOctetCount        Unsigned32,

dot11NonApStaAuthHCCA                      TruthValue,

dot11NonApStaAuthMaxHCCARate               Unsigned32,

dot11NonApStaAuthSinkMulticast             TruthValue,

dot11NonApStaMaxAuthSourceMulticast        TruthValue,

dot11NonApStaMaxAuthSourceMulticastRate    Unsigned32,

dot11NonApStaVoiceFrameCount               Counter32,

dot11NonApStaVideoFrameCount               Counter32,

dot11NonApStaBestEffortFrameCount          Counter32,

dot11NonApStaBackgroundFrameCount          Counter32,

dot11NonApStaHCCAFrameCount                Counter32,

dot11NonApStaVoiceOctetCount               Counter32,

dot11NonApStaVideoOctetCount               Counter32,

dot11NonApStaBestEffortOctetCount          Counter32,

dot11NonApStaBackgroundOctetCount          Counter32,

dot11NonApStaHCCAOctetCount                Counter32,

dot11NonApStaPowerManagementMode           INTEGER,

dot11nonApStaAuthDls                       TruthValue,

dot11nonApStaVlanId                        Unsigned32,

dot11nonApStaVlanName                      OCTET STRING,

dot11nonApStaAddtsResultCode               INTEGER

}

dot11NonApStaAuthAccessCategories OBJECT-TYPE

SYNTAX     BITS {

bestEffort(0)

background(1)

video(2),

voice(3),

}

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The object that represents the access categories which

the non-AP STA is permitted to use regardless of whether

admission control is configured on that AC.  Frames received

on an AC which the non-AP STA is not permitted to use should

be downgraded to best effort.  An AC is permitted to be used

if its corresponding bit is set to 1; otherwise it is not

permitted to be used."

DEFVAL ( 15 )

::= { dot11InterworkingEntry 6 }

dot11NonApStaAuthSinkMulticast OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

non-AP STA is permitted by the SSP to request

multicast services from the AP. The capability

is disabled otherwise."

DEFVAL ( true )

::= { dot11InterworkingEntry 13 }

dot11nonApStaAuthDls OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

non-AP STA is permitted by the SSPN Interface to use

direct link service (DLS).  This object does not mean

the AP is capable of providing DLS service.  This service

is disabled otherwise."

DEFVAL ( true )

::= { dot11InterworkingEntry 26 }

Editing instruction: add the following entries into the MIB in the order specified in the InterworkingTable, replacing <EditorAssigned> with the appropriate values:

dot11NonApStaAuthMaxVoiceOctetCount OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4294967295)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the maximum authorized total octet count

that a STA may use on the voice access category.  If this octet

count is exceeded, the AP should disassociate the non-AP STA.  A

value of zero indicates that there is no octet limit."

DEFVAL ( 0 )

::= { dot11InterworkingEntry <EditorAssigned> }

dot11NonApStaAuthMaxVideoOctetCount OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4294967295)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the maximum authorized total octet count

that a STA may use on the video access category.  If this octet

count is exceeded, the AP should disassociate the non-AP STA.  A

value of zero indicates that there is no octet limit."

DEFVAL ( 0 )

::= { dot11InterworkingEntry <EditorAssigned+1> }

dot11NonApStaAuthMaxBestEffortOctetCount OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4294967295)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the maximum authorized total octet count

that a STA may use on the best effort access category.  If this octet

count is exceeded, the AP should disassociate the non-AP STA.  A

value of zero indicates that there is no octet limit."

DEFVAL ( 0 )

::= { dot11InterworkingEntry <EditorAssigned+2> }

dot11NonApStaAuthMaxBackgroundOctetCount OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4294967295)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the maximum authorized total octet count

that a STA may use on the background access category.  If this octet

count is exceeded, the AP should disassociate the non-AP STA.  A

value of zero indicates that there is no octet limit."

DEFVAL ( 0 )

::= { dot11InterworkingEntry <EditorAssigned+3> }

dot11NonApStaAuthMaxHCCAOctetCount OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4294967295)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the maximum authorized total octet count

that a STA may use with HCCA access.  If this octet

count is exceeded, the AP should disassociate the non-AP STA.  A

value of zero indicates that there is no octet limit."

DEFVAL ( 0 )

::= { dot11InterworkingEntry <EditorAssigned+4> }

dot11NonApStaAuthMaxTotalOctetCount OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4294967295)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the maximum authorized total octet count

that a STA may use on all access categories combined.  If this octet

count is exceeded, the AP should disassociate the non-AP STA.  A

value of zero indicates that there is no octet limit."

DEFVAL ( 0 )

::= { dot11InterworkingEntry <EditorAssigned+5> }