IEEE P802.11
Wireless LANs

| Discussion to clarify On-Demand Beacon, Extended Capability Bits and WAVE definitions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Alastair Malarky | Mark IV | 6030 Ambler Drive, Mississauga, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |

Abstract

FOR DISCUSSION PURPOSES ONLY

The transition to WAVE using the On-Demand Beacon has introduced a number of discrepancies that need resolved.  A large number of comments in LB110 are related to the confusion introduced in 802.11p Draft 3.0 between On-Demand Beacon and WAVE Advertisement.

A number of submissions (see references) attempt to clarify the text and propose definitions but do not fully resolve the issues.  This document attempts to provide a framework for discussion so that we can remove any ambiguity in the next revision of 802.11p.

DISCUSSION

In the 802.11p 3.0 draft we proposed to add (amongst other concepts) the following new items to 802.11:

ON Demand Beacon

Capabilities field within the Extended Capabilities IE in the Beacon Frame

WAVE Mode

WAVE BSS

WAVE Advertisement Frame

WAVE Service Information Element

As a result of ad-hoc discussions, the following major conceptual revisions are proposed :

A STA can operate in WAVE Mode if the MIB attribute dot11WAVEServicesEnabled is true for that STA.

WAVE Mode is a condition of operation in a BSS, but without authentication and association being performed at the MAC and PHY layers, and with optional TSF synchronization.  In WAVE Mode, functionality equivalent to authentication, association and synchronization is provided by higher layers.

A STA in WAVE Mode may transmit WAVE Beacon.  WAVE Beacons are On-Demand Beacons which have the WAVE Mode indication bit set in the Capabilities field of the Extended Capabilities information element and optionally may include WIEs in the On-Demand Beacon frame, where the WIEs are used by higher layers to support the functionality replacing authentication, association and synchronization.

On-Demand Beacon, are similar to beacon frames with the addition of:

an Extended Capabilities field, which has :

One bit is an “On-Demand” indication

One bit is a “WAVE Mode” indication

an optional WIE field(s)

On receipt of an On-Demand Beacon with the WAVE Mode bit set, a MLME-ONDEMANDBEACON.Indication will occur with the BSSType field in the BSSDescription identifying a WAVE BSS.