IEEE P802.11
Wireless LANs

| LB115-MAC CID’s related to A-MPDU |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Naveen K. Kakani | Nokia | 6021 Connection Drive, Irving TX 75039 USA | +1-214-763-1041 | naveen.kakani@nokia.com |
| Tomoko Adachi | Toshiba Corporation |  |  | tomo.adachi@toshiba.co.jp |

Abstract

Addressing some comments from TGn LB115 from the A-MPDU Tab based on TGn Draft 3.01

| CID | Page(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 5027 | 120 | Can't an HT AP transmit a broadcast A-MPDU when the BSS is a pure HT BSS? | Change all those appearing as "multicast" in this subclause (including title) to "multicast/broadcast". | Counter : Change as shown in document 11-07-2815r1 |
| 5380 | 120 | How does this work in a mixed network | Require clarification | Counter : Change as shown in document 11-07-2815r1 |
| 5026 | 120 | "When an HT AP transmits an A-MPDU containing MPDUs with a multicast RA, both of the following apply:" Doesn't this need a "shall ? | Change it to "When an HT AP transmits an A-MPDU containing MPDUs with a multicast RA, both of the following shall apply:" | Counter : Change as shown in document 11-07-2815r1 |
| 5028 | 120 | "all HT STAs associated with the AP" Isn't this the condition when the A-MPDU is a broadcast A-MPDU? For a multicast A-MPDU, only those included in the RA needs to be cared. | Change two places in this subclause appearing as "all HT STAs associated with the AP" to "all HT STAs that are included in the RA". | Reject : The AP is not capable to determine the STAs that are part of a Multicast group |
| 5310 | 120 | Two sentences for the price of one! | Split the "sentence" into two sentences. | Counter : Change as shown in document 11-07-2815r1 |

TGn Editor: on page 122 of D3.01, make the following changes to Section 9.7d.3:

9.7d.3 A-MPDU aggregation of multicast group addressed data frames

An HT AP may transmit an A-MPDU containing MPDUs with a multicast group addressed RA.

A non-AP HT STA shall not transmit an A-MPDU containing an MPDU with a multicast group addressed

RA.

An HT AP shall not transmit an A-MPDU containing group-addressed MPDUs unless the HT-Protection field is set to 3.

When an HT AP transmits an A-MPDU containing group addressed MPDUs,

both of the following shall apply:

— the value of Maximum A-MPDU Length Exponent that applies is the minimum value

in the Maximum A-MPDU Length Exponent subfield of the A-MPDU Parameters field of

the HT Capabilities Element across all HT STAs associated with the AP;

— the value of Minimum MPDU Start Spacing that applies is the maximum value in the Minimum

MPDU Start Spacing subfield of the A-MPDU Parameters field of the HT Capabilities Element

across all HT STA associated with the AP.

References: TGn Draft 3.01