IEEE P802.11
Wireless LANs

| LB115 PHY Comment Resolutions: PHY Characteristics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-group PHY-Characteristics. The following CIDs are addressed: 5262, 5064, 5781, 5780, 5782, 5065, 5347, 5071, 5350, 5783, 5356, 5074, 5067, 5070, and 5069.

The changes marked in this document are based on TGn Draft 3.0

CID        Sec.            Pg.        Ln.             Comment                         Proposal                        Proposed Resolution

| 5262 | 20.3.20.2 | 303-304 | 61-16 | In the case of beamforming (spatial mapping), spectral flatness requirements should not apply since spatial mapping can yield large variations in phase/amplitude in frequency. | Add the following sentence in the beginning of the section: "The spectral flatness requirements in this sections shall not apply in the case spatial mapping in section 20.3.10.10.1 (Spatial mapping) is applied." | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 303, line 60, add the following sentence at the beginning of the paragraph:

“The spectral flatness requirements in this section shall not apply in the case spatial mapping (section 20.3.10.10.1 (Spatial mapping)) is applied.”

| 5064 | 20.3.20.1 | 303 | 33-34 | Transmission with CH_OFF_20U or CH_OFF_20L shall conform to the 20 MHz transmit spectrum mask. | Remove CH_OFF_20U or CH_OFF_20L in this sentence on conforming to the 40 MHz transmit spectrum mask; and add that CH_OFF_20U or CH_OFF_20L shall conform to the 20 MHz transmit spectrum mask. | Reject. Reason for rejection: If this is done, then it becomes regular 20 MHz mask transmission. Idea of having 40 MHz mask is to have fast response, i.e changing to 20 MHz filtering takes time. |
| --- | --- | --- | --- | --- | --- | --- |

| 5781 | 20.3.20.5 | 304 | 34 | "The receiver shall assert PHY-CCA.indication(idle)" This does not accord with the format of the PHY-CCA.indication in 12.3.5.10.2: "PHY-CCA.indication (STATE, channel-list)" | Review all uses of PHY-CCA.Indication and make them conform to the prototype in 12.3.5.10.2. | Reject. Reason for rejection: When STATE is IDLE or when, for the type of PHY in operation, CCA is determined by a single channel, the channel-list parameter is absent. |
| --- | --- | --- | --- | --- | --- | --- |

| 5780 | 20.3.20.5 | 304 | 34 | "The receiver shall assert PHY-CCA.indication(idle)" - wrong verb "assertion" has no meaning in the world of the SAP. | Replace with: "The receiver shall emit a PHY-CCA.indication(idle) primitive" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 304, line 34, modify the sentence as follows:

“The receiver shall emit a PHY-CCA.indication(idle) primitive (see..”

| 5782 | 20.3.20.5 | 304 | 39 | emits aPHY-TXEND.confirm primitive - missing space | add a space after "a" | Reject. This is not a typo, used elsewhere in the spec. |
| --- | --- | --- | --- | --- | --- | --- |

| 5065 | 20.3.20.5 | 304 | 39 | It's "a_PHY-TXEND" not "a PHY-TXEND". | Fix. | Reject. This is not a typo, used elsewhere in the spec. |
| --- | --- | --- | --- | --- | --- | --- |

| 5347 | 20.3.20.7.3 | 305 | 26 | "RMS error, averaged over subcarriers OFDM frames" Actually the M in RMS indicates averaging over subcarriers, spatial streams and OFDM symbols. The RMS values per frame are then averaged over frames | correct | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 305, line 26, modify the text as follows:

“The relative constellation frame averaged RMS error, calculated first by averaging averaged over subcarriers, OFDM frames, and spatial streams shall not exceed a data-rate dependent value according to Table 20-21 (Allowed relative constellation error versus constellation size and coding rate).”

| 5071 | 20.3.20.7.4 | 306 | 43 | We should specify that these are active receive chains. | Change "all receive chains" to "all active receive chains" . | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 309, line 38, modify the text as follows (checked with author of the CID that page and line numbers are incorrect):

“The received power shall be the average of the power in all active receive chains.”

| 5350 | 20.3.21.1 | 307 | 13 | So we have no sensitivity test for STBC modes, LDPC modes etc. I could receive STBC with 99.9999999999999999999999999% PER (return random bits) and still meet the standard. | Add at least some basic test for these modes. Ditto P308L1 | Reject. Reason for rejection: Complex measurement procedure, requires propagation matrix. Also testing procedures are not specified for all modes and PHY features. Basic functionality has to be performed by system developers. |
| --- | --- | --- | --- | --- | --- | --- |

| 5783 | 20.3.21.5 | 308 | 42 | "CCA sensitivity requirements for non-HT PPDUs in the primary channel are described in clauses 17 and 19." - we can be more helpful than that | Refer to the specific subclauses that define these. | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 308, line 42, modify the text as follows:

“CCA sensitivity requirements for non-HT PPDUs in the primary channel are described in clauses 17.3.10.5 and 19.4.6.”

| 5356 | 20.3.21.5.1 | 308 | 52 | This is in a PMD clause yet defines behavior at the PLCP/MAC interface, namely: "PHY-CCA.indicate" | Add signals between PMD and PLCP providing the required raw inputs to the CCA calculation: "CS-above-threshold" for 20, 40 MHz, GF?, and ED for 20, 40 MHz. In a PLCP section (e.g. 20.3.23?), define how these raw inputs are combined with MCS/length information to create the CCA signal sent between PLCP and MAC. Add appropriate signals at the PMD/PLCP interface to fig 20-20. | Reject. Reason for rejection: the proposed signals would add unnecessary complexity to the draft. It should be sufficient that the signal PHY-CCA.indicate(BUSY) is defined. CCA.indicate(BUSY) function is not ambiguous even though it may include both ED and CS CCA. Many functions in the draft are not defined in detail, same is in this case. |
| --- | --- | --- | --- | --- | --- | --- |

| 5074 | 20.3.21.5.2 | 309 |  | It will be difficult for all 802.11 a/b/g/n receivers to tell whether GF preambles are a valid transmission. They will set their CCA threshold to -62 dBm when they detect Greenfield transmissions, and -82 dBm when they detect non-GF transmissions. This will cause an extremely high rate of collisions for GF transmissions. It is evident that the current protection mechanism associated with GF is not sufficient. | Either "lower the CCA threshold for unknown received signal types" or provide better protection mechanism. | Reject. Reason for rejection: The threshold for GF packets is already -72 dBm (see sections 20.3.21.5.1 and 20.3.21.5.2). |
| --- | --- | --- | --- | --- | --- | --- |

| 5067 | 20.3.21.6 | 309 | 38 | We should specify that these are active receive chains. | Change "all receive chains" to "all active receive chains" . | Accept. See CID 5071. |
| --- | --- | --- | --- | --- | --- | --- |

| 5070 | 20.3.21.6 | 309 | 64-65 | This sentence is written very unclearly and misinterpretation may occur. It doesn't seem to match its intended meaning, that for purposes of verifying the RCPI, RF power is measured in a BW equivalent to 1.1 times the channel BW. | Check and reword as "RF power shall be measured in a bandwidth equivalent to 1.1 times of the channel BW." or its alikes. | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 309, line 64, modify the text as follows:

“The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1. The received RF power shall be measured in a bandwidth equivalent to 1.1 times the channel BW.”

| 5069 | 20.3.21.6 | 309 | 64-65 | There may be other equivalent ways to compute this RF power for RCPI verifications, thus it is not necessary to mandate one method. | Change "shall" to "should", or mention the fact that other equilvalent methods are acceptable too. | Reject. Reason for rejection: This sentence only mandates the signal bandwidth over which the power is calculated: “The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.” |
| --- | --- | --- | --- | --- | --- | --- |