IEEE P802.11
Wireless LANs

| LB115 PHY Comment Resolutions: PHY Characteristics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-group PHY-Characteristics. The following CIDs are addressed: 5262, 5064, 5781, 5780, 5782, 5065, 5347, 5071, 5350, 5783, 5356, 5074, 5067, 5070, and 5069.

The changes marked in this document are based on TGn Draft 3.0

CID        Sec.            Pg.        Ln.             Comment                         Proposal                        Proposed Resolution

| 5262 | 20.3.20.2 | 303-304 | 61-16 | In the case of beamforming (spatial mapping), spectral flatness requirements should not apply since spatial mapping can yield large variations in phase/amplitude in frequency. | Add the following sentence in the beginning of the section: "The spectral flatness requirements in this sections shall not apply in the case spatial mapping in section 20.3.10.10.1 (Spatial mapping) is applied." | DEFER. Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 303, line 60, add the following sentence at the beginning of the paragraph:

“The spectral flatness requirements in this section do not apply when spatial mapping (section 20.3.10.10.1 (Spatial mapping)) is used except identity matrix.”

| 5064 | 20.3.20.1 | 303 | 33-34 | Transmission with CH_OFF_20U or CH_OFF_20L shall conform to the 20 MHz transmit spectrum mask. | Remove CH_OFF_20U or CH_OFF_20L in this sentence on conforming to the 40 MHz transmit spectrum mask; and add that CH_OFF_20U or CH_OFF_20L shall conform to the 20 MHz transmit spectrum mask. | Reject. Reason for rejection: If this is done, then it becomes regular 20 MHz mask transmission. Idea of having 40 MHz mask is to have fast response, i.e changing to 20 MHz filtering takes time. |
| --- | --- | --- | --- | --- | --- | --- |

| 5781 | 20.3.20.5 | 304 | 34 | "The receiver shall assert PHY-CCA.indication(idle)" This does not accord with the format of the PHY-CCA.indication in 12.3.5.10.2: "PHY-CCA.indication (STATE, channel-list)" | Review all uses of PHY-CCA.Indication and make them conform to the prototype in 12.3.5.10.2. | Reject. Reason for rejection: “When STATE is IDLE or when, for the type of PHY in operation, CCA is determined by a single channel, the channel-list parameter is absent.” |
| --- | --- | --- | --- | --- | --- | --- |

| 5780 | 20.3.20.5 | 304 | 34 | "The receiver shall assert PHY-CCA.indication(idle)" - wrong verb "assertion" has no meaning in the world of the SAP. | Replace with: "The receiver shall emit a PHY-CCA.indication(idle) primitive" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 304, line 34, modify the sentence as follows:

“The receiver shall assert emit a PHY-CCA.indication(idle) primitive (see..”

| 5782 | 20.3.20.5 | 304 | 39 | emits aPHY-TXEND.confirm primitive - missing space | add a space after "a" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 304, line 39, modify the sentence as follows:

“..emits a PHY-TXEND.confirm primitive..”

| 5065 | 20.3.20.5 | 304 | 39 | It's "a_PHY-TXEND" not "a PHY-TXEND". | Fix. | Reject. This is not a typo, used elsewhere in the spec. |
| --- | --- | --- | --- | --- | --- | --- |

| 5347 | 20.3.20.7.3 | 305 | 26 | "RMS error, averaged over subcarriers OFDM frames" Actually the M in RMS indicates averaging over subcarriers, spatial streams and OFDM symbols. The RMS values per frame are then averaged over frames | correct | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 305, line 26, modify the text as follows:

“The relative constellation frame averaged RMS error, calculated first by averaging averaged over subcarriers, OFDM frames, and spatial streams, shall not exceed a data-rate dependent value according to Table 20-21 (Allowed relative constellation error versus constellation size and coding rate).”

| 5071 | 20.3.20.7.4 | 306 | 43 | We should specify that these are active receive chains. | Change "all receive chains" to "all active receive chains" . | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 309, line 38, modify the text as follows (checked with author of the CID that page and line numbers are incorrect):

“The received power shall be the average of the power in all active receive chains.”

| 5350 | 20.3.21.1 | 307 | 13 | So we have no sensitivity test for STBC modes, LDPC modes etc. I could receive STBC with 99.9999999999999999999999999% PER (return random bits) and still meet the standard. | Add at least some basic test for these modes. Ditto P308L1 | Reject. Reason for rejection: Testing procedures are not specified for all modes and PHY features. Basic functionality has to be verified by system developers. |
| --- | --- | --- | --- | --- | --- | --- |

| 5783 | 20.3.21.5 | 308 | 42 | "CCA sensitivity requirements for non-HT PPDUs in the primary channel are described in clauses 17 and 19." - we can be more helpful than that | Refer to the specific subclauses that define these. | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 308, line 42, modify the text as follows:

“CCA sensitivity requirements for non-HT PPDUs in the primary channel are described in clauses 17.3.10.5 and 19.4.6.”

| 5356 | 20.3.21.5.1 | 308 | 52 | This is in a PMD clause yet defines behavior at the PLCP/MAC interface, namely: "PHY-CCA.indicate" | Add signals between PMD and PLCP providing the required raw inputs to the CCA calculation: "CS-above-threshold" for 20, 40 MHz, GF?, and ED for 20, 40 MHz. In a PLCP section (e.g. 20.3.23?), define how these raw inputs are combined with MCS/length information to create the CCA signal sent between PLCP and MAC. Add appropriate signals at the PMD/PLCP interface to fig 20-20. | Reject. Reason for rejection: the proposed signals would add unnecessary complexity to the draft. It should be sufficient that the signal PHY-CCA.indicate(BUSY) is defined. CCA.indicate(BUSY) function is not ambiguous even though it may include both ED and CS CCA. Many functions in the draft are not defined in detail, same is in this case. Also there is no need for ED indication in the PMD since the indication is provided by PMD_RSSI. |
| --- | --- | --- | --- | --- | --- | --- |

| 5074 | 20.3.21.5.2 | 309 |  | It will be difficult for all 802.11 a/b/g/n receivers to tell whether GF preambles are a valid transmission. They will set their CCA threshold to -62 dBm when they detect Greenfield transmissions, and -82 dBm when they detect non-GF transmissions. This will cause an extremely high rate of collisions for GF transmissions. It is evident that the current protection mechanism associated with GF is not sufficient. | Either "lower the CCA threshold for unknown received signal types" or provide better protection mechanism. | Reject. Reason for rejection: The threshold for GF packets is already -72 dBm (see sections 20.3.21.5.1 and 20.3.21.5.2). |
| --- | --- | --- | --- | --- | --- | --- |

| 5067 | 20.3.21.6 | 309 | 38 | We should specify that these are active receive chains. | Change "all receive chains" to "all active receive chains" . | Accept. See CID 5071. |
| --- | --- | --- | --- | --- | --- | --- |

| 5070 | 20.3.21.6 | 309 | 64-65 | This sentence is written very unclearly and misinterpretation may occur. It doesn't seem to match its intended meaning, that for purposes of verifying the RCPI, RF power is measured in a BW equivalent to 1.1 times the channel BW. | Check and reword as "RF power shall be measured in a bandwidth equivalent to 1.1 times of the channel BW." or its alikes. | Counter, accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, accept in principle.

TGn Editor: on page 309, line 64, modify the text as follows:

“The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1. The received RF power shall be measured in a bandwidth equivalent to 1.1 times the channel BW.”

| 5069 | 20.3.21.6 | 309 | 64-65 | There may be other equivalent ways to compute this RF power for RCPI verifications, thus it is not necessary to mandate one method. | Change "shall" to "should", or mention the fact that other equilvalent methods are acceptable too. | Reject. Reason for rejection: This sentence only mandates the signal bandwidth over which the power is calculated: “The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.” |
| --- | --- | --- | --- | --- | --- | --- |