IEEE P802.11
Wireless LANs

| TGn LB115 Submission for Beam Explicit Quant sub-group |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB115 comments:

5777, 5061, 5258, 5259, 5273, 5062, 5779

The changes marked in this document are based on TGn Draft version P802.11n D3.0.pdf.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolutions

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5777 | 294.07 | 20.3.11.2.1 | "shall be decoded" - there's already a "shall be" in the previous line that covers all these steps. | "shall be" -> are | Accept. Refer to 07/2836r0 |

TGn Editor: In D3.0, clause 20.3.11.2.1, pg 294, line 7, modify text for as follows

…shall be are decoded as a pair of two's complement numbers…

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5061 | 295.04 | 20.3.11.2.1 | The linear value of M_H(k) corresponding to (20-78) should be the min of what's written there and 10^(7/20). | The linear value of M_H(k) corresponding to (20-78) should be the min of what's written there and 10^(7/20). | Reject. Equation 20-79 is correct as written. M_H(k)^lin is equal to m_H(k), accounting for the clipping. When M_H(k) is less than 7, M_H(k)^lin is equal to m_H(k). When M_H(k) is equal to 7, M_H(k)^lin is equal to max{m_H(z)}/10^(7/20). |

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5258 | 295.30 | 20.3.11.2.2 | "In non-compressed beamforming matrix feedback, the beamformee removes the space-time.." there should be a "shall" statement in this sentence. | Write: "In non-compressed beamforming matrix feedback, the beamformee shall remove the space-time.. | Accept. Refer to 07/2836r0 |
| 5259 | 295.64 | 20.3.11.2.3 | "In compressed beamforming matrix feedback, the beamformee removes the space.." there should be a "shall" statement in this sentence. | Write: "In compressed beamforming matrix feedback, the beamformee shall remove the space.." | Accept. Refer to 07/2836r0 |

TGn Editor: In D3.0, clause 20.3.11.2.2, pg 295, line 30, modify text for as follows

In non-compressed beamforming matrix feedback, the beamformee shall removes the space-time stream CSD in…

TGn Editor: In D3.0, clause 20.3.11.2.3, pg 295, line 64, modify text for as follows

In compressed beamforming matrix feedback, the beamformee shall removes the space-time stream CSD in…

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5273 | 295.50 | 20.3.11.2.2 | How the quantization is done needs to be explicitly stated. | delta = 2^(-Nb+1) if V is not equal to 1 Vq= floor(V/delta)*delta else Vq= 1-delta end |  |

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5062 | 296.07 | 20.3.11.2.3 | Should we keep the notation and say "per subcarrier" rather than "per tone"? | Change "per tone" to "per subcarrier". | Reject. The term tone is used throughout the draft |

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5779 | 298.28 | 20.3.11.2.3 | "shall be quantized as described in 7.4.8.8 (Compressed Beamforming frame format)." | Quantization is described in the MAC, and is presumably done in the MAC, having received the compressed beamforming matrices through the abstract interface with arbitrary precision. Putting a normative requirement inside the PHY is not in accord with this architecture. If we need to say anything here, it should be informative. Replace with: "are quantized by the MAC as described in 7.4.8.8 (Compressed Beamforming frame format)." | Accept. Refer to 07/2836r0 |

TGn Editor: In D3.0, clause 20.3.11.2.3, pg 295, line 64, modify text for as follows

…, shall be are quantized as described in 7.4.8.8…