IEEE P802.11
Wireless LANs

| Normative Text Changes for LB108 CID#100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR 97229 | 503-264-7799 | Emily.H.Qi@intel.com |
| Menzo Wentink | Conexant | Oudegracht 3, Utrecht, the Netherlands | +31-65-183-6231 | menzo.wentink@conexant.com |

Abstract

This document is a normative text proposal to provide comment resolutions for TGv LB108 CID #100. Changes are shown with respect to TGv draft D1.02.

The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGv Editor: Change clause 10.3.62 as follows:

- TIM Broadcast Setup

The following MLME primitives support the signaling of TIM Broadcast Setup. TIM Broadcast Setup Protocol Exchange depicts the TIM Broadcast setup process. The figure illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses.

- TIM Broadcast Setup Protocol Exchange

- MLME-TIMBROADCAST.request

- Function

This primitive requests that a TIM Broadcast Request frame be sent to the AP with which the non-AP STA is associated.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TIMBROADCAST.request	(
PeerSTAAddress,
Dialog Token,
TIMBroadcastRequest
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TIM Broadcast process. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the TIM Broadcast request and response transaction. |
| TIMBroadcast-Request | As defined in TIM Broadcast Request element | As defined in TIM Broadcast Request element | Specifies the proposed service parameters for the TIM Broadcast. |

- When Generated

This primitive is generated by the SME to request that a TIM Broadcast Request frame be sent to the AP with which the STA is associated.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a TIM Broadcast Request action management frame. The STA then attempts to transmit this to the AP with which it is associated.

- MLME-TIMBROADCAST.confirm

- Function

This primitive reports the result of a TIM Broadcast procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TIMBROADCAST.confirm(

ResultCode,

PeerSTAAddress,
Dialog Token,

TIMBroadcastResponse
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, MALFORMED REQUEST, REQUESTED INTERVAL TOO LONG, or OVERRIDDEN DUE TO LACK OF RESOURCES | Reports the outcome of a request to send a TIM Broadcast Request frame. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TIM Broadcast process. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the TIM Broadcast request and response transaction. |
| TIMBroadcast- Response | As defined in TIM Broadcast Response element | As defined in TIM Broadcast Response element | Specifies service parameters for the TIM Broadcast. |

- When Generated

This primitive is generated by the MLME as a result of an MLME- TIMBROADCAST.request and indicates the results of the request.

This primitive is generated when the MLME- TIMBROADCAST.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a TIM Broadcast Response frame from the AP.

- Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

- MLME- TIMBROADCAST.indication

- Function

This primitive indicates that a TIM Broadcast Request frame was received from a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- TIMBROADCAST.indication(
PeerSTAAddress,
Dialog Token,
TIMBroadcastRequest
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TIM Broadcast Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the TIM Broadcast request and response transaction. |
| TIMBroadcast- Request | As defined in TIM Broadcast Request element | As defined in TIM Broadcast Request element | Specifies the proposed service parameters for the TIM Broadcast. |

- When Generated

This primitive is generated by the MLME when a valid TIM Broadcast Request frame is received.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 9.2.7.1.1.

- MLME-TIMBROADCAST.response

- Function

This primitive is generated in response to an MLME-TIMBROADCAST.indication requesting a TIM Broadcast Response frame is sent to a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- TIMBROADCAST.response(
PeerSTAAddress,
Dialog Token,
TIMBroadcastResponse
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TIM Broadcast Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the TIM Broadcast request and response transaction. |
| TIM Broadcast Response | As defined in TIM Broadcast Response element | As defined in TIM Broadcast Response element | Specifies service parameters for the TIM Broadcast. |

- When Generated

This primitive is generated by the SME in response to an MLME-TIMBROADCAST.indication requesting a TIM Broadcast Response be sent to a non-AP STA.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a TIM Broadcast Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.