IEEE P802.11
Wireless LANs

| TGn LB115 Submission for PHY Data Comments (Nov. 2007) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Road, San Jose CA, 95131 | +1.408.451.8109 (o) +1.408.893.9357 (m) | George.Vlantis@st.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB115 comments: CID# 5324, 5771, 5824, and 5771.  These comments are collected in the “PHY Data” tab of Document #11-07-2703r0.

The changes marked in this document are based on TGn Draft version P802 11n D3.0.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor:” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolutions

Click on icon below for “PHY Data” tab of “11-07-2703-00-000n-tgn-lb115-phy-ad-hoc-comment-spreadsheet.xls”.

# CID# 5324 (Page 252, Line 39, Clause 20.3.10.2)

Commenter’s Comment:  “It is not specified that all DATA symbols into data field have to be scrambled by using a different non-zero seed for the scrambler initialization.”

Commenter’s Proposed Change:   “Add an appropiate sentence into this section.”

Citations in Draft 802.11n 3.0:

Subclause 20.3.10.2 “Scrambler” on page 272, line 41 of Draft 802.11n 3.0 states in its entirety:

“The data field is scrambled by the scrambler defined 17.3.5.4”

Furthermore, paragraph f) in subclause “20.3.4 Overview of the PPDU encoding process” on page 244, line 35 of Draft 802.11n 3.0 states in its entirety:

“f) Initiate the scrambler with a pseudo-random nonzero seed, generate a scrambling sequence, and XOR (Boolean exclusive OR operation) it with the string of data bits, as described in 17.3.5.4.”

Citations in 802.11REVma:

The subclause cited in Draft 802.11n 3.0 is 17.3.5.4 “PLCP DATA scrambler and descrambler” on page 604 of 802.11 REVma, which states:

“When transmitting, the initial state of the scrambler will be set to a pseudo-random nonzero state.”

Furthermore, paragraph e) in subclause “17.3.2.1 Overview of the PPDU encoding process” on page 596 of 802.11 REVma states in its entirety:

“e) Initiate the scrambler with a pseudo-random nonzero seed, generate a scrambling sequence, and XOR it with the extended string of data bits. Refer to 17.3.5.4 for details.”

Proposed Resolution:  Counter (Accept In Principle).

Discussion:

The commenter is pointing out that the scrambler should be initialized with a seed other than zero.  Therefore, I move to authorize the editor to make the following change to Draft 3.0 that is consistent with text in both Draft 11n 3.0 and 802.11 REVma:

TGn Editor: In D3.0, subclause 20.3.10.2 “Scrambler”, page 272, line 41, modify the sentence as follows:

The dData field is scrambled by the scrambler defined in 17.3.5.4, initialized with a pseudo-random nonzero seed.

# CID# 5771 (Page 272, Line 52, Clause 20.3.10.3)

Commenter’s Comment:  “`For the purposes of determining whether to use one or two BCC FEC encoders, the rate shall be calculated based on the use of an 800 ns GI.’

The ‘shall’ is over the top, because it describes conditions under which the previous ‘is’ statement applies.  If a ‘shall’ is necessary here, it is also necessary in the previous sentence.”

Commenter’s Proposed Change:   “change ‘shall be’ → ‘is’.”

Citations in Draft 802.11n 3.0:

The first paragraph of subclause 20.3.10.3 “Coding” on page 272, lines 41-57 of Draft 802.11n 3.0 states in its entirety:  (Note that all “shall be”s are provided for illustrative purposes, but are not in Draft 3.0)

“The Data field is encoded using either the binary convolutional code (BCC) defined in 17.3.5.5, or the low density parity check (LDPC) code defined in 20.3.10.6 (Low density parity check (LDPC) codes). The encoder is selected by the FEC coding field in the High Throughput Signal Field, as described in 20.3.9.4.3 (The HT SIGNAL field).  A single FEC encoder is always used when LDPC coding is used. When the BCC FEC encoder is used, a single encoder is used, except that two encoders shall be used when the selected MCS has a PHY rate greater than 300 Mb/s. For the purposes of determining whether to use one or two BCC FEC encoders, the rate shall be calculated based on the use of an 800 ns GI. The operation of the BCC FEC is described in 20.3.10.5 (Binary convolutional coding and puncturing). The operation of the LDPC coder is described in 20.3.10.6 (Low density parity check (LDPC) codes).”

Proposed Resolution:  Accept.

Discussion:

The last three sentences of this introductory paragraph were added at various phases of the Joint Proposal and Letter Ballot processes, hence the inconsistency of using “shall be” and “is”.   As the first sentence with “shall be” normatively states the PHY rate condition under which two BCC encoders must be used, and the second sentence just defines the method of how to calculate the PHY rate for this purpose, the second “shall be” seems superfluous.

Therefore, I move to accept the commenter’s Proposed Change to change the “shall be” to “is” and authorize the editor to make the following change to Draft 3.0:

TGn Editor: In D3.0, subclause 20.3.10.3 “Coding”, page 272, lines 52-53, modify the sentence as follows:

For the purposes of determining whether to use one or two BCC FEC encoders, the rate shall beis calculated based on the use of an 800 ns GI.

# CID# 5824 (Page 277, Line 7, Clause 20.3.10.6)

Commenter’s Comment:  “It has been brought to my attention that the "If's" in this paragraph can be misinterpreted as "While's".  That is, the "If's" are not clear whether Equations 20-43 and 20-44 should be executed at most once, or possibly more than once (in which case "While" would be more appropriate).  The "If's" correctly imply that Equations 20-43 and 20-44 should be executed at most once.  (The consequence of repeating these equations would be to append extra symbols on the payload.)  Therefore, I'm suggesting the clarifying change in the "Recommended Change" field of this comment.”

Commenter’s Proposed Change:   “Replace the clause ‘by the following:’ at the end of the sentence with ‘by the following two equations once’.”

Proposed Resolution:  Accept.

Discussion:  (CID #5477 is a related to this comment.)

The conditions expressed starting in Line 7 may remain true after executing Equations 20-43 and 20-44.  The confusion that arose was that if the meaning of the “if’s” on the conditions. Was “if” a true “if” (i.e. executed once) or was it “if until” (i.e. a “while” loop)?  The correct interpretation is that the “if” is a true “if” (otherwise we would have written “while).  The consequence of an execution of Equations 20-43 and 20-44 is to add an additional symbol to the Data field, which is not desirable to do more than once (i.e. more airtime, more power consumption, and more complexity in calculating the duration).     To emphasize that equations 20-43 and 20-44 are to be executed at most once, I move to accept the commenter’s Proposed Change and to authorize the editor to make the following change to Draft 3.0:

TGn Editor: In D3.0, subclause 20.3.10.6.5 “LDPC PPDU encoding process”, paragraph d), page 272, lines 10-11, append to the phrase after the comma as follows:

then increment Navbits and recompute Npunc by the following two equations once:

# CID# 5477 (Page 277 [not 278], Line 10, Clause 20.3.10.6)

Commenter’s Comment:  “there are conditions where the puncturing test are performed and the number of symbols are incremented, but the condition is still met that would indicate further incrementing of the number of symbols.  Clarify that this is only done once.”

Commenter’s Proposed Change:   “change ‘… is true, then increment N_avbits and recompute…’ to ‘… is true, then increment N_avbits once and recompute…’.”

Proposed Resolution:  Counter (Accept in Principle)—refer to resolution of CID #5824.

Discussion:  (CID #5824 is a related to this comment.)

The confusion expressed by the commenter is addressed by the resolution to related CID #5824.  The resolution to CID #5824 is superior in its clarity, because it specifies that both Equations 20-43 and 20-44 are executed at most once.  The commenter’s Proposed Change has the flaw that there is possible ambiguity that only Equation 20-43 is executed at most once.