IEEE P802.11
Wireless LANs

| Proposed draft text For Action No Ack brianh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Brian Hart | Cisco Systems | 170 W Tasman Dr, San Jose, CA, USA | 408-5253346 | brianh@cisco.com |

Abstract

Draft text instructions to correct Draft P802.11y D5.1 clause 11.3, removing Action No Ack from CID 4029 draft text in 07/2683r3 which was approved by TGy 2007-11-13

11.3 STA authentication and association

Noting that for brevity not all of subbullets to bullet a) are present in this draft yet  are not changed by this draft, change item a.2) in D5.1 to be as follows:

a) Class 1 frames (permitted from within States 1, 2, and 3):

2) Management frames

i) Probe request/response

ii) Beacon

iii) Authentication:

Successful authentication enables a STA to exchange Class 2

frames. Unsuccessful authentication leaves the STA in State 1.

iv) Deauthentication:

Deauthentication notification when in State 2 or State 3 changes the STA's state to State 1. The STA shall become authenticated again prior to sending Class 2 frames. Deauthentication notification when in State 3 implies disassociation as well.

v) Announcement traffic indication message (ATIM)

vi) Public ActionSpectrum Management Action: Within an IBSS, action frames are Class 1.

vii) Within an IBSS, all Action frames are Class 1.

vii) Action frames with Category field equal to Radio Measurement and Action field

equal to Measurement Pilot

Change item c) in D5.1 to be as follows:

c) Class 3 frames (if and only if associated; allowed only from within State 3):

1) Data frames

i) Data subtypes: Data frames allowed, i.e., either the "To DS" or "From DS" FC

bits may be set to true to utilize the DSS.

ii) QoS data subtypes allowed to/from non-AP STA(s) that are associated with

AP(s).

iii) Data frames between STAs in a BSS with FC bits "To DS" and "From DS"

both false.

2) Management frames.

i) Within an infrastructure BSS, all Action frames except the following frames:

A) Public Action

i) QoS, DLS and Block Ack Action outside of an IBSS

ii) Radio Measurement Action

3) Control frames

i) Power save (PS)-Poll

ii) Action:

Within an infrastucture BSS, action frames are class 3.

iii) Block Ack (BlockAck)

iiiv) Block Ack Request (BlockAckReq)