IEEE P802.11
Wireless LANs

| Proposed resolution to CID 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |

Abstract

This document contains a proposed resolution to CID 9 (diagnostics category) from LB108.

Comment: “Automatic Tx power should have a corresponding minimum and maximum range analogous to fixed power output”

Proposed resolution: “Provide for a minimum and maximum reporting range for non-AP STA's that use automatic power control.  The same format for fixed output power should be used.”

In section 7.3.2.64, change the paragraph below Table v16 as indicated:

If the Tx Power Mode field is set to be Fixed then this the TxPower field contains one or more Tx Power values where each octet in the Tx Power field contains a 2’s complement value indicating a Tx Power level in dBm. If the Tx Power Mode field is not set to Fixed then this field is omitted. If the Tx Power Mode field is set to Automatic, the TxPower field is two octects in length and contains the STA’s minimum and maximum TxPower levels, encoded as a one octet 2’s complement value, in dBm.

References: