IEEE P802.11
Wireless LANs

| Proposed resolutions to CID 799 and CID 1545 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX | +441784848770 | aashley@nds.com |

Abstract

This document contains a proposed resolution to CID 799 and 1545 from LB108.

Comment 1545: “Need to add a mechanism to allow the STA to refuse a multicast diagnostic measurement request at a particular time due to the lack of resource.”

Comment 799: "Replace the sentence ‘A non-AP STA that supports Multicast Diagnostic Reporting shall set the Multicast Diagnostic capability bit in the Wireless Network Management Capabilities element to 1.’ with ‘A non-AP STA supporting Wireless Network Management Capability element shall set the Multicast Diagnostic capability bit in the Wireless Network Management Capabilities element to 1.’."

In section 11.20.1, modify the paragraph as indicated:

A non-AP STA that supports Multicast Diagnostic Reporting supporting Wireless Network Management Capability element shall set the Multicast Diagnostic capability bit in the Wireless Network Management Capabilities element to 1 if it supports multicast diagnostic reporting. A non-AP STA supporting Wireless Network Management Capability element shall set the Multicast Diagnostic capability bit in the Wireless Network Management Capabilities element to 0 if it does not support multicast diagnostic reporting..

Add to the end of section 11.20.1, add the following paragraph:

A station that declines a request for triggered multicast diagnostic reporting shall send a Measurement Report element (as described in 7.3.2.22) in a Measurement Report frame (as described in 7.4.1.2) with the Measurement Report Mode field set appropriately to indicate the reason for a failed or rejected request.

References: