IEEE P802.11
Wireless LANs

| [RRM Enabled Capabilities Element Audit] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave, Hillsboro, OR 97124 | +1 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This document is an audit of RRM Enabled Capabilities Element bit assignments to the RRMs listed in Annex-A of TGk Draft 9.0.

| RRM (Annex-A) | Description | RRM Enabled Capability element bit assignment | MIB variable + notes |
| --- | --- | --- | --- |
| RRM1 | Radio Measurement Capability | n/a | Capability IE bit12 |
| RRM2 | Action Frame Protocol for measurements | n/a | Header Only |
| RRM2.1 | Radio Measurement Request frame | n/a | Capability IE bit12 |
| RRM2.2 | Radio Measurement Report frame | n/a | Capability IE bit12 |
| RRM2.3 | Link Measurement Request Frame | Bit 0 | dot11RRMLinkMeasurementEnabled |
| RRM2.4 | Link Measurement Report Frame | Bit 0 | dot11RRMLinkMeasurementEnabled In Annex-A, combine with RRM2.3 |
| RRM2.5 | Neighbor Report Request | Bit 1 | dot11RRMNeighborReportEnabled |
| RRM2.5.1 | Generate and transmit Neighbor Report Request | Bit 1 | dot11RRMNeighborReportEnabled In Annex-A, delete RRM2.6.1 |
| RRM2.5.2 | Receive and process Neighbor Report Request | Bit 1 | dot11RRMNeighborReportEnabled In Annex-A delete RRM2.6.2 |
| RRM2.6 | Neighbor Report Response | Bit 1 | dot11RRMNeighborReportEnabled |
| RRM2.6.1 | Generate and transmit Neighbor Report Response | Bit 1 | dot11RRMNeighborReportEnabled |
| RRM2.6.2 | Receive and process Neighbor Report Response | Bit 1 | dot11RRMNeighborReportEnabled |
| RRM3 | General Protocol for requesting and reporting of measurements | n/a | Header Only |
| RRM3.1 | Parallel Measurements | Bit 2 | dot11RRMParallelMeasurementEnabled |
| RRM3.2 | Use of Enable, Request & Report bits to enable/ disable measurement requests and triggered autonomous reports Measurement Requests | n/a | Can be controlled by sending/receiving measurement request with the enable/request/report bits. |
| RRM3.3 | Enable Autonomous Report | n/a | Not needed |
| RRM3.4 | Duration Mandatory | Bits 21-23 Bits 24-26 | dot11RRMMaxMeasurementDuration dot11RRMOffChannelMaxMeasurementDuration |
|  | Measurements on non-operating channels | n/a | dot11RRMNonOperatingChannelMeasurementEnabled – do not need this. For offChannel ‘0’ means not-capable. |
| RRM3.5 | Incapable Indication | n/a | Capability IE bit12 |
| RRM3.6 | Refused Indication | n/a | Capability IE bit12 |
| RRM3.7 | Repeated Measurement | Bit 3 | dot11RRMRepeatedMeasurementEnabled |
| RRM3.8 | Measurement Pause | n/a |  |
| RRM4 | Beacon Measurement | n/a | Header Only |
| RRM4.1 | Beacon Request | n/a | Header Only |
| RRM4.2 | Passive Measurement Mode | Bit 4 | dot11RRMPassiveBeaconMeasurementEnabled Reporting Detail needs an explicit bit? No. Need to add a PICS entry (4.7) as well. Section 11 needs to describe Reporting Detail interpretation/default behavior. |
| RRM4.3 | Active Measurement Mode | Bit 5 | dot11RRMActiveBeaconMeasurementEnabled |
| RRM4.4 | Beacon Table Mode | Bit 6 | dot11RRMTableBeaconMeasurementEnabled |
| RRM4.5 | Reporting Conditions | Bit 7 | dot11RRMReportingConditionsEnabled |
| RRM4.6 | Beacon Report | n/a | Header Only |
| RRM4.7 | Reporting Detail | n/a | Enablement implicit to the state of dot11RRM[Table¦Active¦Passive]BeaconMeasurementEnabled. |
| RRM5 | Frame Measurement | Bit 8 | dot11RRMFrameReportEnabled |
| RRM5.1 | Frame Request | Bit 8 | dot11RRMFrameReportEnabled |
| RRM5.2 | Frame Report | Bit 8 | dot11RRMFrameReportEnabled |
| RRM6 | Channel Load Measurement | Bit 9 | dot11RRMChannelLoadEnabled |
| RRM6.1 | Channel Load Request | Bit 9 | dot11RRMChannelLoadEnabled |
| RRM6.2 | Channel Load Report | Bit 9 | dot11RRMChannelLoadEnabled |
| RRM7 | Noise Histogram Measurement | Bit 10 | dot11RRMNoiseHistogramEnabled |
| RRM7.1 | Noise Histogram Request | Bit 10 | dot11RRMNoiseHistogramEnabled |
| RRM7.2 | Noise Histogram Report | Bit 10 | dot11RRMNoiseHistogramEnabled |
| RRM8 | Statistics Measurement Type | Bit 11 | dot11RRMStatisticsReportEnabled |
| RRM8.1 | STA Statistics Request | Bit 11 | dot11RRMStatisticsReportEnabled |
| RRM8.2 | STA Statistics Report | Bit 11 | dot11RRMStatisticsReportEnabled |
| RRM9 | LCI Measurement Type | Bit 12 | dot11RRMLCIEnabled |
| RRM9.1 | LCI Request | Bit 12 | dot11RRMLCIEnabled |
| RRM9.1.1 | Location Subject | Bit 12 | dot11RRMLCIEnabled |
| RRM9.1.2 | Latitude Requested Resolution | Bit 12 | dot11RRMLCIEnabled |
| RRM9.1.3 | Longitude Requested Resolution | Bit 12 | dot11RRMLCIEnabled |
| RRM9.1.4 | Altitude Requested Resolution | Bit 12 | dot11RRMLCIEnabled |
| RRM9.2 | LCI Report | Bit 12 | dot11RRMLCIEnabled |
| RRM9.3 | Azimuth | n/a | Header Only |
| RRM9.3.1 | Azimuth Request | Bit 13 | dot11RRMAzimuthEnabled |
| RRM9.3.2 | Azimuth Response | Bit 13 | dot11RRMAzimuthEnabled |
| RRM10 | Transmit Stream Category Measurement Type | Bit 14 | dot11RRMTransmitStreamMeasurementEnabled |
| RRM10.1 | Transmit Stream Category Measurement Request | Bit 14 | dot11RRMTransmitStreamMeasurementEnabled |
| RRM10.2 | Transmit Stream Category Measurement Report | Bit 14 | dot11RRMTransmitStreamMeasurementEnabled |
| RRM10.3 | Triggered Transmit Stream/Category Measurement Report | Bit 15 | dot11RRMTriggeredTransmitStreamReportEnabled |
| RRM11 | AP Channel Report | n/a | Header Only |
| RRM11.1 | Generate and transmit AP Channel Report | Bit 16 | dot11RRMAPChannelReportEnabled |
| RRM12 | Neighbor Report Procedure | n/a | Header Only |
| RRM12.1 | Neighbor Report Procedure | Bit 1 | dot11RRMNeighborReportEnabled |
| RRM12.2 | TSF Offset in Neighbor Report | Bit 17 | dot11RRMTSFOffsetEnabled |
| RRM13 | RCPI Measurement | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM13.1 | RCPI Measurement for DSSS PHY at 2.4GHz | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM13.2 | RCPI Measurement for OFDM PHY at 5GHz | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM13.3 | RCPI Measurement for HR DSSS PHY at 2.4GHz | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM13.4 | RCPI Measurement for Extended Rate PHY at 2.4Ghz | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM14 | RCPI Measurement during Active Scanning | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM14.1 | Respond with RCPI information element when requested | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM14.2 | Measurement of RCPI on Probe Request frames | Bit 18 | dot11RRMRCPIMeasurementEnabled (for the PHY mode in operation) |
| RRM15 | RSNI Measurement | Bit 19 | dot11RRMRSNIMeasurementEnabled |
| RRM16 | TPC Information in Beacon and Probe Response frames | n/a | Regulatory Classes/Spectrum Management are implicit here. RRM capable STAs shall implicitly have this capability. |
| RRM16.1 | Country and Transmit Power Control (TPC) Report elements included in Beacon and Probe Response frames | n/a | Regulatory Classes/Spectrum Management are implicit here. RRM capable STAs shall implicitly have this capability. |
| RRM16.2 | Power Constraint element included in Beacon and Probe Response frames | n/a | Regulatory Classes/Spectrum Management are implicit here. RRM capable STAs shall implicitly have this capability. |
| RRM17 | Power Capability elements in Association and Reassociation frames | n/a | Regulatory Classes/Spectrum Management are implicit here. RRM capable STAs shall implicitly have this capability. |
| RRM18 | Management Information Base | Bit 20 | dot11RRMExternalInterfaceMIBEnabled See CID#41; for Annex-Q support |
| RRM18.1 | dot11SMTRRMRequest | Bit 20 | dot11RRMExternalInterfaceMIBEnabled See CID#41; for Annex-Q support |
| RRM18.2 | dot11SMTRRMReport | Bit 20 | dot11RRMExternalInterfaceMIBEnabled See CID#41; for Annex-Q support |
| RRM18.3 | dot11SMTRRMConfig | Bit 20 | dot11RRMExternalInterfaceMIBEnabled See CID#41; for Annex-Q support |
| RRM19 | Measurement Pilot Frame | Bits 27-29 | dot11RRMMeasurementPilotCapability |
| RRM20 | BSS Average Access Delay Elements included in Beacon and Probe Response Frames | Bit 30 | dot11RRMAverageAccessDelayInformationEnabled – use one bit and change interpretation based on QoS STA or not. |
| RRM21 | Antenna Information elements included in Beacon and Probe Response frames | Bit 31 | dot11RRMAntennaInformationEnabled |
| RRM22 | Measurement Pilot Transmission Information element included in beacon and Probe Response frame | Bit 32 | dot11RRMMPTransmissionInformationEnabled – implicit to RRM19 |
| RRM23 | Quiet Interval | n/a | Capability IE:12 |
| RRM23.1 | AP defined Quiet Interval | n/a | Capability IE:12 |
| RRM23.2 | STA defined Quiet Interval | n/a | Capability IE:12 |
| RRM23.3 | STA support for Quiet Interval | n/a | Capability IE:12 |
| RRM24 | BSS Available Admission Capacity | Bit 33 | dot11RRMAvailableAdmissionCapacityEnabled |
| RRM25 | BSS AC Access Delay | n/a | See RRM 20 |