IEEE P802.11
Wireless LANs

| Resolutions for CID 5390 in Channel Sounding Comments of LB115 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hongyuan Zhang | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408 222 1837 | hongyuan@marvell.com |

Abstract

This submission proposed resolutions to CID 5390 in the Channel Sounding comment group. The proposed modifications are based on D3.01.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

CID 5390

| 5390 | 76.24 | 7.3.2.52.6 | The channel estimation capability definition for a device with both NDP and staggered receiving capabilities is vague. For example, if a 2-antenna device (not supporing STBC) can receive NDP with Nsts=3, but can only receive data carried in up to 2 spatial streams, setting this field equal to 2 implies that (3,0) or (3,1) can also be received, which is beyond the capability of the 2-antenna device. | Add a note saying that when staggered sounding is upported, this field should be explained by combining with Supported MCS Field and Rx STBC field. |
| --- | --- | --- | --- | --- |

Proposed Resolution: Counter

Discussions:

For a receiving-sounding capable device, its capability on HT-LTF buffering, and the maximum supported Nsts inferred by its supported MCS (receiver side) and STBC mode are independently defined. However, in the case of NDP sounding, MCS value only infers the number of HT-LTFs, rather than number of space-time streams that will be used in demodulating data symbols.

In the given example, if STBC is not supported, the two-antenna device, when setting this subfield to “2”, cannot receive a staggered sounding packet with Nsts = 3 (i.e. the combinations (3,0) and (3,1)), which is restricted by its supported MCS, even though it can receive a NDP with 3 space-time streams (i.e. buffering 4 HT-LTFs).

Therefore it is necessary to differentiate the setting of this field in NDP, Non-NDP but non-staggered sounding, and staggered sounding. The “(a,b)” combinations listed for staggered sounding case in the spec are also confusing. Furthermore, the current text “supported space time streams” is unclear, it is better to be replaced by “space time streams that can be estimated”.

TGn Editor: in page 76 of D3.01, the 3rd column and last row of Table 7-43k—Subfields of the Transmit Beamforming Capabilities field, make the follwing modifications:

| Channel Estimation Capability | Indicates the maximum number of space time streams (columns of the MIMO channel matrix) for which channel dimensions can be simultaneously estimated. When staggered sounding is supported this limit applies independently to both the data portion and to the extension portion of the long training fields. | Set 0 for 1 space time stream Set 1 for 2 space time streams Set 2 for 3 space time streams Set 3 for 4 space time streams (1) If the reception of NDP sounding is supported, the value indicated by this field is equal to the maximum number of space time streams that can be estimated. (2) If the reception of Non-NDP sounding is supported but the reception of staggered sounding is not supported, the value indicated by this field is equal to the maximum number of space time streams that can be estimated, which is independent from the maximum number of space time streams inferred by Supported MCS Set field and Rx STBC field of HT Capabilities. (3) If the reception of staggered sounding is supported, the value indicated by this field is equal to the maximum number of space time streams that can be estimated, which is independent from the maximum number of space time streams inferred by Supported MCS Set field and Rx STBC field of HT Capabilities. The value indicated by this field is also equal to the maximum number of extension spatial streams that can be estimated, Note: When Non-NDP sounding is supported, the maximum number of space time streams that is supported for demodulating the data symbols shall be calculated based on Supported MCS Set field and Rx STBC field of HT Capabilities, regardless of the value indicated by this field. If the reception of staggered sounding is not supported, the value indicated by this field is equal to the maximum number of supported space time streams. If the reception of staggered sounding is supported: Value 0: (1,0), (1,1) Value 1: (1,0), (2,0), (1,1), (1,2), (2,1), (2,2) Value 2: (1,0), (2,0), (3,0), (1,1), (1,2), (1,3), (2,1), (2,2), (3,1) Value 3: (1,0), (2,0), (3,0), (4,0), (1,1), (1,2), (1,3), (2,1), (2,2), (3,1) where the notation (a, b) indicates: a: supported channel estimation dimensions using long training symbol(s) that will be used for demodulating data symbols. b: supported channel estimation dimensions using long training symbol(s) that will not be used for demodulating data symbols |
| --- | --- | --- |